| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0084 |
| --- | --- |

| Title: | RCE1: Cross-verification of Test 1.1 – LM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunil Lee Chanyul Kim | Email: | sunil.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of RCE1 Test 1.1 on the extended chroma mode for non 4:2:0 format (LM mode). The source code provided by the proponent was reviewed and it was confirmed that the implementation was consistent with the algorithm described in the contribution document. The coding performance is independently evaluated by Samsung and the results match those provided by the proponent.

# Introduction

This contribution reports the cross-verification results of RCE1 Test 1.1 on the extended chroma mode for non 4:2:0 format (LM mode) [1]. In the LM mode, chroma samples are predicted using linear combination of luma samples [2]. At the last meeting, one variant of the LM mode, where parameters for the linear combination are estimated for each CU instead of each TU, is also proposed [2], and evaluated in this contribution. As agreed in RCE1, the LM mode is implemented on top of HEVC range extension software HM10.0_RExt2.0, and its source code and test results are provided to the CE participants. Samsung reviewed the source code and confirmed that the implementation was consistent with the algorithm described in the contribution document. Samsung compiled the source code and did the simulation on a linux cluster under the common test condition of HEVC range extension [3].

# Test results

The cross-verification results of the LM mode are summarized in Table 1 and 2, and they match those provided by the proponent [4].

Table 1. Cross-verification results of Test 1: LM mode as was proposed in HEVC version 1, extended to non-4:2:0

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -19.7% | -17.9% | -18.7% | -15.4% | -14.4% | -15.0% | -11.5% | -10.7% | -11.3% |
| YCbCr 4:4:4 | -2.1% | -7.5% | -7.0% | -2.3% | -4.8% | -6.3% | -2.3% | -3.2% | -4.7% |
| YCbCr 4:2:2 | -1.8% | -5.6% | -4.4% | -1.5% | -2.9% | -2.8% | -1.1% | -1.7% | -1.6% |
| Overall | -8.4% | -10.7% | -10.4% | -6.8% | -7.7% | -8.4% | -5.2% | -5.4% | -6.1% |
|  | -8.4% | -10.7% | -10.5% | -6.8% | -7.9% | -8.5% | -5.2% | -5.5% | -5.9% |
| Enc Time[%] | 107% |  |  | 107% |  |  | 107% |  |  |
| Dec Time[%] | 108% |  |  | 107% |  |  | 106% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -11.1% | -9.8% | -11.8% | -7.9% | -6.3% | -8.5% |  |  |  |
| YCbCr 4:4:4 | -0.9% | -7.6% | -5.6% | -1.0% | -5.1% | -5.0% |  |  |  |
| YCbCr 4:2:2 | -0.8% | -6.4% | -4.3% | -0.7% | -2.8% | -2.8% |  |  |  |
| Overall | -4.6% | -8.0% | -7.5% | -3.4% | -4.8% | -5.6% |  |  |  |
|  | -4.6% | -8.4% | -7.3% | -3.5% | -6.0% | -5.9% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 114% |  |  | 112% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -4.2% | -3.6% | -4.5% | -2.9% | -2.0% | -3.0% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -3.9% | -2.3% | -0.2% | -2.2% | -1.9% |  |  |  |
| YCbCr 4:2:2 | -0.3% | -3.9% | -3.0% | -0.3% | -1.5% | -1.7% |  |  |  |
| Overall | -1.7% | -3.8% | -3.3% | -1.2% | -1.9% | -2.2% |  |  |  |
|  | -1.7% | -3.3% | -3.3% | -1.2% | -2.1% | -2.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 114% |  |  | 111% |  |  |  |  |  |

Table 2. Cross-verification of Test 2: LM mode where parameter calculation is performed on a CU basis

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -17.7% | -16.2% | -16.8% | -13.5% | -12.9% | -13.1% | -9.9% | -9.6% | -9.8% |
| YCbCr 4:4:4 | -1.8% | -6.3% | -5.8% | -1.9% | -4.0% | -5.2% | -1.8% | -2.6% | -3.7% |
| YCbCr 4:2:2 | -0.1% | -1.2% | -0.9% | -0.1% | -0.7% | -0.6% | -0.1% | -0.4% | -0.4% |
| Overall | -7.0% | -8.3% | -8.2% | -5.5% | -6.2% | -6.6% | -4.2% | -4.5% | -4.9% |
|  | -7.0% | -8.3% | -8.3% | -5.6% | -6.2% | -6.7% | -4.2% | -4.5% | -4.7% |
| Enc Time[%] | 107% |  |  | 107% |  |  | 107% |  |  |
| Dec Time[%] | 107% |  |  | 106% |  |  | 105% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -9.4% | -8.7% | -10.3% | -6.6% | -5.5% | -7.3% |  |  |  |
| YCbCr 4:4:4 | -0.8% | -6.3% | -4.8% | -0.8% | -4.2% | -4.2% |  |  |  |
| YCbCr 4:2:2 | -0.1% | -1.2% | -0.9% | -0.1% | -0.6% | -0.6% |  |  |  |
| Overall | -3.7% | -5.6% | -5.6% | -2.6% | -3.5% | -4.2% |  |  |  |
|  | -3.7% | -5.8% | -5.5% | -2.7% | -4.6% | -4.5% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 113% |  |  | 111% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -3.5% | -3.1% | -3.9% | -2.3% | -1.7% | -2.6% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -3.4% | -2.0% | -0.2% | -1.9% | -1.7% |  |  |  |
| YCbCr 4:2:2 | 0.0% | -0.7% | -0.6% | 0.0% | -0.3% | -0.3% |  |  |  |
| Overall | -1.4% | -2.4% | -2.2% | -0.9% | -1.3% | -1.6% |  |  |  |
|  | -1.4% | -2.1% | -2.2% | -0.9% | -1.5% | -1.6% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 113% |  |  | 111% |  |  |  |  |  |

# Conclusion

This contribution reports the cross-verification results of RCE1 Test 1.1 on the LM mode. It was confirmed that the implementation was consistent with the algorithm description and the test results matched those provided by the proponent.

# References

- J. Sole, K. Kawamura, and J. Kim, “HEVC Range Extensions Core Experiment 1 (RCE1): Inter component decorrelation methods,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1121, Jan. 2013.

- J. Kim, “AHG7: The performance of extended intra chroma prediction for non 4:2:0 format,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0240, Jan. 2013.

- D. Flynn and K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1006, Jan. 2013.

- J. Kim, “RCE1: The performance of extended chroma mode for non 4:2:0 format,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0097, Apr. 2013.