| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0086 |
| --- | --- |

| Title: | AHG-17: complexity and performance analysis of SHM1.0 compare to HM8.1 simulcast |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin} @samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This contribution contains performance and complexity analysis of SHM1.0 both IntraBL and RefIdx frameworks compare to single layer coding. Complexity assessment methodology developed for SCE3 &4 by AhG-17 was used. It is reported that memory access SHM1.0 in the worst case doesn’t exceed HEVC. In average for motion compensation test scenarios (RA and LD-P) the coding efficiency compare to HM8.1 simulcast is 18,5% for IntraBL framework and 18,0% for RefIdx framework in terms of Luma BD-rate. Average memory of SHM1.0 is lower than HM8.1 simulcast: 94-96% for IntraBL and 93-94% for RefIdx frameworks. In average for motion compensation test scenarios SHM1.0 IntraBL framework outperforms RefIdx by   0.7% with cost of 1-2% extra memory access in average.

# Introduction

SHM1.0 reference s/w includes 2 frameworks: so-called IntraBL and RefIdx. Both they contain only 2 major tools: interlayer texture prediction and motion vector prediction from base layer. Low-level changes compare to HEVC are allowed in IntraBL framework (such as CU-level signalling for inter-layer texture prediction). RefIdx framework is targeting for high-level only scalable HEVC extension solution. RefIdx framework supposes not to have changes compare to HEVC lower than in slice header.

One of the possible implementation of RefIdx approach is up-sampling (if needed) of whole base layer frame and inclusion it into the enhancement layer reference list. If up-sampling is performed than in this implementation of RefIdx decoder has significant latency. More reasonable decoder implementation for RefIdx framework is local up-sampling for inter-layer predicted blocks only. In this implementation Refidx framework is similar to IntraBL but with PU-level signalling inter-layer texture prediction.

Another difference between 2 frameworks is availability of simplest combined prediction in RefIdx framework: the out-put of RefIdx bi-prediction could be semi-sum of inter-layer and enhancement layer predictions. Based on [2] results (restriction of bi-pred for long-term references in RefIdx framework was tested in Tool Experiment 2) performance impact of this simplest combined prediction is around 0.5% (in random access tests).

# Performance comparison

The performance was tested using common test conditions [1]. Both mandatory (All Intra, RA and LD-P) and optional (LD-B) tests were performed. Results for both mandatory and optional tests are included into the section 7. But only mandatory tests are included into the averaging in this section (in order to simplify comparison with SCE out-put).

Results compare to HM8.1 simulcast are summarized in Table 1. BD-rate is calculated based on base layer plus enhancement layer bit-rate and enhancement layer PSNR [1]. Run-time measurement is from homogenous cluster.

Table 1. Summary of test results vs HM8.1 simulcast.

|  | All Intra tests |  |  |  | Test scenarios with MC (RA, LD-P) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Framework | Luma, BD-rate | Chroma, BD-rate | Enc. Time | Dec. Time | Luma, BD-rate | Chroma, BD-rate | Enc. Time | Dec. Time |
| IntraBL | -27,1% | -26,9% | 114% | 80% | -18,5% | -8,8% | 104% | 96% |
| RefIdx | -26,8% | -26,5% | 114% | 82% | -18,0% | -8,3% | 101% | 97% |
| difference between two |  |  |  |  |  |  |  |  |
| IntraBL vs RefIdx | 0,3% | 0,5% | 99% | 103% | 0,7% | 0,6% | 98% | 101% |

Currently SHM1.0 with IntraBL settings slightly out-performs SHM1.0 RefIdx framework: 0,3% in Intra and 0,7% in MC tests.

# Memory access comparison

The worst case. In SHM1.0 RefIdx framework the MV is restricted to zero on encoder side for inter-layer prediction blocks.  Under this restriction there is no cascading of up-sampling and MC interpolation during inter-layer prediction. Assuming this restriction is mandatory encoder constrain, we can guarantee that the worst case memory access for both IntraBL and RefIdx framework is still interpolation MC for 8x8 bi-predicted PUs. This means that the worst case memory bandwidth of SHVC doesn’t increase HEVC level. In order to meet this requirement it is suggested to make e MV=0 restriction for long –term references in RefIdx framework normative (rather than encoder search restriction is reference s/w).

Average. In a scope of AhG17 activity s/w module for complexity assessment was created [JCTVC-L0440]. This module allows actual memory access and computations during interpolation counting during decoder run. 3 different memory architectures were recommended by NEC for this test: first of them assesses pure memory access and remaining two simulates DDR-2 and DDR-3.

Results of complexity assessment using SHM1.0 bit streams are summarized in Table 2. Memory access and computations are reported for 2 process: MC and inter-layer prediction interpolations.

Table 2. Complexity assessment for HM1.0 vs HM8.1 simulcast.

|  | All Intra tests |  |  |  | Test scenarios with MC (RA, LD-P) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| framework | Pure | DDR-2 | DDR-3 | Mults | Pure | DDR-2 | DDR-3 | Mults |
| IntraBL | INF | INF | INF | INF | 94% | 96% | 96% | 96% |
| RefIdx | INF | INF | INF | INF | 93% | 94% | 94% | 97% |
| RefIdxPic | INF | INF | INF | INF | 134% | 117% | 116% | 149% |
| difference between two |  |  |  |  |  |  |  |  |
| IntraBL vs RefIdx | 91% | 89% | 89% | 98% | 99% | 98% | 98% | 101% |

In “all-intra” test case in single layer coding there is no memory access and computations related to MC interpolation. So we put “INF” mark when SHM is compared with HEVC in “all-intra” test.

In Table 2 “RefIdexPic” indicates RefIdx implementation design with actual whole picture up-sampling. Since up-sampling for regions not predicted as inter-layer texture prediction is redundant overall amount of memory access and computations is much higher for “whole picture up-sampling” than for Refidx framework with block-level up-sampling.

In worst case decoder will perform both bi-prediction MC and up-sampling for the same regions in whole picture up-sampling is used. It means that the worst case memory band width will exceed 23-29% HEVC limit (numbers 23-29% are from AhG17 study for the worst case memory band width [3]).

Due to this significant extra complexity both in average and in the worst case the RefIdx implementation with actual whole picture up-sampling is not considered further in this analysis.

Compare to IntraBL framework the memory access is smaller for RefIdx: 10% smaller in “all-intra” tests and 1-2% smaller in MC tests. This is consistent with performance test results reported in section 2 (slightly higher complexity  slightly better performance).

In order to explain phenomenon of lower average memory access in RefIdx framework (especially in “all-intra” tests) we collected statistics for Luma block sizes predicted using inter-layer texture prediction. This statistics is presented by diagrams on Figures 1-2 (scale is the same on both graphs). The hit-ratio of small inter-layer predicted blocks is higher for IntraBL with SHM1.0 encoder settings. Since prediction cost (both in terms of memory access and computations) is higher for small blocks overall complexity is higher for IntraBL (last line in Table 2).

At the same time in ”all-intra” test the overall amount of samples predicted using inter-layer texture prediction is 13% higher for IntraBL according to collected statistics. The memory access in [3] is measured per picture. So 13% higher amount of samples predicted using inter-layer texture prediction in IntraBL framework explain 11% higher memory access.

Since 10% difference in complexity in “all intra” test (only up-sampling) becomes 1-2% in MC test we may conclude that most part of memory access and computations is associated with MC process but not  with up-sampling.

| Figure 1. Sizes of blocks predicted using inter-layer texture prediction in SHM1.0 IntraBL (“all-intra” tests). |  | Figure 2. Sizes of blocks predicted using inter-layer texture prediction in SHM1.0 RefIdx (“all-intra” tests). |
| --- | --- | --- |

There is one interesting observation from Table 2. This is interesting but average memory access and computation prediction cost is lower in scalable video-codec compare to HEVC single layer coding in MC test scenario. This observation is consistent with data in Table1 (decoding time of HM1.0 is shorter compare to HM8.1). We observe this phenomenon because some portion of samples predicted with MC in single layer coding is predicted using inter-layer texture prediction in SHM1.0. Since bi-directional motion compensation is possible, but up-sampling is rightly equivalent to uni-directional MC the cost is higher for MC compare to inter-layer texture prediction.

# Dynamic range analysis

The maximum accumulated sum in a register and temporal buffer (between horizontal and vertical interpolations) were evaluated in AhG17. The maximum value is accumulated both in case up-sampling and MC interpolation during ½-pel interpolation. Since ½-pel filters are shared between MC and up-sampling SHM1.0 modules dynamic range in SHM1.0 interpolation process is the same with HEVC: no 32 bits register overflow and 16-bits temporal buffers are guaranteed.

# Conclusions

In average computational complexity and memory access of SHM1.0 is 4-6% lower than for HM8.1 simulcast single layer coding, while 18% Luma BD-rate reduction is achieved in motion compensation test scenarios.

The motion vector in inter-layer predicted blocks in RefIdx framework shall be normatively restricted to be 0. This restriction guarantees the worst case memory bandwidth of SHVC will not exceed HEVC limit.

Complexity assessment by AhG17 tool shows negligible difference between IntraBL and RefIdx frameworks (1-2% in motion compensation test scenarios), while IntraBL branch outperforms RefIdx framework 0.7% in terms of Luma BD-rate (0,6% in terms of Chroma BD-rate).

# References

[1] X. Li, J. Boyce, P. Onno, Y. Ye, Common SHM test conditions and software reference configurations, JCTVC-L1009 document, January, 2013.

[2] X. Xiu, Y. He, Y. He, Y. Ye, TE2: Results of test 3.2.1 on inter-layer reference picture placement, JCTVC-L0051 document, January, 2013.

[3] E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440 document, January, 2013.

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Details of tests.

Table 3. SHM1.0 IntraBL vs HM8.1 simulcast performance results

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |  |  |
| Class A | -26,4% | -25,8% | -26,9% |  |  |  |  |  |  |  |  |
| Class B | -20,5% | -19,7% | -19,6% | -31,9% | -32,1% | -32,5% |  |  |  |  |  |
| Overall (Test vs Ref) | -22,2% | -21,4% | -21,7% | -31,9% | -32,1% | -32,5% |  |  |  |  |  |
| Overall (Test vs single layer) | 12,4% | 13,7% | 13,4% | 10,3% | 10,3% | 9,7% |  |  |  |  |  |
| EL only (Test vs Ref) | -34,8% | -34,0% | -34,1% | -57,3% | -57,3% | -57,6% |  |  |  |  |  |
| Enc Time[%] | 115,8% |  |  | 112,8% |  |  |  |  |  |  |  |
| Dec Time[%] | 81,6% |  |  | 78,7% |  |  |  |  |  |  |  |
| Enc Mem[%] | 99,9% |  |  | 100,3% |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |  |  |
|  | Y | U | V | Y | U | V | Y |  | U |  | V |
| Class A | -18,1% | -5,3% | -7,2% |  |  |  | -22,2% |  | -13,5% |  | -13,1% |
| Class B | -15,5% | -8,4% | -6,6% | -26,7% | -19,1% | -17,7% | -20,0% |  | -10,3% |  | -7,0% |
| Overall (Test vs Ref) | -16,2% | -7,5% | -6,8% | -26,7% | -19,1% | -17,7% | -20,6% |  | -11,2% |  | -8,7% |
| Overall (Test vs single layer) | 19,3% | 32,0% | 33,0% | 16,5% | 28,5% | 30,9% | 14,9% |  | 28,9% |  | 32,7% |
| EL only (Test vs Ref) | -26,1% | -18,1% | -17,3% | -48,2% | -41,6% | -40,4% | -32,7% |  | -24,1% |  | -21,6% |
| Enc Time[%] | 103,9% |  |  | 104,1% |  |  | 102,6% |  |  |  |  |
| Dec Time[%] | 96,1% |  |  | 94,0% |  |  | 96,9% |  |  |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  | 100,0% |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |  |  |
|  | Y | U | V | Y | U | V | Y |  | U |  | V |
| Class A | -12,7% | 0,6% | -0,8% |  |  |  | -14,3% |  | -7,1% |  | -5,9% |
| Class B | -11,0% | -6,5% | -4,3% | -22,6% | -15,9% | -14,3% | -13,1% |  | -4,0% |  | -0,1% |
| Overall (Test vs Ref) | -11,5% | -4,4% | -3,3% | -22,6% | -15,9% | -14,3% | -13,4% |  | -4,9% |  | -1,7% |
| Overall (Test vs single layer) | 26,1% | 36,5% | 38,2% | 22,5% | 33,1% | 36,0% | 22,3% |  | 34,4% |  | 39,1% |
| EL only (Test vs Ref) | -18,1% | -11,4% | -10,2% | -40,2% | -33,7% | -32,5% | -20,7% |  | -12,3% |  | -9,1% |
| Enc Time[%] | 104,5% |  |  | 105,1% |  |  | 103,1% |  |  |  |  |
| Dec Time[%] | 97,2% |  |  | 95,4% |  |  | 98,6% |  |  |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  | 100,0% |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U |  | V |  |
| Class A | -11,6% | 1,2% | -0,4% |  |  |  | -14,3% | -7,3% |  | -6,3% |  |
| Class B | -9,8% | -6,2% | -4,4% | -21,6% | -16,3% | -15,0% | -12,3% | -5,7% |  | -2,7% |  |
| Overall (Test vs Ref) | -10,3% | -4,1% | -3,2% | -21,6% | -16,3% | -15,0% | -12,9% | -6,1% |  | -3,7% |  |
| Overall (Test vs single layer) | 28,2% | 37,3% | 38,6% | 24,7% | 33,0% | 35,1% | 23,8% | 33,5% |  | 37,1% |  |
| EL only (Test vs Ref) | -16,8% | -10,9% | -10,0% | -39,1% | -34,0% | -33,0% | -20,1% | -13,5% |  | -11,1% |  |
| Enc Time[%] | 102,3% |  |  | 103,1% |  |  | 101,8% |  |  |  |  |
| Dec Time[%] | 96,5% |  |  | 94,1% |  |  | 97,0% |  |  |  |  |
| Enc Mem[%] | 100,0% |  |  | 99,9% |  |  | 100,0% |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |  |  |

Table 4. SHM1.0 RefIdx vs HM8.1 simulcast performance results

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -26,1% | -24,8% | -25,9% |  |  |  |  |  |  |
| Class B | -20,2% | -18,8% | -18,7% | -31,8% | -32,3% | -32,6% |  |  |  |
| Overall (Test vs Ref) | -21,9% | -20,5% | -20,7% | -31,8% | -32,3% | -32,6% |  |  |  |
| Overall (Test vs single layer) | 12,8% | 15,0% | 14,8% | 10,6% | 9,9% | 9,5% |  |  |  |
| EL only (Test vs Ref) | -34,5% | -33,1% | -33,2% | -57,3% | -57,5% | -57,7% |  |  |  |
| Enc Time[%] | 115,2% |  |  | 112,0% |  |  |  |  |  |
| Dec Time[%] | 84,5% |  |  | 80,2% |  |  |  |  |  |
| Enc Mem[%] | 114,8% |  |  | 112,3% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -17,7% | -5,2% | -7,0% |  |  |  | -21,4% | -12,2% | -11,7% |
| Class B | -14,9% | -7,4% | -5,7% | -26,2% | -19,4% | -17,9% | -19,4% | -9,7% | -5,6% |
| Overall (Test vs Ref) | -15,7% | -6,8% | -6,0% | -26,2% | -19,4% | -17,9% | -19,9% | -10,4% | -7,3% |
| Overall (Test vs single layer) | 20,1% | 33,1% | 34,1% | 17,4% | 28,1% | 30,6% | 15,8% | 30,0% | 34,7% |
| EL only (Test vs Ref) | -25,5% | -17,4% | -16,5% | -47,7% | -41,8% | -40,5% | -31,8% | -23,0% | -19,9% |
| Enc Time[%] | 102,1% |  |  | 100,0% |  |  | 101,6% |  |  |
| Dec Time[%] | 97,1% |  |  | 94,3% |  |  | 98,2% |  |  |
| Enc Mem[%] | 103,5% |  |  | 103,0% |  |  | 102,5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -12,6% | 0,7% | -0,8% |  |  |  | -13,5% | -6,3% | -5,3% |
| Class B | -10,6% | -5,3% | -3,4% | -22,4% | -16,1% | -14,5% | -12,3% | -4,2% | 0,0% |
| Overall (Test vs Ref) | -11,1% | -3,5% | -2,6% | -22,4% | -16,1% | -14,5% | -12,7% | -4,8% | -1,5% |
| Overall (Test vs single layer) | 26,6% | 37,8% | 39,1% | 22,9% | 32,8% | 35,7% | 23,5% | 34,6% | 39,4% |
| EL only (Test vs Ref) | -17,7% | -10,6% | -9,6% | -40,2% | -34,2% | -32,8% | -19,7% | -11,9% | -8,6% |
| Enc Time[%] | 102,9% |  |  | 99,8% |  |  | 102,2% |  |  |
| Dec Time[%] | 97,9% |  |  | 95,4% |  |  | 98,9% |  |  |
| Enc Mem[%] | 104,9% |  |  | 104,2% |  |  | 103,5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -11,5% | 1,6% | 0,1% |  |  |  | -13,9% | -6,6% | -5,2% |
| Class B | -9,6% | -5,1% | -3,0% | -21,5% | -16,1% | -14,7% | -11,9% | -4,8% | -0,8% |
| Overall (Test vs Ref) | -10,2% | -3,2% | -2,1% | -21,5% | -16,1% | -14,7% | -12,5% | -5,3% | -2,1% |
| Overall (Test vs single layer) | 28,5% | 38,7% | 40,2% | 24,8% | 33,2% | 35,6% | 24,3% | 34,6% | 39,5% |
| EL only (Test vs Ref) | -16,6% | -10,1% | -9,0% | -39,3% | -34,2% | -33,0% | -19,6% | -12,6% | -9,3% |
| Enc Time[%] | 101,5% |  |  | 99,4% |  |  | 101,2% |  |  |
| Dec Time[%] | 97,3% |  |  | 94,9% |  |  | 98,1% |  |  |
| Enc Mem[%] | 104,9% |  |  | 104,3% |  |  | 103,6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 5. SHM1.0 RefIdx vs IntraBL performance results

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0,3% | 1,4% | 1,5% |  |  |  |  |  |  |
| Class B | 0,4% | 1,3% | 1,2% | 0,2% | -0,3% | -0,1% |  |  |  |
| Overall (Test vs Ref) | 0,4% | 1,3% | 1,3% | 0,2% | -0,3% | -0,1% |  |  |  |
| Overall (Test vs single layer) | 12,8% | 15,0% | 14,8% | 10,6% | 9,9% | 9,5% |  |  |  |
| EL only (Test vs Ref) | 0,5% | 1,5% | 1,5% | 0,1% | -0,6% | -0,4% |  |  |  |
| Enc Time[%] | 99,5% |  |  | 99,3% |  |  |  |  |  |
| Dec Time[%] | 103,5% |  |  | 101,9% |  |  |  |  |  |
| Enc Mem[%] | 114,9% |  |  | 111,9% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,5% | 0,3% | 0,3% |  |  |  | 1,1% | 1,6% | 1,7% |
| Class B | 0,8% | 1,3% | 1,2% | 0,8% | -0,1% | -0,1% | 0,8% | 0,6% | 1,4% |
| Overall (Test vs Ref) | 0,7% | 1,0% | 1,0% | 0,8% | -0,1% | -0,1% | 0,9% | 0,9% | 1,5% |
| Overall (Test vs single layer) | 20,1% | 33,1% | 34,1% | 17,4% | 28,1% | 30,6% | 15,8% | 30,0% | 34,7% |
| EL only (Test vs Ref) | 1,0% | 1,3% | 1,2% | 1,1% | 0,0% | 0,0% | 1,6% | 1,6% | 2,2% |
| Enc Time[%] | 98,3% |  |  | 96,0% |  |  | 99,0% |  |  |
| Dec Time[%] | 101,0% |  |  | 100,3% |  |  | 101,3% |  |  |
| Enc Mem[%] | 103,5% |  |  | 103,1% |  |  | 102,5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,2% | 0,3% | 0,2% |  |  |  | 0,9% | 1,0% | 0,8% |
| Class B | 0,5% | 1,4% | 1,0% | 0,3% | -0,1% | -0,1% | 0,9% | 0,0% | 0,1% |
| Overall (Test vs Ref) | 0,5% | 1,1% | 0,8% | 0,3% | -0,1% | -0,1% | 0,9% | 0,3% | 0,3% |
| Overall (Test vs single layer) | 26,6% | 37,8% | 39,1% | 22,9% | 32,8% | 35,7% | 23,5% | 34,6% | 39,4% |
| EL only (Test vs Ref) | 0,5% | 1,1% | 0,8% | -0,3% | -0,8% | -0,8% | 1,4% | 0,7% | 0,8% |
| Enc Time[%] | 98,5% |  |  | 95,0% |  |  | 99,2% |  |  |
| Dec Time[%] | 100,7% |  |  | 100,1% |  |  | 100,3% |  |  |
| Enc Mem[%] | 104,9% |  |  | 104,2% |  |  | 103,5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,2% | 0,5% | 0,6% |  |  |  | 0,5% | 0,8% | 1,2% |
| Class B | 0,3% | 1,3% | 1,5% | 0,1% | 0,3% | 0,5% | 0,4% | 0,9% | 1,9% |
| Overall (Test vs Ref) | 0,2% | 1,1% | 1,3% | 0,1% | 0,3% | 0,5% | 0,5% | 0,9% | 1,7% |
| Overall (Test vs single layer) | 28,5% | 38,7% | 40,2% | 24,8% | 33,2% | 35,6% | 24,3% | 34,6% | 39,5% |
| EL only (Test vs Ref) | 0,3% | 1,1% | 1,2% | -0,6% | -0,5% | -0,2% | 0,7% | 1,1% | 2,0% |
| Enc Time[%] | 99,2% |  |  | 96,5% |  |  | 99,5% |  |  |
| Dec Time[%] | 100,8% |  |  | 100,8% |  |  | 101,2% |  |  |
| Enc Mem[%] | 104,9% |  |  | 104,4% |  |  | 103,6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 6. Average memory access and computations for SHM1.0 IntraBL relative to HM8.1 single layer coding.

|  | AI HEVC 2x |  |  |  | AI HEVC 1.5x |  |  |  |  |  |  | HM8.1 simulcast |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |  |  | SHM1.0 IntraBL |  |
| Class A | INF | INF | INF | INF |  |  |  |  |  |  |  |  |  |
| Class B | INF | INF | INF | INF | INF | INF | INF | INF |  |  |  |  |  |
| Overall | INF | INF | INF | INF | INF | INF | INF | INF |  |  |  |  |  |
|  | RA HEVC 2x |  |  |  | RA HEVC 1.5x |  |  |  | RA HEVC SNR |  |  |  |  |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |
| Class A | 92% | 92% | 90% | 91% |  |  |  |  | 97% | 99% | 98% | 88% |  |
| Class B | 95% | 96% | 95% | 89% | 92% | 94% | 93% | 97% | 98% | 100% | 100% | 90% |  |
| Overall | 94% | 95% | 94% | 89% | 92% | 94% | 93% | 97% | 97% | 100% | 100% | 89% |  |
|  | LD-P HEVC 2x |  |  |  | LD-P HEVC 1.5x |  |  |  | LD-P HEVC SNR |  |  |  |  |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |
| Class A | 90% | 91% | 89% | 106% |  |  |  |  | 96% | 100% | 99% | 86% |  |
| Class B | 93% | 95% | 94% | 99% | 91% | 93% | 92% | 111% | 99% | 103% | 103% | 88% |  |
| Overall | 93% | 93% | 93% | 101% | 91% | 93% | 92% | 111% | 98% | 102% | 102% | 88% |  |
|  | LD_B HEVC 2x |  |  |  | LD-B HEVC 1.5x |  |  |  | LD-B HEVC SNR |  |  |  |  |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |
| Class A | 86% | 85% | 84% | 89% |  |  |  |  | 89% | 91% | 90% | 85% |  |
| Class B | 89% | 90% | 90% | 87% | 85% | 86% | 85% | 97% | 91% | 94% | 94% | 88% |  |
| Overall | 88% | 89% | 88% | 87% | 85% | 86% | 85% | 97% | 90% | 93% | 93% | 87% |  |
| Avg. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MC: | 94% | 96% | 96% | 96% |  |  |  |  |  |  |  |  |  |

Table 7. Average memory access and computations for SHM1.0 IntraBL relative to HM8.1 single layer coding.

|  | AI HEVC 2x |  |  |  | AI HEVC 1.5x |  |  |  |  |  |  | HM8.1 simulcast |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |  |  | SHM1.0 RefIdx PU |  |  |
| Class A | INF | INF | INF | INF |  |  |  |  |  |  |  |  |  |  |
| Class B | INF | INF | INF | INF | INF | INF | INF | INF |  |  |  |  |  |  |
| Overall | INF | INF | INF | INF | INF | INF | INF | INF |  |  |  |  |  |  |
|  | RA HEVC 2x |  |  |  | RA HEVC 1.5x |  |  |  | RA HEVC SNR |  |  |  |  |  |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |  |
| Class A | 92% | 93% | 92% | 96% |  |  |  |  | 97% | 99% | 99% | 89% |  |  |
| Class B | 94% | 95% | 94% | 90% | 90% | 91% | 90% | 98% | 98% | 100% | 100% | 90% |  |  |
| Overall | 94% | 94% | 94% | 92% | 90% | 91% | 90% | 98% | 97% | 99% | 99% | 90% |  |  |
|  | LD-P HEVC 2x |  |  |  | LD-P HEVC 1.5x |  |  |  | LD-P HEVC SNR |  |  |  |  |  |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |  |
| Class A | 90% | 90% | 89% | 106% |  |  |  |  | 96% | 98% | 98% | 86% |  |  |
| Class B | 93% | 94% | 94% | 101% | 90% | 90% | 89% | 110% | 98% | 100% | 101% | 89% |  |  |
| Overall | 92% | 93% | 92% | 102% | 90% | 90% | 89% | 110% | 98% | 100% | 100% | 88% |  |  |
|  | LD_B HEVC 2x |  |  |  | LD-B HEVC 1.5x |  |  |  | LD-B HEVC SNR |  |  |  |  |  |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |  |
| Class A | 87% | 86% | 86% | 95% |  |  |  |  | 91% | 92% | 92% | 85% |  |  |
| Class B | 90% | 90% | 90% | 90% | 84% | 85% | 84% | 100% | 93% | 94% | 94% | 88% |  |  |
| Overall | 89% | 89% | 88% | 92% | 84% | 85% | 84% | 100% | 92% | 93% | 94% | 87% |  |  |
|  | 93% | 94% | 94% | 97% |  |  |  |  |  |  |  |  |  |  |

Table 8. Average memory access and computations for SHM1.0 RefIdx relative to SHM1.0 IntraBL.

|  | AI HEVC 2x |  |  |  | AI HEVC 1.5x |  |  |  |  |  |  | HM8.1 simulcast |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |  |  | SHM1.0 RefIdx PU |  |
| Class A | 90% | 88% | 89% | 98% |  |  |  |  |  |  |  |  |  |
| Class B | 91% | 89% | 90% | 98% | 92% | 88% | 89% | 98% |  |  |  |  |  |
| Overall | 91% | 89% | 90% | 98% | 92% | 88% | 89% | 98% |  |  |  |  |  |
|  | RA HEVC 2x |  |  |  | RA HEVC 1.5x |  |  |  | RA HEVC SNR |  |  |  |  |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |
| Class A | 101% | 101% | 101% | 106% |  |  |  |  | 100% | 100% | 100% | 101% |  |
| Class B | 99% | 99% | 99% | 101% | 97% | 96% | 96% | 101% | 100% | 99% | 99% | 100% |  |
| Overall | 100% | 100% | 100% | 103% | 97% | 96% | 96% | 101% | 100% | 99% | 100% | 100% |  |
|  | LD-P HEVC 2x |  |  |  | LD-P HEVC 1.5x |  |  |  | LD-P HEVC SNR |  |  |  |  |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |
| Class A | 100% | 99% | 100% | 101% |  |  |  |  | 99% | 98% | 98% | 100% |  |
| Class B | 100% | 99% | 100% | 102% | 98% | 97% | 97% | 99% | 99% | 97% | 98% | 100% |  |
| Overall | 100% | 99% | 100% | 101% | 98% | 97% | 97% | 99% | 99% | 97% | 98% | 100% |  |
|  | LD_B HEVC 2x |  |  |  | LD-B HEVC 1.5x |  |  |  | LD-B HEVC SNR |  |  |  |  |
|  | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults | 0 | 4 | 5 | Mults |  |
| Class A | 101% | 101% | 102% | 107% |  |  |  |  | 103% | 102% | 102% | 101% |  |
| Class B | 100% | 100% | 100% | 105% | 99% | 98% | 98% | 102% | 102% | 100% | 101% | 100% |  |
| Overall | 101% | 100% | 100% | 105% | 99% | 98% | 98% | 102% | 102% | 101% | 101% | 100% |  |
| Avg. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MC | 99% | 98% | 98% | 101% |  |  |  |  |  |  |  |  |  |
| Intra | 91% | 89% | 89% | 98% |  |  |  |  |  |  |  |  |  |