| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0087 |
| --- | --- |

| Title: | SCE4: De-noising filter for SNR scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin} @samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This contribution contains performance and complexity analysis of de-noising filter proposed for SNR scalability. Tool as tested as part of SCE4. Proposed de-noising filter is implemented as 2D separable 5 taps FIR with 4-bits precision coefficients. De-noising filter is applied only for Luma component if resolutions of reference and enhancement layers are the same (SNR test in common test conditions). Average (SNR RA and LD-P) performance improvement achieved vs SHM1.0 IntraBL anchor is 2,13%. In RefIdx framework average performance improvement is 1,44%. Implementation in IntraBL framework has additional feature: CU level on/off switching, which is the source of higher performance gain in IntraBL framework.

# Introduction

De-noising inter-layer filtering was proposed in [2] and included into SCE4 study. This was 2D non-separable 7-taps FIR:

{1,	-5, 11, 50, 11, -5, 1}/64. 					(1)

If resolution of reference and enhancement layers are the same then no up-sampling interpolation is needed and inter-layer filter is not applied in SHM1.0 (SNR test is common test conditions [1]). In order to equalize inter-layer filter application in case of spatial scalability and SNR scalability we proposed to apply inter-layer filter with de-noising effect for when resolutions of reference and enhancement layers are equal.

Since filter (1) is not longer than inter-layer filter in spatial scalability case and precision of filter coefficients are the same in a worst case memory access, computational complexity and dynamic range of this filtering process is not higher than in SHM1.0 (and not higher than in HEVC). The worst case memory access and computations is interpolation MC for 8x8 bi-predicted PUs.

In order to reduce average filtering complexity and simplify h/w implementation length of de-noising filter (1) was reduced from 7 to 5 taps and precision of filter coefficients was reduced from 6 to 4 bits:

{-1, 3, 12, 3, -1}/16. 						(2)

Spectrum characteristic requirements during filter generation were the same with [2] and so frequency responses are very close (refer to Figure 1) and effect both filter have is the same.

SHM1.0 has 2 frameworks: IntraBL and RefIdx. Proposed tool was implemented for both with only one difference. RefIdx is supposed to be high level syntax only solution (at least parsing part of RefIdx is claimed to be the same with HEVC). But IntraBL framework already has CU-level signalling for inter-layer texture prediction. In suggested for IntraBL framework design de-noising filter can be switch on and off on CU level with 1 bit flag signalling.  This 1bit flag is encoded with ctx after texture_rl_flag[ x0 ][ y0 ] and needed only if texture_rl_flag[ x0 ][ y0 ] = true. Details of syntax change for IntraBL framework are included in to the section 4.

Tool is not applied if spatial resolutions of reference and current layers are different, so there is performance change compare to SHM1.0 in SNR tests only.

De-noising filter is applied for Luma component only.

# Performance and complexity study

The performance was tested using common test conditions [1]. Both mandatory (SNR RA and LD-P) and optional (LD-B) tests were performed for spatial scalability x2 and x1.5 cases. Results for both mandatory and optional tests are included into this section. But only mandatory tests are included into the averaging in this section (in order to simplify comparison with SCE out-put).

Table 1. Summary of test results vs SHM1.0

| Test CSE4: 4.1.1 | Y-BD rate, RA&LD-P | Enc Time, RA&LD-P | Dec Time, RA&LD-P | The worst case |  | Actual |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Memory | Comp. | Memory | Comp. |
| vs IntraBL | -2,1% | 102% | 103% | 100% | 100% | 101% | 106% |
| vs RefIdx | -1,4% | 100% | 127% | 100% | 100% | 102% | 108% |

Results compare to SHM1.0 are summarized in Table 1 for both IntraBL and RefIdx frameworks. BD-rate is calculated based on reference layer plus enhancement layer bit-rate and enhancement layer PSNR [1].

The gain in IntraBL framework is higher due to CU level on/off option. Similar effect can be achieved in RefIdx framework by PU level switching realized w/o low-level changes [4]. With PU level switching the gain in RegIdx framework goes to 2.1% (same with intraBL).

Run-time measurement is from homogenous cluster. In IntraBL framework we implemented proposed inter-layer filter on CU level: only for regions use inter-layer texture prediction access to the reference layer and inter-layer filtering are needed. As a result there is no redundant inter-layer filtering and memory access and decoding time increment 3% only (compare to no inter-layer filtering at all).

In RefIdx framework whole reference layer picture is processed by proposed inter-layer filter before become reference frame for enhancement layer. As a result sometimes inter-layer filtering is redundant (for regions not use inter-layer texture prediction). Decoding time increment is 27% in this case. These results are shown here for demonstration only. We don’t recommend using whole picture inter-layer filtering in RefIdx framework. With loss-less code modification proposed inter-layer filtering can be converted to PU-based processing and decoding time increment compare to “no inter-layer filtering” case will be on a same level (<3%) with IntraBL framework.

Details of performance test results for IntraBL and RefIdx frameworks are shown in Table 2 and Table 3.

| Table 2. Performance in IntraBL framework. | Table 3. Performance in RefIdx framework. |
| --- | --- |
| RA HEVC SNR. Y; U; V. Class A; -3,8%; -0,7%; -0,6%. Class B; -1,0%; 0,1%; 0,3%. Overall (Test vs Ref); -1,8%; -0,1%; 0,0%. Overall (Test vs single layer); 12,7%; 28,7%; 32,7%. EL only (Test vs Ref); -2,7%; -0,9%; -0,7%. Enc Time[%]; 102,0%. Dec Time[%]; 103,6%. Enc Mem[%]; 130,5%. BL Match; Matched. LD-P HEVC SNR. Y; U; V. Class A; -4,9%; -0,9%; -0,7%. Class B; -1,4%; 0,2%; 0,5%. Overall (Test vs Ref); -2,4%; -0,1%; 0,2%. Overall (Test vs single layer); 19,3%; 34,3%; 39,4%. EL only (Test vs Ref); -3,3%; -0,9%; -0,6%. Enc Time[%]; 102,1%. Dec Time[%]; 102,7%. Enc Mem[%]; 130,8%. BL Match; Matched. Optional Tests. LD-B HEVC SNR. Y; U; V. Class A; -3,4%; -0,7%; -0,7%. Class B; -0,8%; -0,1%; 0,2%. Overall (Test vs Ref); -1,5%; -0,3%; -0,1%. Overall (Test vs single layer); 21,8%; 33,1%; 37,0%. EL only (Test vs Ref); -2,1%; -0,8%; -0,6%. Enc Time[%]; 101,5%. Dec Time[%]; 103,4%. Enc Mem[%]; 130,8%. BL Match; Matched. | RA HEVC SNR. Y; U; V. -3,7%; -0,9%; -0,9%. 0,2%; -0,5%; -0,6%. -0,9%; -0,6%; -0,7%. 14,7%; 29,2%; 33,9%. -1,3%; -1,0%; -1,1%. 99,9%. 127,1%. 130,3%. Matched. LD-P HEVC SNR. Y; U; V. -4,9%; -1,1%; -0,9%. -0,8%; -0,7%; -0,5%. -2,0%; -0,8%; -0,7%. 20,9%; 33,5%; 38,5%. -2,7%; -1,5%; -1,3%. 99,8%. 127,8%. 130,7%. Matched. Optional Tests. LD-B HEVC SNR. Y; U; V. -3,2%; -0,8%; -0,7%. -0,1%; -0,9%; -1,1%. -1,0%; -0,9%; -1,0%. 23,1%; 33,5%; 38,1%. -1,3%; -1,2%; -1,3%. 99,9%. 128,1%. 130,7%. Matched. |

In a scope of AhG17 activity s/w module for complexity assessment was created [3]. This module allows actual memory access and computations during interpolation counting during decoder run. 3 different memory architectures were recommended by NEC for this test: first of them assesses pure memory access and remaining two simulates DDR-2 and DDR-3.

Results of complexity assessment using SHM1.0 bit streams are summarized in Table 4 (IntraBL framework) and Table 5 (RefIdx framework). Memory access and computations are reported for 2 processes: MC and inter-layer prediction interpolations.

| Table 4. Complexity assessment in IntraBL framework. | Table 5. Complexity assessment in RefIdx framework. |
| --- | --- |
| RA HEVC SNR. Pure; DDR-2; DDR-3; Mults; Adds. Class A; 102%; 100%; 100%; 106%; 105%. Class B; 101%; 100%; 100%; 103%; 103%. Overall; 101%; 100%; 100%; 104%; 104%. LD-P HEVC SNR. Pure; DDR-2; DDR-3; Mults; Adds. Class A; 104%; 101%; 101%; 112%; 111%. Class B; 103%; 101%; 101%; 107%; 107%. Overall; 103%; 101%; 101%; 109%; 108%. LD-B HEVC SNR. Pure; DDR-2; DDR-3; Mults; Adds. Class A; 102%; 100%; 100%; 107%; 107%. Class B; 101%; 100%; 100%; 103%; 103%. Overall; 101%; 100%; 100%; 104%; 104%. | RA HEVC SNR. Pure; DDR-2; DDR-3; Mults; Adds. 102%; 100%; 100%; 107%; 106%. 102%; 102%; 102%; 105%; 105%. 102%; 101%; 101%; 106%; 105%. LD-P HEVC SNR. Pure; DDR-2; DDR-3; Mults; Adds. 103%; 100%; 100%; 112%; 111%. 104%; 102%; 102%; 110%; 109%. 103%; 102%; 101%; 110%; 110%. LD-B HEVC SNR. Pure; DDR-2; DDR-3; Mults; Adds. 102%; 100%; 100%; 108%; 108%. 103%; 102%; 102%; 107%; 106%. 102%; 101%; 101%; 107%; 107%. |

During complexity assessment we assume PU base inter-layer filtering for RefIdx framework. Memory access increment compare to anchor is 2% for RefIdx and 1% for IntraBL.  We have this small difference due to CU level on/off switching in IntraBL frame work (some inter-layer predicted block doesn’t use de-noising filtering).

Because of the same reason the increment of computational complexity due to proposed inter-layer filtering is also smaller for IntraBL framework (6%) compare to 8% in RefIdx.

# Conclusions

Based on test results:

Average Luma BD-rate gain: 2.1% (IntraBL) and 1.4% (RefIdx)

Average Memory access: 101% (IntraBL) and 102% (RefIdx)

Samsung proposes adoption of 2D separable de-noising filter into next SHVC Test Model and reference software.

We would like to thank Inter Digital for careful and comprehensive cross-check of presented test data.

# References

[1] X. Li, J. Boyce, P. Onno, Y. Ye, Common SHM test conditions and software reference configurations, JCTVC-L1009 document, January, 2013.

[2] E. Alshina, A. Alshin, J.H.Park (Samsung), On integer samples filtering during up-sampling, JCTVC-L0087 document, January, 2013.

[3] E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440 document, January, 2013.

[4] W. Pu, V. Seregin, J. Chen, X. Li, M. Karczewicz (Qualcomm), E. Alshina, A. Alshin, Y.Cho (Samsung), Non-SCE4: Switchable Filter on Integer Position, JCTVC-M0273 document, April, 2013

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification text changes.

Following modifications (highlighted in yellow) are needed for specification in chapters G.8.1.3.1.3, G.8.1.3.1.5 and H.7.3.9.5.

- G.8.1.3.1.3. Luma sample interpolation process

Inputs to this process are

–	a (nPicWRL)x(nPicHRL) array rlPicSampleL,

–	a luma sample location ( xP, yP ) relative to the top-left luma sample of the current picture.

Output of this process is a interpolated luma sample value intLumaSampleL.

- Error: Reference source not found specifies the 8-tap filter coefficients fL[ p, x ] with p = 0..165 and x = 0..7 used for the luma resampling process.

Table G-1 16-phase luma resampling filter

| phase p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 2 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 3 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 4 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 5 | −1 | 4 | -11 | 52 | 26 | -8 | 3 | -1 |
| 6 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 7 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 8 | −1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 |
| 9 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 10 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 11 | −1 | 3 | -8 | 26 | 52 | -11 | 4 | -1 |
| 12 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 13 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 14 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 15 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 16 | 0 | -4 | 12 | 48 | 12 | -4 | 0 | 0 |

The value of the interpolated luma sample IntLumaSample  is derived by applying the following ordered steps:

- The derivation process for reference layer sample locations used in resampling as specified in subclause  is invoked with cIdx equal to 0 and luma sample location ( xP, yP ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample  as output.

- The variables xRef and xPhase are derived by

xRef     = ( xRef16 >> 4 )	(G-xx)
xPhase = ( xRef16 ) % 16	(G-xx)

- The variables yRef and yPhase are derived by

yRef     = ( yRef16 >> 4 )	(G-xx)
yPhase = ( yRef16 ) % 16	(G-xx)

- The sample value tempArray[ n] with n = 0..7, is derived as follows.

yPosRL = Clip3( 0, nPicHRL – 1, yRef + n – 1 ) 	(G-xx)

tempArray[n] = fL[ xPhase, 0 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef – 3), yPosRL ] +
	fL[ xPhase, 1 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef – 2), yPosRL ] +
	fL[ xPhase, 2 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef – 1), yPosRL ] +
	fL[ xPhase, 3 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef      ), yPosRL ] +
	fL[ xPhase, 4 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef + 1), yPosRL ] +	(G-xx)
	fL[ xPhase, 5 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef + 2), yPosRL ] +
	fL[ xPhase, 6 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef + 3), yPosRL ] +
	fL[ xPhase, 7 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef + 4), yPosRL ]

- The interpolated luma sample value intLumaSample is derived as follows.

intLumaSample = ( fL[ yPhase, 0 ] * tempArray [ 0 ] +	
	fL[ yPhase, 1 ] * tempArray [ 1 ] +	
	fL[ yPhase, 2 ] * tempArray [ 2 ] +	
	fL[ yPhase, 3 ] * tempArray [ 3 ] +	
	fL[ yPhase, 4 ] * tempArray [ 4 ] +	(G-xx)
	fL[ yPhase, 5 ] * tempArray [ 5 ] +	
	fL[ yPhase, 6 ] * tempArray [ 6 ] +	
	fL[ yPhase, 7 ] * tempArray [ 7 ] + 211) >> 12

intLumaSample = Clip3( 0, ( 1 << BitDepthY) – 1 , intLumaSample )	(G-xx)

- G 8.1.3.1.5. Derivation process for reference layer sample locations

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left picture sample of the color component of the current picture specified by cIdx,

–	denosing_flt_flag value at  a sample location ( xP, yP ),

Output of this process is a sample location (xRef16, yRef16 ) specifying the collocated reference sample location in units of 1/16-th sample.

If cIdx is equal to 0 and denosing_flt_flag is true then xRef16 = 16 and yRef16 =16

Otherwise,

The variable xRef16 is derived as follow:

xRef16 = ( xP * nPicWRL * 16 + nPicW / 2) / nPicW					(G-xx)

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

yRef16 = (yP * nPicHRL * 16 + nPicH / 2 ) / nPicH				(G-xx)

–	Otherwise, the variable yRef16 is derived as follow:

yRef16 = ( yP * nPicHRL * 16 + nPicH / 2 ) / nPicH – offset	(G-xx)

where the value of offset is derived as specified by the following:

if (nPicH is equal to nPicHRL)
			offset = 0
		otherwase if (nPicH is equal to 1.5 * nPicHRL)
			offset = 1
		otherwase if (nPicH is equal to 2.0 * nPicHRL)
			offset = 2

- H.7.3.9.5. Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( nuh_layer_id > 0 && InterLayerTextureRlEnableFlag) { |  |
| texture_rl_flag[ x0 ][ y0 ] | ae(v) |
| if( texture_rl_flag[ x0 ][ y0 ] && slice_denoising_flt_flag) { |  |
| denosing_flt_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( ! texture_rl_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSizeY ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| Else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if(( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) ¦¦ PredMode[ x0 ][ y0 ] = = MODE_TEXTURE_RL) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

denosing_flt_flag [ x0 ][ y0 ] specifies denoising filter application the prediction mode for the current coding unit. denosing_flt_flag [ x0 ][ y0 ] equal to 1 specifies that the prediction samples are derived using reconstructed samples of the reference layer representation with filtering of int-pel positions; denosing_flt_flag [ x0 ][ y0 ] equal to 0 specifies that for current coding unit of int-pel positions filtering is not applied.

When denosing_flt_flag is encoded only if texture_rl_flag is equal to 1  and slice_denoising_flt_flag is true.

When denosing_flt_flag [ x0 ][ y0 ] not present, it is inferred to be equal to 0.

The slice_denoising_flt_flag is not encoded. The slice_denoising_flt_flag  is true if both horizontal and vertical resolutions of reference and current layer are the same: (nPicWRL == nPicWRL )&& (nPicHRL== nPicHRL). Otherwise slice_denoising_flt_flag  is false.