| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0088 |
| --- | --- |

| Title: | AHG-17,13: complexity and performance analysis of different length up-sampling filters in SHM1.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin} @samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This contribution contains performance and complexity analysis of SHM1.0 both IntraBL and RefIdx frameworks with up-sampling filters of different length.  Complexity assessment methodology developed for SCE3 &4 by AhG-17 was used. Briefly by shorten up-sampling filter to 6 taps Luma and 2 taps Chroma we can achieve 3% average memory access reduction with 0,1%(×2) and 0,4%(×1,5) Luma BD-rate performance drop. There is 0,3%(×2) and 0,4%(×1,5) Chroma BD-rate gain from this change. There is not reduction for the memory band-width in a worst case. Average numbers are reported for motion compensation test scenarios.

# Introduction

SHM1.0 reference s/w includes 2 frameworks: so-called IntraBL and RefIdx. Both they contain only 2 major tools: interlayer texture prediction and motion vector prediction from base layer. The main source of SHM1.0 gain is inter-layer texture prediction which requires up-sampling in spatial scalability case.

Following HEVC motion compensation interpolation filter design SHM1.0 uses 8 taps Luma and 4 taps Chroma up-sampling filters. For the same target phase shifts (0, 1/2, 1/4, 3/4, 1/8, 3/8, 5/8, 7/8) up-sampling interpolation filters are the same with motion compensation HEVC filter. It means that HEVC table for interpolation filters in 64 bytes only. It goes to 192 bytes if 8-taps Luma and 4-taps Chroma up-samplign filters for 16 phases are added (it was taken into account that 4 Luma filters and 8 Chroma filters are shared between MC and up-sampling module).  If Luma up-sampling filter is 6-taps and Chroma up-sampling filter is 2 taps then overall size for interpolation filter coefficients storage is exactly 192 bytes (less memory for each up-sampling filter, but no sharing between MC and up-sampling processes).

In this paper we study performance/complexity impact of reduction up-sampling filter length to 6 taps for Luma and to 2 taps for Chroma.

Shorter up-sampling filters were generated using the same methodology (so-called DCTIF) with SHM1.0 up-sampling filters. Difference in filter coefficients is reflected in Table 1 for Luma and Table 2 for Chroma up-sampling interpolation.

Table 1. Luma up-sampling filters.

| Phase shift | 8 taps Luma up-sampling filter | 6 taps Luma up-sampling filter |
| --- | --- | --- |
| 0 | { 0, 0, 0, 64, 0, 0, 0, 0}/64 | { 0, 0, 0, 64, 0, 0, 0, 0}/64 |
| 1/3 | {-1, 4, -11, 52, 26, -8, 3, -1}/64 | { 2, -10, 52, 26, -8, 2}/64 |
| 1/2 | {-1, 4, -11, 40, 40, -11, 4, -1}/64 | {2, -9, 39, 39, -9, 2}/64 |
| 2/3 | {-1, 3, -8, 26, 52, -11, 4, -1}/64 | {2, -8, 26, 52, -10, 2}/64 |

Table 2. Chroma up-sampling filters.

| Phase shift | 4 taps Chroma up-sampling filter | 2 taps Chroma up-sampling filter |
| --- | --- | --- |
| 0 | {0, 64, 0, 0}/64 | { 64, 0}/64 |
| 1/4 | {-4, 54, 16, -2}/64 | { 48, 16}/64 |
| 1/3 | {-5, 50, 22, -3}/64 | { 42, 22}/64 |
| 3/8 | {-6, 46, 28, -4}/64 | { 40, 24}/64 |
| 1/2 | {-4, 36, 36, -4}/64 | { 32, 32}/64 |
| 7/12 | {-4, 30, 43, -5}/64 | { 26, 38}/64 |
| 2/3 | {-3, 22, 50, -5}/64 | { 22, 42}/64 |
| 7/8 | { -2, 10, 58, -2 }/64 | { 8, 56}/64 |
| 11/12 | {-1, 5, 62, -2}/64 | { 6, 58}/64 |

For target phase shift ½ 6-taps DCTIF was already proposed once for HEVC MC in JCTVC-F247[3].

For target phase shift 1/3 (used for scalability ×1.5) frequency responses for 6-taps Luma DCTIF up-sampling filters compare to SHM1.0 8-taps up-sampling are shown on figures 1 and 2. Additionally we show 6-taps filter proposed by FastVDO. Straight line shows frequency response of ideal filter.

| Figure 1. Frequency response in amplitude domain for 1/3 filters: DCTIF 6 taps SHM1.0 8-taps filter 6-taps filter proposed by FastVDO (5 bits) | Figure 2. Frequency response in phase domain for 1/3 filters: DCTIF 6 taps SHM1.0 8-taps filter 6-taps filter proposed by FastVDO (5 bits) |
| --- | --- |

For target phase shift 1/2 (used for scalability ×2) frequency response for 6-taps Luma DCTIF up-sampling filters compare to SHM1.0 8-taps up-sampling are shown on figure 3. Again for comparison we show 6-taps filter proposed by FastVDO.

Figure 3. Frequency response in amplitude domain for ½ filters: DCTIF 6 taps SHM1.0 8-taps filter 6-taps filter proposed by FastVDO (5 bits)

Due to lower coefficients precision of FastVDO’s filter deviation from ideal frequency response is noticeably higher compare to DCTIF. Especially in phase domain low-frequency part of spectrum will be shifted by ~0,28 instead 1/3. Error accumulation due to the bomb in amplitude frequency response for FastVDO’s filter is higher.

# Performance comparison

The performance was tested using common test conditions [1]. Both mandatory (All Intra, RA and LD-P) and optional (LD-B) tests were performed for spatial scalability x2 and x1.5 cases. Results for both mandatory and optional tests are included into the section 7. But only mandatory tests are included into the averaging in this section (in order to simplify comparison with SCE out-put).

Results compare to SHM1.0 are summarized in Table 3 for both IntraBL and RefIdx frameworks. BD-rate is calculated based on reference layer plus enhancement layer bit-rate and enhancement layer PSNR [1]. Run-time measurement from homogenous cluster shows under noise level variation and not reported.

Surprisingly reduction of Chroma up-sampling filter taps provides small Chroma BD-rate gain. But in terms of Luma BD-rate there is always performance degradation.

Table 3. Summary of test results vs HM8.1 simulcast.

| Test and framework | All Intra tests |  |  |  | Test scenarios with MC (RA, LD-P) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ×2 |  | ×1.5 |  | ×2 |  | ×1.5 |  |
|  | Luma, BD-rate | Chroma, BD-rate | Luma, BD-rate | Chroma, BD-rate | Luma, BD-rate | Chroma, BD-rate | Luma, BD-rate | Chroma, BD-rate |
| IntraBL |  |  |  |  |  |  |  |  |
| L6C4 | 0,30% | 0,33% | 0,39% | 0,46% | 0,05% | 0,00% | 0,24% | 0,08% |
| L8C2 | 0,15% | -0,19% | 0,34% | -0,62% | 0,05% | -0,34% | 0,21% | -0,47% |
| L6C2 | 0,44% | 0,14% | 0,73% | -0,15% | 0,10%? | -0,28% | 0,44% | -0,41% |
| RefIdx |  |  |  |  |  |  |  |  |
| L6C4 | 0,30% | 0,33% | 0,39% | 0,46% | 0,06% | 0,09% | 0,24% | 0,13% |
| FastVDO | 0,67% | 0,85% | 1,10% | 1,30% | 0,22% | 0,61% | 1,01% | 0,87% |
| L8C2 | 0,15% | -0,02% | 0,34% | -0,33% | 0,06% | -0,17% | 0,20% | -0,30% |
| L6C2 | 0,45% | 0,31% | 0,73% | 0,14% | 0,10% | -0,08% | 0,44% | -0,17% |

Performance degradation is about twice higher in “all-intra” test (compare to MC test scenario). If we reduce up-sampling filter length then we degrade the quality for inter-layer prediction. In RA and LD-P tests MC prediction successfully replaces inter-layer prediction after this change. Since there is no MC prediction in “all-intra” test losses are higher (0,73% in average for ×1.5 scalability).

Performance degradation is twice higher for ×1.5 scalability than for ×2 scalability. The explanation for this the phase shift used in ×2 scalability is ½. This fractional position is hardest for interpolation due to the longest distance to reference samples.

# Memory access comparison

The worst case. In SHM1.0 the worst case memory access is MC interpolation for 8×8 bi-predicted PUs. In RefIdx framework the smallest inter-layer prediction unit size is 4×8 and for IntraBl framework - 8×8. Up-sampling filters in SHM1.0 have the same length and coefficients precision with HEVC interpolation MC filters. So up-sampling is equivalent to MC uni-prediciton and so the up-sampling complexity in the worst case is within HEVC limit. Shorten up-sampling filter for inter-layer texture prediction doesn’t help the worst case memory band-width reduction.

Average. In a scope of AhG17 activity s/w module for complexity assessment was created [2]. This module allows actual memory access and computations during interpolation counting during decoder run. 3 different memory architectures were recommended by NEC for this test: first of them assesses pure memory access and remaining two simulates DDR-2 and DDR-3.

Table 4. Complexity assessment for IntraBL framework.

|  | All Intra tests |  |  |  | Test scenarios with MC (RA, LD-P) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tool | Pure | DDR-2 | DDR-3 | Mults | Pure | DDR-2 | DDR-3 | Mults |
| L6C4 | 83% | 95% | 94% | 77% | 98% | 99% | 99% | 96% |
| L8C2 | 86% | 82% | 85% | 89% | 99% | 98% | 98% | 98% |
| L6C2 | 70% | 77% | 79% | 66% | 97% | 97% | 97% | 93% |

Results of complexity assessment using SHM1.0 bit streams are summarized in Table 4-5 (for IntraBL and RefIdx frameworks). Memory access and computations are reported for 2 process: MC and inter-layer prediction interpolations.

Table 5. Complexity assessment for RefIdx framework.

|  | All Intra tests |  |  |  | Test scenarios with MC (RA, LD-P) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tool | Pure | DDR-2 | DDR-3 | Mults | Pure | DDR-2 | DDR-3 | Mults |
| L6C4 | 85% | 96% | 95% | 77% | 98% | 99% | 99% | 95% |
| L8C2 | 87% | 82% | 85% | 89% | 99% | 98% | 98% | 98% |
| L6C2 | 72% | 78% | 80% | 67% | 97% | 97% | 97% | 93% |

Since in “all-intra” test inter-layer texture prediction is the only one tool which requires memory access we observe (up to 30%) reduction of memory access due to shorter up-sampling filter. But we shall not forget that this reduction is accomplished by almost 1% BD-rate performance drop.

In MC test scenario (which is actual use-case) reduction of up-sampling filter size saves only 3% memory access. But performance drop is not that small (0,43%) in this case.

There is one more observation. If one shortens up-sampling filter size then higher reduction of memory access is observed if pure memory access is counted (up to 30% reduction). But considering modern memory architectures (DDR-2, DDR-3) benefits from shortening up-sampling filter already less impressive (23% for the same test).

# Conclusions

We would like to summarize presented test results as follows. Changing of DCTIF up-sampling filter from 8-taps Luma/4-taps Chroma design to 6-taps Luma/2-taps Chroma leads to 0,43%(×1.5) / 0,1% (×2) Luma BD-rate drop in MC test scenario. Higher drop is expected for arbitrary scalability ratio. In average this modification saves only 3% memory band-width. And there is no memory band-width reduction in a worst case.

# References

[1] X. Li, J. Boyce, P. Onno, Y. Ye, Common SHM test conditions and software reference configurations, JCTVC-L1009 document, January, 2013.

[2] E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440 document, January, 2013.

[3] E. Alshina, A.Alshin, CE3: DCT derived interpolation filter test by Samsung., JCTVC-F247 document, July, 2011.

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Details of tests.

Table 6. Performance test results of 6-taps DCTIF Luma up-sampling filter instead 8-taps in SHM1.0 (IntraBL).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,5% | 0,5% | 0,5% |  |  |  |
| Class B | 0,2% | 0,3% | 0,3% | 0,4% | 0,5% | 0,5% |
| Overall (Test vs Ref) | 0,3% | 0,3% | 0,3% | 0,4% | 0,5% | 0,5% |
| Overall (Test vs single layer) | 12,8% | 14,0% | 13,8% | 10,8% | 10,8% | 10,2% |
| EL only (Test vs Ref) | 0,6% | 0,6% | 0,6% | 1,2% | 1,3% | 1,3% |
| Enc Time[%] | 100,1% |  |  | 100,0% |  |  |
| Dec Time[%] | 100,4% |  |  | 99,4% |  |  |
| Enc Mem[%] | 99,9% |  |  | 99,6% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,30% | 0,33% |  | 0,39% | 0,46% |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,2% | 0,2% | 0,2% |  |  |  |
| Class B | 0,1% | 0,0% | 0,1% | 0,2% | 0,1% | 0,1% |
| Overall (Test vs Ref) | 0,1% | 0,1% | 0,1% | 0,2% | 0,1% | 0,1% |
| Overall (Test vs single layer) | 19,5% | 32,1% | 33,2% | 16,8% | 28,7% | 31,0% |
| EL only (Test vs Ref) | 0,2% | 0,2% | 0,2% | 0,7% | 0,6% | 0,6% |
| Enc Time[%] | 100,1% |  |  | 99,9% |  |  |
| Dec Time[%] | 100,4% |  |  | 100,5% |  |  |
| Enc Mem[%] | 99,9% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,0% | 0,0% |  |  |  |
| Class B | -0,1% | -0,1% | -0,1% | 0,3% | 0,1% | 0,1% |
| Overall (Test vs Ref) | 0,0% | -0,1% | -0,1% | 0,3% | 0,1% | 0,1% |
| Overall (Test vs single layer) | 26,1% | 36,4% | 38,1% | 22,8% | 33,2% | 36,0% |
| EL only (Test vs Ref) | 0,0% | -0,1% | -0,1% | 0,7% | 0,5% | 0,5% |
| Enc Time[%] | 100,1% |  |  | 99,9% |  |  |
| Dec Time[%] | 100,4% |  |  | 101,1% |  |  |
| Enc Mem[%] | 99,9% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,05% | 0,00% |  | 0,24% | 0,08% |  |

Table 7. Performance test results of 6-taps DCTIF Luma up-sampling filter instead 8-taps in SHM1.0 (RefIdx).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,5% | 0,5% | 0,5% |  |  |  |
| Class B | 0,2% | 0,3% | 0,3% | 0,4% | 0,5% | 0,5% |
| Overall (Test vs Ref) | 0,3% | 0,3% | 0,3% | 0,4% | 0,5% | 0,5% |
| Overall (Test vs single layer) | 13,2% | 15,4% | 15,1% | 11,0% | 10,4% | 10,0% |
| EL only (Test vs Ref) | 0,6% | 0,6% | 0,6% | 1,2% | 1,3% | 1,3% |
| Enc Time[%] | 99,9% |  |  | 99,9% |  |  |
| Dec Time[%] | 100,1% |  |  | 100,2% |  |  |
| Enc Mem[%] | 99,9% |  |  | 100,7% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,30% | 0,33% |  | 0,39% | 0,46% |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,3% | 0,2% | 0,2% |  |  |  |
| Class B | 0,1% | 0,1% | 0,1% | 0,2% | 0,2% | 0,1% |
| Overall (Test vs Ref) | 0,1% | 0,1% | 0,1% | 0,2% | 0,2% | 0,1% |
| Overall (Test vs single layer) | 20,3% | 33,3% | 34,3% | 17,6% | 28,3% | 30,8% |
| EL only (Test vs Ref) | 0,2% | 0,2% | 0,2% | 0,7% | 0,7% | 0,7% |
| Enc Time[%] | 100,2% |  |  | 100,1% |  |  |
| Dec Time[%] | 101,5% |  |  | 101,1% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,1% | 0,1% |  |  |  |
| Class B | -0,1% | 0,0% | 0,1% | 0,3% | 0,2% | 0,0% |
| Overall (Test vs Ref) | 0,0% | 0,0% | 0,1% | 0,3% | 0,2% | 0,0% |
| Overall (Test vs single layer) | 26,6% | 37,8% | 39,2% | 23,2% | 33,0% | 35,7% |
| EL only (Test vs Ref) | 0,0% | 0,1% | 0,1% | 0,7% | 0,7% | 0,5% |
| Enc Time[%] | 100,2% |  |  | 100,1% |  |  |
| Dec Time[%] | 100,6% |  |  | 99,8% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,06% | 0,09% |  | 0,24% | 0,13% |  |

Table 8. Performance test results of 2-taps DCTIF Chroma up-sampling filter instead 4-taps in SHM1.0 (IntraBL).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | -0,2% | -0,3% |  |  |  |
| Class B | 0,2% | -0,1% | -0,2% | 0,3% | -0,6% | -0,7% |
| Overall (Test vs Ref) | 0,2% | -0,2% | -0,2% | 0,3% | -0,6% | -0,7% |
| Overall (Test vs single layer) | 12,6% | 13,5% | 13,1% | 10,7% | 9,7% | 9,0% |
| EL only (Test vs Ref) | 0,3% | -0,1% | -0,2% | 1,0% | -0,1% | -0,3% |
| Enc Time[%] | 99,9% |  |  | 100,1% |  |  |
| Dec Time[%] | 100,0% |  |  | 99,2% |  |  |
| Enc Mem[%] | 100,0% |  |  | 99,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,15% | -0,19% |  | 0,34% | -0,62% |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,2% | -0,3% |  |  |  |
| Class B | 0,1% | -0,2% | -0,2% | 0,2% | -0,1% | -0,1% |
| Overall (Test vs Ref) | 0,1% | -0,1% | -0,2% | 0,2% | -0,1% | -0,1% |
| Overall (Test vs single layer) | 19,4% | 31,9% | 32,8% | 16,8% | 28,4% | 30,8% |
| EL only (Test vs Ref) | 0,1% | 0,0% | -0,2% | 0,7% | 0,3% | 0,4% |
| Enc Time[%] | 100,0% |  |  | 99,9% |  |  |
| Dec Time[%] | 100,3% |  |  | 100,5% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,3% | -0,7% |  |  |  |
| Class B | 0,0% | -0,6% | -0,5% | 0,2% | -0,9% | -0,8% |
| Overall (Test vs Ref) | 0,0% | -0,5% | -0,6% | 0,2% | -0,9% | -0,8% |
| Overall (Test vs single layer) | 26,1% | 35,9% | 37,4% | 22,8% | 32,0% | 35,0% |
| EL only (Test vs Ref) | 0,0% | -0,5% | -0,6% | 0,5% | -0,7% | -0,5% |
| Enc Time[%] | 100,0% |  |  | 99,8% |  |  |
| Dec Time[%] | 100,4% |  |  | 99,0% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,05% | -0,34% |  | 0,21% | -0,47% |  |

Table 9. Performance test results of 2-taps DCTIF Chroma up-sampling filter instead 4-taps in SHM1.0 (RefIdx).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,0% | -0,1% |  |  |  |
| Class B | 0,2% | 0,0% | 0,0% | 0,3% | -0,3% | -0,4% |
| Overall (Test vs Ref) | 0,2% | 0,0% | 0,0% | 0,3% | -0,3% | -0,4% |
| Overall (Test vs single layer) | 13,0% | 15,1% | 14,7% | 11,0% | 9,6% | 9,1% |
| EL only (Test vs Ref) | 0,3% | 0,1% | 0,0% | 1,0% | 0,2% | 0,1% |
| Enc Time[%] | 100,0% |  |  | 99,9% |  |  |
| Dec Time[%] | 100,0% |  |  | 100,1% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,2% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,15% | -0,02% |  | 0,34% | -0,33% |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,2% | -0,1% |  |  |  |
| Class B | 0,1% | -0,1% | 0,0% | 0,2% | 0,1% | 0,2% |
| Overall (Test vs Ref) | 0,1% | 0,0% | 0,0% | 0,2% | 0,1% | 0,2% |
| Overall (Test vs single layer) | 20,2% | 33,2% | 34,2% | 17,6% | 28,2% | 30,9% |
| EL only (Test vs Ref) | 0,1% | 0,1% | 0,1% | 0,7% | 0,6% | 0,7% |
| Enc Time[%] | 100,2% |  |  | 100,0% |  |  |
| Dec Time[%] | 101,4% |  |  | 101,8% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,1% | -0,5% |  |  |  |
| Class B | 0,0% | -0,5% | -0,3% | 0,2% | -0,8% | -0,7% |
| Overall (Test vs Ref) | 0,0% | -0,4% | -0,3% | 0,2% | -0,8% | -0,7% |
| Overall (Test vs single layer) | 26,7% | 37,4% | 38,7% | 23,1% | 31,8% | 34,8% |
| EL only (Test vs Ref) | 0,1% | -0,3% | -0,3% | 0,5% | -0,5% | -0,5% |
| Enc Time[%] | 100,2% |  |  | 100,2% |  |  |
| Dec Time[%] | 100,9% |  |  | 100,4% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,06% | -0,17% |  | 0,20% | -0,30% |  |

Table 10. Performance test results of 6-taps Luma/2-taps Chroma DCTIF p-sampling filter instead 8/4-taps in SHM1.0 (IntraBL).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,6% | 0,3% | 0,2% |  |  |  |
| Class B | 0,4% | 0,1% | 0,1% | 0,7% | -0,1% | -0,2% |
| Overall (Test vs Ref) | 0,4% | 0,2% | 0,1% | 0,7% | -0,1% | -0,2% |
| Overall (Test vs single layer) | 12,9% | 13,9% | 13,5% | 11,1% | 10,2% | 9,5% |
| EL only (Test vs Ref) | 0,8% | 0,5% | 0,4% | 2,2% | 1,2% | 1,0% |
| Enc Time[%] | 100,1% |  |  | 100,0% |  |  |
| Dec Time[%] | 100,6% |  |  | 100,0% |  |  |
| Enc Mem[%] | 99,7% |  |  | 99,4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,44% | 0,14% |  | 0,73% | -0,15% |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,3% | 0,3% | -0,1% |  |  |  |
| Class B | 0,1% | -0,1% | -0,1% | 0,4% | 0,0% | 0,0% |
| Overall (Test vs Ref) | 0,2% | 0,0% | -0,1% | 0,4% | 0,0% | 0,0% |
| Overall (Test vs single layer) | 19,6% | 32,1% | 32,9% | 17,1% | 28,6% | 31,0% |
| EL only (Test vs Ref) | 0,4% | 0,2% | 0,1% | 1,4% | 0,9% | 1,0% |
| Enc Time[%] | 100,2% |  |  | 100,0% |  |  |
| Dec Time[%] | 100,3% |  |  | 101,5% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | -0,2% | -0,5% |  |  |  |
| Class B | 0,0% | -0,6% | -0,5% | 0,4% | -0,8% | -0,9% |
| Overall (Test vs Ref) | 0,0% | -0,5% | -0,5% | 0,4% | -0,8% | -0,9% |
| Overall (Test vs single layer) | 26,1% | 35,9% | 37,5% | 23,1% | 32,2% | 34,9% |
| EL only (Test vs Ref) | 0,0% | -0,5% | -0,5% | 1,1% | -0,2% | -0,2% |
| Enc Time[%] | 100,1% |  |  | 100,0% |  |  |
| Dec Time[%] | 100,1% |  |  | 99,1% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,10% | -0,28% |  | 0,44% | -0,41% |  |

Table 11. Performance test results of 6-taps Luma/2-taps Chroma DCTIF p-sampling filter instead 8/4-taps in SHM1.0 (RefIdx).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,6% | 0,5% | 0,4% |  |  |  |
| Class B | 0,4% | 0,3% | 0,2% | 0,7% | 0,2% | 0,1% |
| Overall (Test vs Ref) | 0,4% | 0,3% | 0,3% | 0,7% | 0,2% | 0,1% |
| Overall (Test vs single layer) | 13,3% | 15,4% | 15,1% | 11,4% | 10,1% | 9,6% |
| EL only (Test vs Ref) | 0,8% | 0,7% | 0,6% | 2,2% | 1,5% | 1,5% |
| Enc Time[%] | 100,1% |  |  | 100,0% |  |  |
| Dec Time[%] | 100,3% |  |  | 100,1% |  |  |
| Enc Mem[%] | 99,8% |  |  | 99,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,45% | 0,31% |  | 0,73% | 0,14% |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,3% | 0,4% | 0,1% |  |  |  |
| Class B | 0,2% | 0,0% | 0,1% | 0,4% | 0,3% | 0,3% |
| Overall (Test vs Ref) | 0,2% | 0,2% | 0,1% | 0,4% | 0,3% | 0,3% |
| Overall (Test vs single layer) | 20,4% | 33,4% | 34,3% | 17,9% | 28,4% | 31,0% |
| EL only (Test vs Ref) | 0,4% | 0,3% | 0,3% | 1,4% | 1,2% | 1,3% |
| Enc Time[%] | 100,3% |  |  | 100,2% |  |  |
| Dec Time[%] | 101,4% |  |  | 101,7% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,2% | -0,1% | -0,4% |  |  |  |
| Class B | -0,1% | -0,5% | -0,1% | 0,4% | -0,6% | -0,6% |
| Overall (Test vs Ref) | 0,0% | -0,4% | -0,2% | 0,4% | -0,6% | -0,6% |
| Overall (Test vs single layer) | 26,6% | 37,4% | 38,8% | 23,4% | 32,0% | 34,9% |
| EL only (Test vs Ref) | 0,0% | -0,3% | -0,2% | 1,2% | 0,1% | 0,1% |
| Enc Time[%] | 100,3% |  |  | 100,3% |  |  |
| Dec Time[%] | 101,6% |  |  | 100,9% |  |  |
| Enc Mem[%] | 100,0% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | 0,10% | -0,08% |  | 0,44% | -0,17% |  |