| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0090 |
| --- | --- |

| Title: | Low-pass filter for Combined Inter Prediction. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexander Alshin Elena Alshina 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 { alexander_b.alshin, elena_a.alshina }@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This contribution contains description of new prediction mode for SHVC which combines inter-layer prediction texture with enhancement layer motion compensation (MC) prediction. Application of 2D separable symmetrical 5-taps low-pass filter allows increasing influence of low frequencies from Inter layer prediction and preserving high frequencies of enhancement layer in resulting prediction. Proposed new combined prediction mode is enabled on CU level for Inter CUs. This additional mode provides average 1.7% (Luma) and 7,9% (Chroma ) BD-rate gain compare to SHM1.0 (IntraBL framework). If proposed combined mode is restricted for 2Nx2N Inter CUs and combination with bi-pred is not allowed for smallest CUs then performance gain is 1.5% (Luma) and 6,9% (Chroma) but the worst case memory band-width is within HEVC limit and encoding time overhead is 3% only.

# Introduction

The pixel-wise linear combination of Inter-layer texture prediction and enhancement layer MC prediction is the key idea of combined prediction mode (JCTVC-K0036) and adaptive predictor compensation (JCTVC-K0033). They both were studied in TEB3 (JCTVC-L0206[1] and JCTVC-L0072[2]) and then in SCE3. Such kind of combination improves both Inter layer texture and enhancement layer MC predictions since their errors are not correlated.  However Inter layer texture prediction has specific properties (such as blur as demonstrated on the Figure 1).

Figure 1. Original signal (left) inter layer prediction (right).

Inter-layer texture prediction is results of up-sampling or results of decoding with high QP (low quality). Using up-sampling signal we cannot distinguish fine details of signal which is corresponded high band in frequency domain from the other side we may expect that motion compensation on enhancement layer can predict fine details more precisely. So the goal of this contribution is to construct new combination of inter-layer texture prediction (PredBL) and enhancement layer prediction (PredEL) which will preserve high frequency band of PredEL and amplify low frequency band of PredBL.

# Proposed method

The simplest combination which is semi-sum of PredBL and PredEL predictions was studied in [1, 2]:

ComibedPredEL=(PredBL+ PredEL +1)>>1                      (1)

Here another combination is proposed instead:

ComibedPredEL =clip( PredBL+ PredEL +Â·[PredBL - PredEL] +1)>>1            (2)

Where Â·[ ] is 2D separable symmetrical filter which in each direction is 5-taps FIR filter with following coefficients:

{ 11, 13, 16, 13, 11}/64.                               (3)

The frequency response of this filter is shown on Figure 2. Additionally frequency response of de-noising filter [5] is shown. This figure explains that operator Â preserves only very narrow diapason of low frequencies.

Figure 2. Frequency response: black line is proposed filter.

Rewriting the formula (2) we obtain the following equivalent form:

ComibedPredEL =( (I+ Â)·[PredBL] + (I- Â)·[PredEL] +1)>>1        (4)

Here I is identity operator. From formula (4) it is clear that low-pass band from PredBL is enhanced; but d low-pass band PredEL signal is dumped compare to combination (1). So, (2) works as weighted combination in frequency domain.

For illustration different terms in (2) in comparison with original signal are shown on Figure 3. Up-sampled base layer signal (PredBL) predicts well original signal, but without any fine details. Prediction from enhancement layer (PredEL) contains a lot fine details, but only some of them exist in original signal, remaining are just coding error. Semi-sum (3) provides regularization but weight assigned for low-pass band is not high enough, in opposite coding noise is still present in semi-sum. Proposed CombinedPredEL predicts original signal the best from all 4 variants.

Figure 3. Comparison of predictions and original signal.

# Performance test results

For performance test CombinePredEL(2) was implemented as additional mode which is enabled on CU level (similar to [1]). Both [1] (SCE3_test3.2) and proposed combined prediction were implemented for Luma component only. In our implementation the difference between PredBL and PredEL was calculated inside predicted block only. During filtration Â· for [PredBL - PredEL] with coefficients () we assume that difference (PredBL - PredEL) is equal boundary value outside of prediction block. So compare to implementation of (1) in SCE3_test3.2 there is no additional memory access for proposed combined prediction. But there are additional computations for 5-taps symmetrical filter (3).

Proposed combined prediction was tested according to [3] in random access and low-delay-P test cases (both spatial and SNR scalability). Optional low-delay-B test results are also provided but not included into the averaging. Average Luma and Chroma BD-rate together with Encoding/Decoding time for proposed tool compare to SCE3_test_3.2 are summarized in table 1. More details about this test can be found in section 8 and attached excel spreadsheet.

Run-time is quite similar for both algorithms. Despite combined prediction is applied for Luma only there is significant Chroma BD-rate gain (probably due to the current lambda settings in SHM1.0).

Proposed combined prediction provides extra 0,5% (Luma) and 2,8% (Chroma) BD-rate gain compare to semi-sum (1).

Table 1. Performance comparison between SCE3 test 3.2 and proposed CombinedPredEL (applied for all part sizes)

| Test | Luma, BD-rate | Chroma, BD-rate | EncTime | DecTime |
| --- | --- | --- | --- | --- |
| SCE3 test3.2 | -1,24% | -5,16% | 114% | 100% |
| CombinedPredEL | -1,74% | -7,93% | 115% | 102% |

During analysis of combined prediction usage we have found that most frequently combined prediction is used in Inter CUs with 2N×2N partition size. This observation is consistent with other SCE3 tests.

In order to reduce the worst case memory band-width, speed-up encoder and reduce side information we have restricted combined mode application for 2N×2N only. With this restriction the most part of the gain is preserved, but encoder time increment is down to 3% only (compare to 15% if all Inter CUs are allowed for combination). Results for SCE3 test 3.2 with mentioned restriction are described in [6].

Table 2. Performance comparison between SCE3 test 3.2 and proposed CombinedPredEL (only for 2N×2N mode.

| Test | Luma, BD-rate | Chroma, BD-rate | EncTime | DecTime |
| --- | --- | --- | --- | --- |
| SCE3 test3.2 | -0,99% | -4,00% | 104% | 100% |
| CombinedPredEL | -1,56% | -7,05% | 103% | 100% |

As it will be shown in section 4 even is combine mode is restricted for 2N×2N only the worst case memory band-width for some memory architectures exceeds HEVC the worst case. In order to keep the worst case memory bandwidth under HEVC limit we disallow combination of 8×8 PredBL with bi-directional MC from enhancement layer (but combination with enhancement layer uni-pred is allowed).

The performance test results for combined predictions with the worst case memory band-width under HEVC limit are summarized in Table 3.

In all variants proposed Combined Prediction provides more than 0,5% extra Luma BD-rate gain and about 3% extra Chroma BD-rate gain compare to the simplest variant of Combined prediction (which is semi-sum).

Table 3. Performance comparison between SCE3 test 3.2 and proposed CombinedPredEL (the worst case memory band-width for both within HEVC limit).

| Test | Luma, BD-rate | Chroma, BD-rate | EncTime | DecTime |
| --- | --- | --- | --- | --- |
| SCE3 test3.2 | -0,95% | -3,95% | 104% | 100% |
| CombinedPredEL | -1,50% | -6,88% | 103% | 101% |

# Complexity assessment

For proposed combined prediction complexity assessment was done according to [4]. Results for the theoretical worst case are summarized in Table 4. Numbers in Table 4 show the worst case memory access and computations for combined mode. So numbers under 100% indicate that combined mode is less complex compare the absolutely worst case in HEVC (which is 8x8 MC bi-prediction).

In Tables 4 and 5 “2Nx2N” indicates constrain of combined mode for 2Nx2N partitioning only and “R” (“restricted”) indicates that additionally combination of inter-layer and MC bi-predictions is prohibited in smallest CUs (8x8). For “R” variant the worst case memory access doesn’t exceed HEVC limit.

Table 4. Theoretical the worst case memory band—width and computations.

| Test | Mults | Adds | MemBand 4x2 | MemBand 8x2 |
| --- | --- | --- | --- | --- |
| SCE_test3.2 | 145% | 145% | 133% | 133% |
| CombinePredEL | 169% | 168% | 133% | 133% |
| 2Nx2N CombinePredEL | 169% | 168% | 133% | 133% |
| R-CombinePredEL | 134% | 133% | 100% | 100% |

During additional decoder run actual memory access was evaluated for 3 different memory architectures. An actual memory access and computations, estimated during additional decoder, are summarized in Table 5.

Table 5. Actual memory access and computations compare to anchor.

| Test | PureMemory | DDR2 | DDR3 | Mults |
| --- | --- | --- | --- | --- |
| SCE_test3.2 | 106% | 105% | 105% | 106% |
| CombinePredEL | 110% | 108% | 108% | 113% |
| 2Nx2N CombinePredEL | 109% | 107% | 108% | 113% |
| R-CombinePredEL | 108% | 106% | 106% | 112% |

# Conclusions

Based on presented test results

- 1,50% average Luma BD-rate gain in SHM1.0

- 6,88% average Chroma BD-rate gain in SHM1.0

- Only 3% Encoding time increment

- The worst case memory access and computations within HEVC limit

- Only 6-8% average memory band-width increment compare to SHM1.0

- No additional reference frame access (such as GRP-like tools)

Samsung suggests adoption of proposed combined prediction into the next version of SHVC test model. Restriction of combined prediction for 2Nx2N Inter CUs only and disallowing combination with bi-pred in smallest CUs guarantees the worst case memory band-width within HEVC limit.

# References

[1] V. Seregin, J. Chen, M. Karczewicz, TE3: Combined inter mode (test 4.6.2.2) JCTVC-L0206 document, January, 2013.

[2] T.-D. Chuang, Y.-W. Huang, P. Lai, S. Liu, S. Lei TE3-4.3.1: Adaptive predictor compensation by using reconstructed BL texture. JCTVC-L0072 document, January, 2013.

[3] X. Li, J. Boyce, P. Onno, Y. Ye, Common SHM test conditions and software reference configurations, JCTVC-L1009 document, January, 2013.

[4] E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440 document, January, 2013.

[5] E. Alshina, A. Alshin, SCE4: De-noising filter for SNR scalability, JCTVC-M0087 document, April, 2013.

[6] E. Alshina, A. Alshin Non SCE3: Simplified design for combined prediction (test 3.2), JCTVC-M0092 document, April, 2013.

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Details of tests.

Table 6. Performance test results of proposed Combined Prediction w/o restrictions vs SHM1.0 IntraBL.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,8% | -5,9% | -6,8% |  |  |  | -0,9% | -4,6% | -6,2% |
| Class B | -0,7% | -5,1% | -6,2% | -0,8% | -6,2% | -7,4% | -1,0% | -6,6% | -8,2% |
| Overall (Test vs Ref) | -0,8% | -5,3% | -6,4% | -0,8% | -6,2% | -7,4% | -1,0% | -6,0% | -7,6% |
| Overall (Test vs single layer) | 18,4% | 24,7% | 24,4% | 15,6% | 20,3% | 21,1% | 13,7% | 20,4% | 21,8% |
| EL only (Test vs Ref) | -1,4% | -6,1% | -7,2% | -1,5% | -7,5% | -8,8% | -1,3% | -6,7% | -8,5% |
| Enc Time[%] | 118,0% |  |  | 112,8% |  |  | 113,2% |  |  |
| Dec Time[%] | 102,3% |  |  | 102,3% |  |  | 102,0% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2,0% | -7,6% | -8,6% |  |  |  | -2,9% | -6,6% | -8,7% |
| Class B | -2,1% | -6,0% | -7,3% | -2,1% | -9,2% | -10,6% | -3,2% | -10,1% | ##### |
| Overall (Test vs Ref) | -2,1% | -6,4% | -7,7% | -2,1% | -9,2% | -10,6% | -3,1% | -9,1% | ##### |
| Overall (Test vs single layer) | 23,4% | 27,4% | 27,3% | 19,9% | 20,4% | 21,2% | 18,5% | 21,2% | 22,3% |
| EL only (Test vs Ref) | -3,2% | -7,7% | -9,0% | -3,8% | -11,3% | -12,9% | -4,3% | -10,5% | ##### |
| Enc Time[%] | 120,0% |  |  | 112,9% |  |  | 114,8% |  |  |
| Dec Time[%] | 102,0% |  |  | 101,0% |  |  | 100,5% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,2% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1,1% | -6,6% | -7,2% |  |  |  | -1,5% | -5,3% | -6,9% |
| Class B | -1,4% | -5,0% | -6,0% | -1,1% | -7,2% | -8,1% | -1,5% | -6,6% | -8,0% |
| Overall (Test vs Ref) | -1,3% | -5,5% | -6,3% | -1,1% | -7,2% | -8,1% | -1,5% | -6,2% | -7,7% |
| Overall (Test vs single layer) | 26,5% | 29,6% | 29,6% | 23,3% | 23,3% | 24,2% | 21,8% | 24,7% | 26,0% |
| EL only (Test vs Ref) | -2,1% | -6,4% | -7,3% | -2,5% | -8,9% | -9,8% | -2,1% | -7,1% | -8,6% |
| Enc Time[%] | 115,4% |  |  | 110,2% |  |  | 111,4% |  |  |
| Dec Time[%] | 103,2% |  |  | 103,3% |  |  | 103,2% |  |  |
| Enc Mem[%] | 100,2% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -1,7% | -7,9% |  |  |  |  |  |  |  |
|  | 115% |  |  |  |  |  |  |  |  |
|  | 102% |  |  |  |  |  |  |  |  |

Table 7. Performance test results of proposed Combined Prediction restricted for 2Nx2N mode only vs SHM1.0 IntraBL.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,6% | -4,5% | -5,4% |  |  |  | -0,8% | -3,8% | -5,4% |
| Class B | -0,6% | -4,3% | -5,2% | -0,6% | -5,2% | -6,3% | -0,9% | -5,8% | -7,2% |
| Overall (Test vs Ref) | -0,6% | -4,3% | -5,2% | -0,6% | -5,2% | -6,3% | -0,8% | -5,2% | -6,7% |
| Overall (Test vs single layer) | 18,6% | 26,2% | 26,0% | 15,8% | 21,6% | 22,6% | 13,8% | 21,6% | 23,2% |
| EL only (Test vs Ref) | -1,1% | -5,0% | -6,0% | -1,3% | -6,4% | -7,6% | -1,2% | -5,9% | -7,6% |
| Enc Time[%] | 103,5% |  |  | 101,0% |  |  | 103,0% |  |  |
| Dec Time[%] | 100,5% |  |  | 100,6% |  |  | 99,8% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1,8% | -6,6% | -7,5% |  |  |  | -2,7% | -5,8% | -7,8% |
| Class B | -2,0% | -5,4% | -6,6% | -1,9% | -8,5% | -9,7% | -3,0% | -9,3% | -11,2% |
| Overall (Test vs Ref) | -1,9% | -5,7% | -6,9% | -1,9% | -8,5% | -9,7% | -2,9% | -8,3% | -10,2% |
| Overall (Test vs single layer) | 23,6% | 28,5% | 28,5% | 20,1% | 21,5% | 22,5% | 18,8% | 22,5% | 23,8% |
| EL only (Test vs Ref) | -3,0% | -6,9% | -8,1% | -3,7% | -10,6% | -12,0% | -4,0% | -9,7% | -11,7% |
| Enc Time[%] | 104,2% |  |  | 100,3% |  |  | 103,6% |  |  |
| Dec Time[%] | 101,2% |  |  | 100,9% |  |  | 99,9% |  |  |
| Enc Mem[%] | 100,2% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1,0% | -5,7% | -6,4% |  |  |  | -1,4% | -4,7% | -6,2% |
| Class B | -1,3% | -4,6% | -5,7% | -1,0% | -6,6% | -7,5% | -1,4% | -6,1% | -7,4% |
| Overall (Test vs Ref) | -1,2% | -4,9% | -5,9% | -1,0% | -6,6% | -7,5% | -1,4% | -5,7% | -7,1% |
| Overall (Test vs single layer) | 26,6% | 30,4% | 30,4% | 23,3% | 24,1% | 25,0% | 22,0% | 25,6% | 27,0% |
| EL only (Test vs Ref) | -2,1% | -5,9% | -6,8% | -2,5% | -8,3% | -9,3% | -2,0% | -6,5% | -8,0% |
| Enc Time[%] | 103,0% |  |  | 100,2% |  |  | 102,4% |  |  |
| Dec Time[%] | 100,6% |  |  | 99,9% |  |  | 100,0% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -1,6% | -7,0% |  |  |  |  |  |  |  |
|  | 103% |  |  |  |  |  |  |  |  |
|  | 100% |  |  |  |  |  |  |  |  |

Table 8. Performance test results of proposed Low Complexity Combined Prediction (the worst case is within HEVC limit)  vs SHM1.0 IntraBL.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,5% | -3,9% | -4,7% |  |  |  | -0,6% | -3,3% | -4,6% |
| Class B | -0,5% | -4,0% | -4,9% | -0,5% | -5,1% | -6,0% | -0,7% | -5,6% | -6,9% |
| Overall (Test vs Ref) | -0,5% | -4,0% | -4,9% | -0,5% | -5,1% | -6,0% | -0,7% | -4,9% | -6,2% |
| Overall (Test vs single layer) | 18,7% | 26,7% | 26,5% | 15,9% | 21,8% | 22,9% | 14,1% | 22,0% | 23,9% |
| EL only (Test vs Ref) | -1,0% | -4,7% | -5,6% | -1,3% | -6,3% | -7,4% | -1,0% | -5,6% | -7,0% |
| Enc Time[%] | 103,8% |  |  | 101,2% |  |  | 103,3% |  |  |
| Dec Time[%] | 102,0% |  |  | 101,6% |  |  | 101,7% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1,8% | -6,6% | -7,5% |  |  |  | -2,7% | -5,8% | -7,8% |
| Class B | -2,0% | -5,4% | -6,6% | -1,9% | -8,5% | -9,7% | -3,0% | -9,3% | -11,2% |
| Overall (Test vs Ref) | -1,9% | -5,7% | -6,9% | -1,9% | -8,5% | -9,7% | -2,9% | -8,3% | -10,2% |
| Overall (Test vs single layer) | 23,6% | 28,5% | 28,5% | 20,1% | 21,5% | 22,5% | 18,8% | 22,5% | 23,8% |
| EL only (Test vs Ref) | -3,0% | -6,9% | -8,1% | -3,7% | -10,6% | -12,0% | -4,0% | -9,7% | -11,7% |
| Enc Time[%] | 104,2% |  |  | 100,3% |  |  | 103,6% |  |  |
| Dec Time[%] | 101,2% |  |  | 100,9% |  |  | 99,9% |  |  |
| Enc Mem[%] | 100,2% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,9% | -5,4% | -6,0% |  |  |  | -1,2% | -4,3% | -5,5% |
| Class B | -1,2% | -4,5% | -5,4% | -0,9% | -6,6% | -7,4% | -1,2% | -6,2% | -7,3% |
| Overall (Test vs Ref) | -1,1% | -4,8% | -5,6% | -0,9% | -6,6% | -7,4% | -1,2% | -5,6% | -6,8% |
| Overall (Test vs single layer) | 26,8% | 30,7% | 30,8% | 23,5% | 24,1% | 25,1% | 22,2% | 25,7% | 27,4% |
| EL only (Test vs Ref) | -1,9% | -5,6% | -6,4% | -2,3% | -8,3% | -9,1% | -1,7% | -6,4% | -7,7% |
| Enc Time[%] | 103,3% |  |  | 100,6% |  |  | 102,9% |  |  |
| Dec Time[%] | 103,0% |  |  | 103,3% |  |  | 102,0% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -1,5% | -6,9% |  |  |  |  |  |  |  |
|  | 103% |  |  |  |  |  |  |  |  |
|  | 101% |  |  |  |  |  |  |  |  |