| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0091 |
| --- | --- |

| Title: | Non SCE1-Combined Intra Prediction with low-pass filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexander Alshin Elena Alshina 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 { alexander_b.alshin, elena_a.alshina}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This contribution contains description of new Intra prediction that is mixture of inter-layer texture and enhancement layer (EL) Intra prediction. 2D separable symmetrical 5-taps low-pass filter is used for non-trivial combination of two predictions. This allows saving of low frequencies of inter-layer texture prediction and high frequencies of EL in combination. Proposed tool shows in average 0,6% Luma and 1,8% Chroma BD-rate gain (all-intra ×2) and 0,2% Luma and 0,8% Chroma BD-rate gain (all-intra ×1,5).

# Introduction

Inter layer texture prediction is shown at the Figure 1.

Figure 1. Inter layer prediction (left), original signal (right).

Using up-sampling signal we cannot distinguish fine details of signal which correspond high band in frequency domain. From the other side we can expect that the boundary samples of EL which are used for Intra prediction provide necessary information about high frequencies. The DC correction of INTRA was studied in JCTVC-L0036 [1] where in one prediction unit every pixel of enhancement layer Intra prediction was adjusted by constant to reach DC value of corresponding up-sampled base layer (BL). However not only DC value can be corrected using up-sampled base layer signal. Our goal is to compensate other low frequencies additionally to DC. It can be done with forward transform; coefficients cleaning and inverse transform chain. But the same effect can be reached by application of low-pass filter. So our goal is to construct the mixture of BL and EL prediction which will save high frequencies of EL and low frequencies from BL.

# Proposed method

For each Intra PU on enhancement layer we modify Intra prediction as follows

Pred’EL= PredEL +Â·[PredEL - PredEL],                          (1)

Here

- PredEL is enhancement layer Intra prediction for current PU,

- PredEL is up-sampled collocated base layer block,

- Â·[ ] is low-pass filter. It is 2D separable symmetrical, 5-taps FIR filter with following coefficients:

α = { 11, 13, 16, 13, 11}/64.                              (2)

Filtration is implemented as follows:

Â•[Y]=Bhor[Bver [Y]]

shift = 12

The frequency response of the filter (2) is shown on Figure 2.

Figure 2. Frequency response of proposed low-pass filter.

Rewriting formula (1) to the following equivalent form can be obtained

Pred’El=  Â·[PredBl] + (I- Â)·[PredEl],                           (3)

Here I is identity operator. It becomes clear that summand Â·[PredBl] preserves low frequencies from PredBL, while summand (I- Â)·[PredEl] saves high frequencies from enhancement layer prediction PredEL.

Proposed combined prediction is applied for Luma component only in our implementation.

# Test result

Proposed combined prediction was tested according to [2] in all-intra test cases (for spatial scalability ×1,5 and × 2).

Performance test results of proposed Low pass filter in Intra Combined Prediction vs SHM1.0 IntraBL are summarized in Table 1.

Table 1. Performance of proposed Combined Intra Prediction compare to SHM1.0 (IntraBL framework)

Accurate run-time measurement will be added in the later revision of this document.

# Conclusions

Proposed combined Intra prediction provides 0,6% (Luma) /1,8% (Chroma) BD-rate gain in “all-intra ×2” test and 0,2% (Luma)/ 0,8% (Chroma) BD-rate gain in “all-intra ×1,5” test with low additional complexity.

Base on presented results Samsung suggests adoption of proposed combined Intra prediction into the next version of SHVC test model.

# References

[1]  J. Lainema, K. Ugur  TE3: Results of test 4.1.2 on intra DC correction, JCTVC-L0036  document, January, 2013.

[2] X. Li, J. Boyce, P. Onno, Y. Ye, Common SHM test conditions and software reference configurations, JCTVC-L1009 document, January, 2013.

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).