| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0092 |
| --- | --- |

| Title: | Non SCE3: Simplified design for combined prediction (test 3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_b.alshin} @samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

Simplifications for combined prediction tested in SCE4 as test 3.2 are proposed in this contribution. With minor performance degradation encoding time was reduced from 114% to 104%. The worst case memory band-width for combined prediction with suggested modifications is lower than HEVC the worst case. In average simplified version of test 3.2 provides 0,95% Luma and 3,95% Chroma BD-rate gain compare to SHM1.0 (IntraBL framework).

# Introduction

The pixel-wise linear combination of Inter-layer texture prediction and enhancement layer MC prediction is the key idea of combined prediction mode (JCTVC-K0036) and adaptive predictor compensation (JCTVC-K0033). They both were studied in TEB3 ([1] and [2]) and now in SCE3.

This variant of combined prediction provides noticeable gain and requires access to only reconstructed collocated base-layer which is any-way needed for inter-layer texture prediction. Compare to Generalized Residual Prediction (GRP), for example, this variant of combined prediction provides better complexity-performance trade-off on our opinion.

The only draw-backs of simplest version of combined prediction (test 3.2) on pour opinion is not small increment of encoding time (114% compare to SHM1.0) and memory access in the worst case exceeds HEVC limit. In this contribution we try to resolve both these problems.

# Proposed simplifications

In SCE3 test 3.2 combined prediction

ComibedPredEL=(PredBL+ PredEL +1)>>1                      (1)

was implemented as additional mode enabled on CU level. Combined mode in SCE3 test 3.2 was allowed for Inter CUs with any partitioning (including asymmetrical and rectangular). Due to RD-cost check for all these variants encoding time for SCE3 test 3.2 was 14% higher than anchor (run-time measured using Samsung’s homogeneous cluster).

Combination with any-type of motion compensation (MC) prediction existing in HEVC was allowed (including 4x8 uni-pred and 8x8 bi-pred). Implementation-wise up-sampling for combined prediction can be done on CU basis. Since both up-sampling and MC interpolation are performed in a worst case memory band-width exceeds HEVC limit. The worst case memory band-with in HEVC corresponds to 8x8 MC bi-prediction blocks.

Theoretical study for the worst case memory band-width was done as a part of AhG17 activity. For SHM1.0 anchor results were released as a part of JCTVC-L0440 package [4]. In [4] template memory access was measured in number of bytes to read from the memory per one sample prediction. This measurement was done for 2 different memory patterns (4x2 and 8x2) is JCTVC-L0440. In Table 1 we report SCE4 test 3.2 the worst case memory access. Additionally 1D memory patterns (4x1 and 8x1) are included. These memory patterns were studied in HEVC-CE3 (2011).

Table 1. The worst case memory access for SCE4 test 3.2

| Memory pattern | HEVC the worst | SCE3 test 3.2 the worst | relative |
| --- | --- | --- | --- |
| 1x4 | 13,8 | 18,4 | 134% |
| 1x8 | 16,5 | 22,1 | 134% |
| 2x4 | 15,0 | 20,0 | 133% |
| 2x8 | 18,0 | 24,0 | 133% |

During analysis of combined prediction usage we have found that most frequently combined prediction is used in Inter CUs with 2N×2N partition size. This observation is consistent with other SCE3 tests.

In order to speed-up encoder and reduce side information we have restricted combined mode application for 2N×2N only. After this restriction the minimal block size for combined prediction is 8x8. The worst case memory access is still 8x8 MC bi-prediction (so Table 1 reflect the worst case complexity). If this restriction is applied then combined prediction flag is encoded only if Inter CU uses 2Nx2N partiotion.

If additionally for 8x8 CUs combination (1) is allowed only between up-sampled base layer signal and uni-prediction from enhancement layer (8x8 bi-pred is not allowed for combination) then the worst case memory band access for combined mode (1) doesn’t not exceed HEVC worst case, as shown in Table 2 (8x8 bi-pred which is equivalent for 8x8 combined prediction is the worst case).

Table 2. The worst case memory access for combined mode restricted for 2Nx2N and 8x8 bi-pred is not allowed for combination.

| Memory pattern | HEVC the worst | SCE3 test 3.2 (2Nx2N only) the worst | relative |
| --- | --- | --- | --- |
| 1x4 | 13,8 | 13,8 | 100% |
| 1x8 | 16,5 | 16,5 | 100% |
| 2x4 | 15,0 | 15,0 | 100% |
| 2x8 | 18,0 | 18,0 | 100% |

In order not to make scalable HEVC extension more complex than HEVC we would suggest to restrict combined prediction (1) for 2Nx2N Inter CUs and disallow combination between up-sampled base layer signal and bi-prediction for smallest CUs (8x8).

# Performance test results

All tests were performed following SCE3 description: random access and low-delay-P tests were performed in SHM1.0 IntraBL framework.

Brief summary of test results is shown in Table 3.

Table 3. Summary of performance tests.

| Test | Luma, BD-rate | Chroma, BD-rate | Enc. Time | Dec. Time |
| --- | --- | --- | --- | --- |
| SCE3 test3.2 | -1,24% | -5,16% | 114% | 100% |
| Only 2Nx2N | -0,99% | -4,00% | 104% | 100% |
| 2Nx2N & 8x8 bi-pred is not allowed for combination | -0,95% | -3,95% | 104% | 100% |

Detail of test results are shown in Table 4 (2Nx2N only) and Table 5 (memory band-width within HEVC limit).

Table 4. Performance test results if combined prediction (1) restricted for 2Nx2N inter CUs only.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,1% | -1,1% | -1,3% |  |  |  | -0,2% | -1,4% | -1,7% |
| Class B | -0,1% | -1,5% | -1,8% | -0,2% | -2,0% | -2,3% | -0,7% | -3,8% | -4,7% |
| Overall (Test vs Ref) | -0,1% | -1,4% | -1,7% | -0,2% | -2,0% | -2,3% | -0,5% | -3,1% | -3,8% |
| Overall (Test vs single layer) | 19,2% | 30,1% | 30,7% | 16,2% | 25,8% | 27,6% | 14,3% | 24,3% | 26,9% |
| EL only (Test vs Ref) | -0,2% | -1,5% | -1,8% | -0,4% | -2,3% | -2,8% | -0,6% | -3,4% | -4,2% |
| Enc Time[%] | 105,0% |  |  | 103,3% |  |  | 103,8% |  |  |
| Dec Time[%] | 100,6% |  |  | 100,3% |  |  | 101,0% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,2% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,8% | -2,6% | -2,8% |  |  |  | -1,2% | -3,4% | -4,0% |
| Class B | -1,2% | -2,7% | -3,3% | -1,1% | -4,5% | -5,0% | -2,5% | -7,5% | -8,9% |
| Overall (Test vs Ref) | -1,1% | -2,6% | -3,1% | -1,1% | -4,5% | -5,0% | -2,1% | -6,3% | -7,5% |
| Overall (Test vs single layer) | 24,7% | 32,7% | 33,7% | 21,1% | 26,7% | 28,8% | 19,8% | 25,1% | 27,5% |
| EL only (Test vs Ref) | -1,4% | -3,1% | -3,6% | -2,1% | -5,7% | -6,2% | -2,8% | -7,2% | -8,4% |
| Enc Time[%] | 105,3% |  |  | 103,1% |  |  | 104,1% |  |  |
| Dec Time[%] | 100,4% |  |  | 99,8% |  |  | 99,7% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,2% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 5. Performance test results if memory band-width of combined prediction (1) is kept within HEVC limit.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,1% | -0,9% | -1,0% |  |  |  | -0,1% | -1,1% | -1,3% |
| Class B | -0,1% | -1,4% | -1,7% | -0,2% | -1,9% | -2,3% | -0,5% | -3,8% | -4,6% |
| Overall (Test vs Ref) | -0,1% | -1,2% | -1,5% | -0,2% | -1,9% | -2,3% | -0,4% | -3,1% | -3,7% |
| Overall (Test vs single layer) | 19,2% | 30,3% | 30,9% | 16,3% | 25,8% | 27,7% | 14,4% | 24,4% | 27,2% |
| EL only (Test vs Ref) | -0,1% | -1,4% | -1,6% | -0,4% | -2,3% | -2,7% | -0,5% | -3,4% | -4,1% |
| Enc Time[%] | 105,2% |  |  | 103,5% |  |  | 104,0% |  |  |
| Dec Time[%] | 100,0% |  |  | 100,3% |  |  | 101,2% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,8% | -2,6% | -2,8% |  |  |  | -1,2% | -3,4% | -4,0% |
| Class B | -1,2% | -2,7% | -3,3% | -1,1% | -4,5% | -5,0% | -2,5% | -7,5% | -8,9% |
| Overall (Test vs Ref) | -1,1% | -2,6% | -3,1% | -1,1% | -4,5% | -5,0% | -2,1% | -6,3% | -7,5% |
| Overall (Test vs single layer) | 24,7% | 32,7% | 33,7% | 21,1% | 26,7% | 28,8% | 19,8% | 25,1% | 27,5% |
| EL only (Test vs Ref) | -1,4% | -3,1% | -3,6% | -2,1% | -5,7% | -6,2% | -2,8% | -7,2% | -8,4% |
| Enc Time[%] | 105,4% |  |  | 103,1% |  |  | 104,2% |  |  |
| Dec Time[%] | 100,7% |  |  | 100,2% |  |  | 100,3% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Complexity assessment

For proposed combined prediction complexity assessment was done according to [4]. Results for the theoretical worst case are summarized in Tables 1-3. Numbers in these tables show the worst case memory access and computations for combined mode. So numbers equal to 100% indicate that combined mode is no more less complex compare the absolutely worst case in HEVC (which is 8x8 MC bi-prediction).

During additional decoder run actual memory access was evaluated for 3 different memory architectures. An actual memory access and computations are summarized in Table 5.

Table 5. Actual memory access and computations compare to anchor.

| Test | PureMemory | DDR2 | DDR3 | Mults |
| --- | --- | --- | --- | --- |
| SCE_test3.2 | 106% | 105% | 105% | 106% |
| + restricted for 2Nx2N | 105% | 104% | 104% | 105% |
| + no combination with bi-pred for 8x8 CUs | 105% | 103% | 103% | 105% |

# Conclusions

Samsung suggests keeping the worst case memory band-width of HEVC scalable extension within HEVC limits. In order to achieve this with minor performance drop combined prediction tested in SCE3 test 3.2 should be restricted for 2Nx2N Inter CUs only and combination with bi-pred should be not allowed for 8x8 CUs. Even with these restrictions combined prediction provides in average 0,95% Luma and 3,95% Chroma BD-rate gain compare to SHM1.0, with only 4% encoding time increment.

# References

[1] V. Seregin, J. Chen, M. Karczewicz, TE3: Combined inter mode (test 4.6.2.2) JCTVC-L0206 document, January, 2013.

[2] T.-D. Chuang, Y.-W. Huang, P. Lai, S. Liu, S. Lei TE3-4.3.1: Adaptive predictor compensation by using reconstructed BL texture. JCTVC-L0072 document, January, 2013.

[3] X. Li, J. Boyce, P. Onno, Y. Ye, Common SHM test conditions and software reference configurations, JCTVC-L1009 document, January, 2013.

[4] E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440 document, January, 2013.

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).