| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0093 |
| --- | --- |

| Title: | SCE2: Deblocking boundary strength modification for Intra BL |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun KimByeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea Liwei Guo Geert Van Der Auwera Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA, 92121 USA | Tel: Email: | [82-10-8893-4332] [chulkeun.kim@lge.com] |
| Source: | LG Electronics, Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, a deblocking modification for intra BL is proposed for HEVC scalable extension. The boundary strengths(BS) of intra BL coded blocks are set to 1. Simulation results reportedly show 0.0%, 0.1%, 0.0%, 0.1%, 0.0% and 0.0% BD-rate savings on average for AI-2x, AI-1.5x, RA-2x, RA-1.5x, LDP-2x, and LDP-1.5x, respectively, compared with SHM 1.0

# Introduction

In SHM 1.0, upsampled base layer samples can be used as sample predictors for enhancement layer coding. In JCTVC-L0069, L0284, a deblocking modification for Intra BL is proposed. This modification was decided to be investigated on top of SHM 1.0 as a part of core experiment 2 (CE2). The rest of this contribution is organized as follows. The proposed algorithm will be described in section 2, draft text modifications will be described in section 3, simulation results will be shown in section 4, and finally conclusions will be given in section 5.

# Proposed method

In SHM 1.0 deblocking, intra BL PU are treated as the intra coded blocks. Therefore BS of intra BL PU is set to 2. If BS is higher than 1, chroma deblocking filter is on. In this contribution, BS decision method for Intra-BL and chroma deblocking on/off decision method are modified as follow.

If at least one of the blocks is coded as intra mode, the filtering strength is set to 2; otherwise, if at least one of the blocks is coded in intra BL mode, then the BS is set to 1. If BS is higher than 0 and at least one of the adjacent blocks are coded as Intra/Intra BL, chroma deblocking filter is on.

# Draft text

In this proposal, the following modifications with yellow highlight are presented.

##### 8.7.2.4 Derivation process of boundary filtering strength

….

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If the sample p0 or q0 is in the luma coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, If the sample p0 or q0 is in the luma coding block of a coding unit coded with intra BL prediction mode, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a luma transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

…

##### 8.7.2.5.1  Vertical edge filtering process

…

The filtering process for edges in the chroma coding blocks of current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1  <<  ( log2CbSize − 3 ).

- For xDk equal to k  <<  2 with k = 0..nD − 1 and yDm equal to m  <<  2 with m = 0..nD − 1, the following applies:

- When bS[ xDk*2 ][ yDm*2 ] is greater than 1 and ( ( ( xC / 2 + xDk ) >> 3 ) << 3 ) is equal to xC / 2 + xDk, the following ordered steps apply.

- When bS[ xDk * 2 ][ yDm * 2 ] is higher than 0 and the sample p0 or q0 is in the luma coding block of a coding unit coded with intra/intra BL prediction mode and ( ( ( xCb / 2 + xDk )  >>  3 )  <<  3 ) is equal to xCb / 2 + xDk, the following ordered steps apply:

- The filtering process for chroma block edges as specified in subclause Error: Reference source not found is invoked with the chroma picture sample array recPictureCb, the location of the chroma coding block ( xCb / 2, yCb / 2 ), the chroma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_VER, and a variable cQpPicOffset set equal to pps_cb_qp_offset as inputs, and the modified chroma picture sample array recPictureCb as output.

- The filtering process for chroma block edges as specified in subclause Error: Reference source not found is invoked with the chroma picture sample array recPictureCr, the location of the chroma coding block ( xCb / 2, yCb / 2 ), the chroma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_VER, and a variable cQpPicOffset set equal to pps_cr_qp_offset as inputs, and the modified chroma picture sample array recPictureCr as output.

…

##### 8.7.2.5.2	Horizontal  edge filtering process

…

The filtering process for edges in the chroma coding blocks of current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1  <<  ( log2CbSize − 3 ).

- For yDm equal to m  <<  2 with m = 0..nD − 1 and xDk equal to k  <<  2 with k = 0..nD − 1, the following applies:

- When bS[ xDk*2 ][ yDm*2 ] is greater than 1 and ( ( (yC / 2 + yDm ) >> 3 ) << 3) is equal to yC / 2 + yDm, the following ordered steps apply.

- When bS[ xDk * 2 ][ yDm * 2 ] is higher than 0 and the sample p0 or q0 is in the luma coding block of a coding unit coded with intra/intra BL prediction mode and ( ( ( yCb / 2 + yDm )  >>  3 )  <<  3 ) is equal to yCb / 2 + yDm, the following ordered steps apply:

- The filtering process for chroma block edges as specified in subclause Error: Reference source not found is invoked with the chroma picture sample array recPictureCb, the location of the chroma coding block ( xCb / 2, yCb / 2 ), the chroma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_HOR, and a variable cQpPicOffset set equal to pps_cb_qp_offset as inputs, and the modified chroma picture sample array recPictureCb as output.

- The filtering process for chroma block edges as specified in subclause Error: Reference source not found is invoked with the chroma picture sample array recPictureCr, the location of the chroma coding block ( xCb / 2, yCb / 2 ), the chroma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_HOR, and a variable cQpPicOffset set equal to pps_cr_qp_offset as inputs, and the modified chroma picture sample array recPictureCr as output.

…

# Results

# On common test conditions

# On additional CE2 sequences

| Sequence | BL QP | EL QP | Scalability |  |  | bitrate | Y psnr | U psnr | V psnr |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DucksTakeOff | 34 | 36 | 2X | RA | SHM1.0 | 2763.2016 | 27.5183 | 31.5425 | 36.1466 |
|  |  |  |  |  | Proposal | 2762.7416 | 27.5174 | 31.5455 | 36.1461 |
| DucksTakeOff | 38 | 34 | SNR | RA | SHM1.0 | 3017.7504 | 28.3323 | 31.7833 | 36.3303 |
|  |  |  |  |  | Proposal | 3015.6728 | 28.3309 | 31.7889 | 36.3297 |
| WestWindsEasy | 34 | 36 | 2X | LDP | SHM1.0 | 990.7768 | 33.1639 | 43.1609 | 46.2575 |
|  |  |  |  |  | Proposal | 991.9512 | 33.1647 | 43.1600 | 46.2564 |
| WestWindsEasy | 38 | 34 | SNR | LDP | SHM1.0 | 1318.3384 | 34.2474 | 43.7977 | 46.7466 |
|  |  |  |  |  | Proposal | 1319.3984 | 34.2463 | 43.7963 | 46.7474 |

# Conclusion

This contribution presents BS modification for Intra BL and Chroma deblocking on/off decision method. Simulation results reportedly show 0.0%, 0.1%, 0.0%, 0.1%, 0.0% and 0.0% BD-rate savings on average for AI-2x, AI-1.5x, RA-2x, RA-1.5x, LDP-2x, and LDP-1.5x, respectively, compared with SHM 1.0

It is proposed to discuss this modification, and adopt this technique.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).