| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0095 |
| --- | --- |

| Title: | SCE1-1.1: Filling unavailable reference samples in intra prediction at enhancement layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | [82-10-8893-4332] [chulkeun.kim@lge.com] |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, filling unavailable reference samples in intra prediction at enhancement layer (EL) is proposed for HEVC scalable extension. Unavailable samples are replaced with the corresponding sample at base layer (BL) for intra prediction. Simulation results reportedly show 0.2%, 0.0% BD-rate savings on average for AI-2x, AI-1.5x, respectively, compared with SHM 1.0 anchors.

# Introduction

Current padding process is as shown in figure 1. Padding is performed when at least one sample is unavailable for intra prediction. Starting from A, padding is performed by copying its previous pixel (Ppre) to the current (Pcur). If A is not available, set A equal to the first available one (Pnext). If all samples are unavailable, (1 << ( BitDepthY -1)) is substituted for the values of all samples. In JCTVC-L0099 [1], filling unavailable reference samples in intra prediction at enhancement layer is proposed for HEVC scalable extension and these techniques were decided to be investigated on top of SHM 1.0 as a part of core experiment 1 (CE1).

Figure 1. Padding process.

# Proposed Methods

In this contribution, if there is any unavailable reference sample during intra prediction EL, the corresponding sample at BL will replace it. For example, in Figure 2, left-bottom reference samples are unavailable and then are replaced with corresponding samples at BL.

Figure 2.  Filling unavailable reference samples in intra prediction at enhancement layer

# Experimental Results

The proposed method was implemented on the SHM 1.0 and the performance was measured under both of intra configuration. Table 1 shows the experimental results of proposed method. Detailed results are described in the accompanied excel file.

Table 1. Experimental results

# References

- C. Kim, B. Jeon, “TE 3: Results of test 4.1.3 on filling unavailable reference samples in intra prediction at enhancement layer,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0099, Jan. 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).