| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0096_r1 |
| --- | --- |

| Title: | AHG 9: On enhancement layer CRA |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | [82-10-8893-4332] [chulkeun.kim@lge.com] |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution suggests further discussing the agreement made in previous meeting about allowing different random access point period across layer when using CRA NAL unit. It is assessed that random access scenario may not work when a CRA picture in higher layer has non-IRAP collocated picture. Two possible options are also suggested to overcome the problem that may resulted from the scenario.

In the first revision of this contribution, the modification to the proposed solutions is described to cover the same issue to the pictures that follow the CRA in decoding order.

# Problem description

In the 12th JCTVC meeting, as proposed in [1], it was agreed to allow CRA NALU type to be used when nuh_layer_id is greater than 0 in which inter-layer prediction is allowed but inter prediction is not. Furthermore, it was also agreed to allow different random access point (RAP) period which means that a CRA NAL unit with nuh_layer_id greater than 0 may have collocated picture with NAL unit type other than IRAP. While the original intension probably aimed at having more IRAP in lower layer than in higher layer, the agreed point opens possibility of having CRA NAL unit in higher layer with collocated picture that is not an IRAP. In such case, the random access scenario may not work.

Figure 1 illustrates the issue when a CRA picture in higher layer has a non-IRAP collocated picture. Picture ‘B’ of the enhancement layer is CRA picture while picture ‘b’ in the lower layer is not an IRAP. As agreed, picture ‘B’ can use inter-layer prediction but not inter prediction. Therefore, picture ‘B’ can refer to picture ‘b’ in the lower layer. However, since picture ‘b’ is not an IRAP, it may refer to other picture, to picture ‘a’ for example. In such situation, random access will not work at picture ‘B’ since picture ‘a’ may not be available. The similar issue is also identified for any pictures that follow the CRA in decoding order until the picture that has corresponding collocated IRAP.

Figure 1: CRA at enhancement layer with non-IRAP collocated picture

We suggest to further discuss the agreement to allow CRA NAL units be not aligned across layers. For starter, in the next section we provide two options to overcome the issue.

# Proposal

Since inter-layer prediction may involve two types: inter-layer syntax prediction and inter-layer texture prediction, it is possible to allow inter-layer syntax prediction in the situation where a higher layer CRA picture has non IRAP collocated picture because it does not require decoder to reconstruct the non IRAP collocated picture. Considering these, there are two possible options to support having a higher layer CRA picture with non IRAP collocated picture, they are:

Option 1: Inter-layer prediction is allowed for CRA NAL units with nuh_layer_id greater than 0, while inter prediction is disallowed. However, when CRA picture with nuh_layer_id greater than 0 has corresponding collocated non-IRAP in the reference layer, inter-layer prediction shall not be allowed for the CRA picture and any pictures follow it in decoding order until the picture with corresponding collocated IRAP in the reference layer.

Option 2: Inter-layer prediction is allowed for CRA NAL units with nuh_layer_id greater than 0, while inter prediction is disallowed. However, when CRA picture with nuh_layer_id greater than 0 has corresponding collocated non-IRAP in the reference layer, inter-layer texture prediction shall not be allowed but inter-layer syntax prediction shall be allowed for the CRA picture and any pictures follow it in decoding order until the picture with corresponding collocated IRAP in the reference layer

# References

- M. Hannuksela, “On SHVC RAP pictures,” Document JCTVC-L0039, January 2013.

- J. Boyce and Y. Chen, “Joint BoG report on extension high-level syntax,” Document JCTVC-L0441, January 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).