| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0097 |
| --- | --- |

| Title: | RCE1: The performance of extended chroma mode for non 4:2:0 format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Email: | jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution reports the performance of chroma prediction which predicts chroma samples using linear combination of luma samples for non 4:2:0 formats. The average gains over YUV 444 and 420 sequences are -1.9%, -6.6%, -5.7% for the case of All Intra Main Tier and -1.9%, -3.9%, -4.6% for High Tier and -1.7%, -2.4%, -3.1% for Super High Tier. The gains for RGB sequences are -19.7%, -17.9%, -18.7% for the case of All Intra Main Tier and -15.4%, -14.4%, -15.0% for High Tier and	-11.5%, -10.7%, -11.3% for Super High Tier. It also proposes that the parameters for the linear combination are acquired by each CU to reduce the number of the derivation to get them. It reports -0.9%, -3.7%, -3.3% for the case of All Intra Main Tier and -1.0%, -2.3%, -2.9% for High Tier and -0.9%, -1.5%, -2.0% for Super High Tier in the average of YUV 444 and YUV 420. It also shows -17.7%, -16.2%, -16.8%	 for AIMT and -13.5%, -12.9%, -13.1% for AIHT and -9.9%, -9.6%, -9.8% for AIST for RGB sequences.

# Introduction

For HEVC, chroma prediction based on the linear combination of luma samples which was called LM chroma in short was proposed [1-4]. It utilized the relations between chroma samples and corresponding luma samples and estimated the chroma samples by linear combination of the luma samples. Since the chroma samples for the current block are not known for decoder, the parameters for the linear combination are calculated with the neighbouring samples.

In case of 4:2:0 sampling, chroma samples are predicted as as follows.

predC[x,y]=α×(PL[2x,2y]+PL[2x,2y+1])>>1+β	(1),

where PL indicates luma samples.

The derivation of the parameter α and β can be described as follows.

α=R(PL,P'C)R(PL,PL)β=M(P'C)−α×M(PL)	(2),

where P'c indicates reconstructed chroma samples around the target block. And PL indicates the luma samples considering the phase difference between luma and chroma samples, which is derived by

PL[x,y]=(PL[2x,2y]+PL[2x,2y+1])>>1	(3).

Fig. 1 depicts chroma samples and luma samples used for the prediction and the calculation in the case of 4:2:0 sampling. The locations of the samples used for the derivation of α and β are shown as gray circles and the location of luma samples used for the prediction of chroma samples are shown as red circles.

Fig. 1 chroma and luma samples involved in LM chroma prediction

Since chroma prediction is done by TU, these parameters α and β are also derived by each TU with the neighbours around the TU.

# Proposed Method

## Extended chroma prediction using linear combination of luma samples

We propose to use the extended chroma prediction using linear combination of luma samples (LM chroma) for non 4:2:0 sampling. The basic algorithm remains as before regardless of the samplings but the samples involved should be changed.

For the case of 4:4:4 sampling, all of luma and chroma neighbours are used for the parameter calculation and all of luma samples in the block are used for the prediction of chroma samples.

For the case of 4:2:2 sampling, downsampling of luma samples is required but it is not need to consider the phase difference between luma and chroma samples unlike 4:2:0 sampling. The locations of the samples used for the derivation of α and β are shown as blue circles and the location of luma samples used for the prediction of chroma samples are shown as red circles in Fig. 2. Since chroma prediction is performed by each NxN sub-TU, these parameters α and β are also derived by each TU with the neighbours around the TU.

Fig. 2 chroma and luma samples involved in LM chroma prediction for 4:2:2 sampling

## Derivation of the parameters

We propose that the parameters are derived per each CU not per each TU to reduce the number of calculation. Once parameters are calculated, they are used for all TUs in the CU. If the parameters are calculated per each TU, the derivation process could be invoked 256 times for 64x64 CU with 256 4x4 TUs in worst case.

The proposed method reduces total number of derivation and total number of access to memories which stores luma samples without changing the calculation itself. The difference in process from the current method is depicted in Fig. 3. When a chroma CU has more than one prediction modes, the parameter derivation is performed if at least one of them is extended chroma mode.

for(LCUs)

{

for(CUs)

{

for(TUs)

{

intra prediction luma;

if (LMchroma) Paramter derivation;

intra prediction chroma;

}

}

}

for(LCUs)

{

for(CUs)

{

if (one of prediction is LMchroma) Paramter derivation;

for(TUs)

{

intra prediction luma;

intra prediction chroma;

}

}

}

Fig 1 Process of parameter derivation

# Experimental Results

The experimental result is shown below. The proposed method is implemented on top of HM9.0-range-extensions provided. The detailed results are described in the accompanied excel files. The anchor of the test is HM10.0-range-extensions ver 2 and the chroma prediction is used in all of Main, Hight and Super High Tier configuration as specified in [7].

## Performance of extended chroma prediction

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -19.7% | -17.9% | -18.7% | -15.4% | -14.4% | -15.0% | -11.5% | -10.7% | -11.3% |
| YCbCr 4:4:4 | -2.1% | -7.5% | -7.0% | -2.3% | -4.8% | -6.3% | -2.3% | -3.2% | -4.7% |
| YCbCr 4:2:2 | -1.8% | -5.6% | -4.4% | -1.5% | -2.9% | -2.8% | -1.1% | -1.7% | -1.6% |
| Overall | -8.4% | -10.7% | -10.4% | -6.8% | -7.7% | -8.4% | -5.2% | -5.4% | -6.1% |
|  | -8.4% | -10.7% | -10.5% | -6.8% | -7.9% | -8.5% | -5.2% | -5.5% | -5.9% |
| Enc Time[%] | 104% |  |  | 104% |  |  | 105% |  |  |
| Dec Time[%] | 108% |  |  | 107% |  |  | 106% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -11.1% | -9.8% | -11.8% | -7.9% | -6.3% | -8.5% |  |  |  |
| YCbCr 4:4:4 | -0.9% | -7.6% | -5.6% | -1.0% | -5.1% | -5.0% |  |  |  |
| YCbCr 4:2:2 | -0.8% | -6.4% | -4.3% | -0.7% | -2.8% | -2.8% |  |  |  |
| Overall | -4.6% | -8.0% | -7.5% | -3.4% | -4.8% | -5.6% |  |  |  |
|  | -4.6% | -8.4% | -7.3% | -3.5% | -6.0% | -5.9% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 120% |  |  | 116% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -4.2% | -3.6% | -4.5% | -2.9% | -2.0% | -3.0% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -3.9% | -2.3% | -0.2% | -2.2% | -1.9% |  |  |  |
| YCbCr 4:2:2 | -0.3% | -3.9% | -3.0% | -0.3% | -1.5% | -1.7% |  |  |  |
| Overall | -1.7% | -3.8% | -3.3% | -1.2% | -1.9% | -2.2% |  |  |  |
|  | -1.7% | -3.3% | -3.3% | -1.2% | -2.1% | -2.2% |  |  |  |
| Enc Time[%] | 102% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 124% |  |  | 118% |  |  |  |  |  |

## Performance of extended chroma prediction

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -17.7% | -16.2% | -16.8% | -13.5% | -12.9% | -13.1% | -9.9% | -9.6% | -9.8% |
| YCbCr 4:4:4 | -1.8% | -6.3% | -5.8% | -1.9% | -4.0% | -5.2% | -1.8% | -2.6% | -3.7% |
| YCbCr 4:2:2 | -0.1% | -1.2% | -0.9% | -0.1% | -0.7% | -0.6% | -0.1% | -0.4% | -0.4% |
| Overall | -7.0% | -8.3% | -8.2% | -5.5% | -6.2% | -6.6% | -4.2% | -4.5% | -4.9% |
|  | -7.0% | -8.3% | -8.3% | -5.6% | -6.2% | -6.7% | -4.2% | -4.5% | -4.7% |
| Enc Time[%] | 105% |  |  | 105% |  |  | 106% |  |  |
| Dec Time[%] | 109% |  |  | 107% |  |  | 106% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -9.4% | -8.7% | -10.3% | -6.6% | -5.5% | -7.3% |  |  |  |
| YCbCr 4:4:4 | -0.8% | -6.3% | -4.8% | -0.8% | -4.2% | -4.2% |  |  |  |
| YCbCr 4:2:2 | -0.1% | -1.2% | -0.9% | -0.1% | -0.6% | -0.6% |  |  |  |
| Overall | -3.7% | -5.6% | -5.6% | -2.6% | -3.5% | -4.2% |  |  |  |
|  | -3.7% | -5.8% | -5.5% | -2.7% | -4.6% | -4.5% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 121% |  |  | 117% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -3.5% | -3.1% | -3.9% | -2.3% | -1.7% | -2.6% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -3.4% | -2.0% | -0.2% | -1.9% | -1.7% |  |  |  |
| YCbCr 4:2:2 | 0.0% | -0.7% | -0.6% | 0.0% | -0.3% | -0.3% |  |  |  |
| Overall | -1.4% | -2.4% | -2.2% | -0.9% | -1.3% | -1.6% |  |  |  |
|  | -1.4% | -2.1% | -2.2% | -0.9% | -1.5% | -1.6% |  |  |  |
| Enc Time[%] | 102% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 123% |  |  | 116% |  |  |  |  |  |

# Number of calculation and time delay of proposed chroma prediction mode

## Number of calculation for a 2Nx2N chroma TU

| number of parameter derivation |  | 1 |
| --- | --- | --- |
| L & C | addition | (2N-1)*2 + (2N-1)*2 |
| LL & LC | addition | (2N-1)*2 + (2N-1)*2 |
|  | multiplication | 2N*2 + 2N*2 |
| Etc. | addition | 14 |
|  | multiplication | 9 |
| Total | addition | 16N+6 |
|  | multiplication | 8N+9 |

## Number of computation in a CU

### 64x64 CU with four 32x32 TUs

|  |  | Method 1 | Method 2 |
| --- | --- | --- | --- |
| number of parameter derivation |  | 4 | 1 |
| L & C | addition | 4*(32*2) + 4*(32*2) | 64*2 + 64*2 |
| LL & LC | addition | 4*(32*2) + 4*(32*2) | 64*2 + 64*2 |
|  | multiplication | 4*(32*2) + 4*(32*2) | 64*2 + 64*2 |
| Etc. | addition | 4*14 | 14 |
|  | multiplication | 4*9 | 9 |
| Total | addition | 1080 | 526 |
|  | multiplication | 548 | 261 |

### 64x64 CU with 256 4x4 TUs

|  |  | Method 1 | Method 2 |
| --- | --- | --- | --- |
| number of parameter derivation |  | 256 | 1 |
| L & C | addition | 256*(4*2) + 256*(4*2) | 64*2 + 64*2 |
| LL & LC | addition | 256*(4*2) + 256*(4*2) | 64*2 + 64*2 |
|  | multiplication | 256*(4*2) + 256*(4*2) | 64*2 + 64*2 |
| Etc. | addition | 256*14 | 14 |
|  | multiplication | 256*9 | 9 |
| Total | addition | 11776 | 526 |
|  | multiplication | 6400 | 265 |

# References

- J. Kim, S. Park, Y. Choi, Y. Jeon, B. Jeon, “New intra chroma prediction using inter-channel correlation”, JCTVC-B021, 2nd JCT-VC Meeting, Geneva, Switzerland, July 2010

- J. Chen, V. Seregin, “Chroma intra prediction by reconstructed luma samples”, JCTVC-C206, 3rd Meeting, Guangzhou, China, Oct. 2010

- J. Chen, V. Seregin, S. Lee, W-J Han, J. Kim, B. Jeon, “CE6.a: Chroma intra prediction by reconstructed luma samples”, JCTVC-D350, 4th Meeting, Daegu, Korea, Jan. 2011

- J. Chen, V. Seregin, W-J Han, J. Kim, B. Jeon, “CE6.a.4: Chroma intra prediction by reconstructed luma samples”, JCTVC-E266, 5th Meeting, Geneva, Switzerland, Mar. 2011

- J. Kim, J. Park, B. Jeon, “AHG7: The performance of extended chroma mode for non 4:2:0 format”, JCTVC-K0253, 11th Meeting, Shanghai, China, Oct. 2012

- J. Kim, B. Jeon, “AHG7: The performance of extended intra chroma prediction for non 4:2:0 format”, JCTVC-L0240, 12th Meeting, Geneva, Switzerland, Jan. 2013

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1006, 12th Meeting, Geneva, Switzerland, Jan. 2013

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).