| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0098 |
| --- | --- |

| Title: | AHG5: Seperate chroma transform depth for non 4:2:0 format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Email: | jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, chroma transform depth could be decided independently from luma transform depth for a CU. To indicate chroma transform depth, transform split flags for chroma components are signalled. To reduce encoding time and bits to encode chroma transform split flag, this contribution also proposes to limit chroma transform size up to luma transform size. It reports average gain 0.0%, -0.6%, -0.7% in the case of AIMT,  0.0%, -0.6%, -0.5%	in the case of AIHT and -0.1%, -0.5%, -0.4% in the case of AISH. It also reports -0.4%	, -0.1%, -0.2% and -0.5%, 0.0%, -0.1% for RAMT and RAHT respectively and -0.5%, 0.5%, 0.3% and -0.6%, 0.3%, 0.4% for LBMT and LBHT respectively. When chroma transform depth is limited, it reports gain of -0.1%, -0.4%, -0.6% for AIMT, -0.1%, -0.4%, -0.4% for AIHT and -0.1%, -0.3%, -0.3% for AIST. Also it reports -0.9%, 0.6%, 0.3% and -0.9%, 0.7%, 0.5% for RAMT and RAHT respectively and -1.2%, 1.3%, 1.0% and -1.2%, 1.1%, 1.3% for LBMT and LBHT respectively.

# Proposed Method

## chroma transform depth independent from luma transform depth

We propose to use separate transform depth for chroma component. Two chroma components use a common transform depth independently decided from luma transform depth. To indicate the depth, another transform split flags are signalled. Chroma transform split flag, CBF and coefficients are coded before luma transform information to utilize chroma CBF for luma CBF inference.

## chroma transform depth partially dependent on luma transform depth

Considering time to decide chroma transform depth and burden to signal transform split flags, we propose to limit the depth of chroma transform less than or equal to luma transform. It may help to reduce encoding time and also the bits coded for chroma transform depth. Since chroma transform depth depends on luma transform depth and luma CBF depends on chroma CBF, chroma transform split flag, CBF and coefficients are coded before luma transform information and luma transform split flag is coded depending on chroma transform depth. In other words, luma transform split flag is coded when the luma transform depth is greater than or equal to chroma transform depth. Otherwise it is inferred as true.

# Experimental Results

The experimental result is shown below. The proposed method is implemented on top of HM10.0-range-extensions ver 2 provided. The detailed results are described in the accompanied excel files. The anchor of the test is HM10.0-range-extensions ver 2 and the test under the condition defined in [1]. Encoding and decoding time are not reliable but encoding time will be increased since the proposed methods requires additional RD cost calculation to decide chroma transform depth.

## chroma transform depth independent from luma transform depth

Table 1 The results of separate chroma transform depth independent from luma transform depth

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | -0.3% | -0.2% | 0.0% | -0.3% | -0.2% | -0.1% | -0.2% | -0.2% |
| YCbCr 4:4:4 | -0.1% | -1.2% | -1.1% | 0.0% | -1.0% | -0.7% | -0.1% | -0.8% | -0.6% |
| YCbCr 4:2:2 | -0.1% | -0.6% | -0.8% | 0.0% | -0.5% | -0.5% | 0.0% | -0.5% | -0.4% |
| Overall | 0.0% | -0.6% | -0.7% | 0.0% | -0.6% | -0.5% | -0.1% | -0.5% | -0.4% |
|  | -0.1% | -0.7% | -0.7% | 0.0% | -0.6% | -0.5% | -0.1% | -0.5% | -0.5% |
| Enc Time[%] | 139% |  |  | 141% |  |  | 143% |  |  |
| Dec Time[%] | 109% |  |  | 108% |  |  | 106% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.6% | 0.2% | 0.3% | -0.6% | 0.1% | 0.2% |  |  |  |
| YCbCr 4:4:4 | -0.3% | -0.3% | -0.4% | -0.5% | 0.0% | -0.1% |  |  |  |
| YCbCr 4:2:2 | -0.3% | -0.1% | -0.7% | -0.3% | -0.1% | -0.3% |  |  |  |
| Overall | -0.4% | -0.1% | -0.2% | -0.5% | 0.0% | -0.1% |  |  |  |
|  | -0.4% | 0.0% | -0.3% | -0.5% | -0.6% | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 120% |  |  | 117% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.8% | 0.5% | 0.6% | -0.8% | 0.3% | 0.4% |  |  |  |
| YCbCr 4:4:4 | -0.5% | 0.5% | 0.4% | -0.7% | 0.4% | 0.5% |  |  |  |
| YCbCr 4:2:2 | -0.3% | 0.5% | -0.2% | -0.3% | 0.3% | 0.2% |  |  |  |
| Overall | -0.5% | 0.5% | 0.3% | -0.6% | 0.3% | 0.4% |  |  |  |
|  | -0.5% | 0.8% | 0.3% | -0.6% | 0.2% | 0.3% |  |  |  |
| Enc Time[%] | 104% |  |  | 105% |  |  |  |  |  |
| Dec Time[%] | 125% |  |  | 119% |  |  |  |  |  |

## chroma transform depth partially dependent on luma transform depth

Table 2 The results of separate chroma transform depth partially dependent on luma transform depth

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YCbCr 4:4:4 | -0.2% | -0.7% | -0.9% | -0.2% | -0.6% | -0.6% | -0.2% | -0.5% | -0.4% |
| YCbCr 4:2:2 | -0.1% | -0.4% | -0.7% | -0.1% | -0.4% | -0.4% | 0.0% | -0.4% | -0.3% |
| Overall | -0.1% | -0.4% | -0.6% | -0.1% | -0.4% | -0.4% | -0.1% | -0.3% | -0.3% |
|  | -0.1% | -0.4% | -0.6% | -0.1% | -0.4% | -0.4% | -0.1% | -0.3% | -0.3% |
| Enc Time[%] | 109% |  |  | 111% |  |  | 114% |  |  |
| Dec Time[%] | 108% |  |  | 107% |  |  | 106% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -1.4% | 1.1% | 0.9% | -1.1% | 1.0% | 1.1% |  |  |  |
| YCbCr 4:4:4 | -0.9% | 0.6% | 0.2% | -1.3% | 1.0% | 0.7% |  |  |  |
| YCbCr 4:2:2 | -0.3% | 0.1% | -0.5% | -0.4% | 0.1% | -0.2% |  |  |  |
| Overall | -0.9% | 0.6% | 0.3% | -0.9% | 0.7% | 0.5% |  |  |  |
|  | -0.9% | 0.5% | 0.2% | -1.0% | 0.2% | 0.6% |  |  |  |
| Enc Time[%] | 90% |  |  | 90% |  |  |  |  |  |
| Dec Time[%] | 122% |  |  | 118% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -1.8% | 1.6% | 1.6% | -1.3% | 1.2% | 1.7% |  |  |  |
| YCbCr 4:4:4 | -1.5% | 1.4% | 1.5% | -1.8% | 1.5% | 1.9% |  |  |  |
| YCbCr 4:2:2 | -0.3% | 0.7% | 0.0% | -0.4% | 0.5% | 0.3% |  |  |  |
| Overall | -1.2% | 1.3% | 1.0% | -1.2% | 1.1% | 1.3% |  |  |  |
|  | -1.2% | 1.1% | 1.0% | -1.1% | 1.0% | 1.3% |  |  |  |
| Enc Time[%] | 93% |  |  | 92% |  |  |  |  |  |
| Dec Time[%] | 124% |  |  | 118% |  |  |  |  |  |

## chroma transform depth independent from luma transform depth with modified deblocking filter

Currently in HM10.0-range-extensions ver 2, deblocking filter is applied to the TU boundaries which are decided by transform depth. In the proposed method, since chroma components have different transform depth from luma component, the boundaries where deblocking filter is applied should be changed according to the chroma transform depth for chroma components. BS and the filter coefficients remain as before. Table 3 shows the results of modified deblocking filter against first method (2.1).

Table 3 The results of separate chroma transform depth with modified deblocking filter

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| YCbCr 4:4:4 | 0.0% | -0.1% | -0.3% | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.2% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 98% |  |  | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
|  | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# References

D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1006, 12th Meeting, Geneva, Switzerland, Jan. 2013

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).