| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0099 |
| --- | --- |

| Title: | AHG5: using zero-depth chroma transform unit for 4:2:2 format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Email: | jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes to enable zero-depth chroma transform always for 4:2:2 format. Encoder can choose the depth of chroma transform between zero and that identical to luma transform depth. One flag is signalled to indicate that the chroma transform depth is same as luma transform or not when signalling is necessary. It reports gains of 0.0%, -1.3%, -1.6% for AIMT, 0.0%, -0.8%, -0.9% for AIHT, and 0.0%, -0.6%, -0.5% for AIST in average. It also reports -0.2%, -1.0%, -1.3% and -0.3%, -0.4%, -0.4% gain for RAMT and RAHT respectively and -0.2%, -0.4%, -0.6% and -0.3%, 0.1%, 0.5% gain for LBMT and LBHT respectively

# Introduction

In HEVC ver2 Range Extension, two square transforms are applied for one transform unit for chroma components in 4:2:2 format. It causes unexpectedly smaller chroma transform than luma transform. For example, when CU size is 64x64 and transform depth is 1, 32x32 transforms are used for luma components but 16x16 transforms are used for chroma components. Fig. 1 depicts this situation.

Fig. 1 When luma transform size is 32x32, chroma transform size is set 16x16

To ease the issue, JCTVC-L0151 [1] proposed applying two 32x32 chroma transforms for 64x64 CU. Encoders decide which size is better and signal that the size of chroma transform is 32x32 or 16x16. Fig. 2 shows the options the encoder can choose.

Fig. 2 When CU size is 64x64 and luma transform depth is 1, chroma transform size can be 32x32 or 16x16

# Proposed Method

We propose that the encoder decides between zero depth and same depth to luma transform unit for chroma transform unit for every size of CU. Therefore one flag indicates that the depth of chroma is same as luma transform or not is signalled each CU which has non-zero depth luma transform.

Fig. 3 When luma transform depth is more than 0, chroma transform depth can be 0 or same as luma transform depth

When luma transform depth is zero or zero chroma transform depth produces identical transform size to luma transform depth, the flag is not necessary to be signalled. For example, when a CU is 8x8 and luma transform depth is 1, chroma components are transformed by two 4x4 transforms for both case that chroma transform depth is 0 and chroma transform depth is 1 since there is not 2x2 transform . Fig. 3 depicts this case.

Fig. 4 chroma transform size is identical when CU size is 8x8 and luma transform depth is 1

# Experimental Results

The experimental result is shown below. The proposed method is implemented on top of HM10.0-range-extensions ver2 provided. The detailed results are described in the accompanied excel files. The anchor of the test is HM10.0-range-extensions ver 2 and the test is performed only for 4:2:2 format under the condition defined in [2] since the proposed methods are applied only to 4:2:2 format. Encoding and decoding time are not reliable but encoding time will be increased since we have to check RD cost of relatively bigger chroma transforms.

Table 1 The result when zero depth chroma transform is always checked

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 |  |  |  |  |  |  |  |  |  |
| YCbCr 4:4:4 |  |  |  |  |  |  |  |  |  |
| YCbCr 4:2:2 | 0.0% | -1.3% | -1.6% | 0.0% | -0.8% | -0.9% | 0.0% | -0.6% | -0.5% |
| Overall | 0.0% | -1.3% | -1.6% | 0.0% | -0.8% | -0.9% | 0.0% | -0.6% | -0.5% |
|  | 0.0% | -1.3% | -1.6% | 0.0% | -0.9% | -0.9% | 0.0% | -0.6% | -0.4% |
| Enc Time[%] | 106% |  |  | 107% |  |  | 108% |  |  |
| Dec Time[%] | 108% |  |  | 106% |  |  | 105% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 |  |  |  |  |  |  |  |  |  |
| YCbCr 4:4:4 |  |  |  |  |  |  |  |  |  |
| YCbCr 4:2:2 | -0.2% | -1.0% | -1.3% | -0.3% | -0.4% | -0.4% |  |  |  |
| Overall | -0.2% | -1.0% | -1.3% | -0.3% | -0.4% | -0.4% |  |  |  |
|  | -0.2% | -1.0% | -1.3% | -0.3% | -0.4% | -0.4% |  |  |  |
| Enc Time[%] | 117% |  |  | 119% |  |  |  |  |  |
| Dec Time[%] | 122% |  |  | 117% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 |  |  |  |  |  |  |  |  |  |
| YCbCr 4:4:4 |  |  |  |  |  |  |  |  |  |
| YCbCr 4:2:2 | -0.2% | -0.4% | -0.6% | -0.3% | 0.1% | 0.5% |  |  |  |
| Overall | -0.2% | -0.4% | -0.6% | -0.3% | 0.1% | 0.5% |  |  |  |
|  | -0.2% | -0.3% | -0.6% | -0.3% | 0.3% | 0.4% |  |  |  |
| Enc Time[%] | 120% |  |  | 121% |  |  |  |  |  |
| Dec Time[%] | 128% |  |  | 120% |  |  |  |  |  |

## Encoding time reduction

Encoding time can be reduced by the way to decide chroma transform depth. For the results in Table 1, luma transform depth is recacluated with zero depth chroma transform. Table 2 show the results when luma transform depth is decided with chroma transform depth and then the luma transform depth is reused with zero depth chroma transform for inter cases to decide chroma transform depth.

Table 2 The results when luma transform depth is fixed before chroma transform depth is selected

(compared to the proposed method)

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 |  |  |  |  |  |  |
| YCbCr 4:4:4 |  |  |  |  |  |  |
| YCbCr 4:2:2 | 0.1% | -0.1% | -0.1% | 0.2% | -0.1% | -0.2% |
| Overall | 0.1% | -0.1% | -0.1% | 0.2% | -0.1% | -0.2% |
|  | 0.1% | 0.0% | -0.1% | 0.2% | -0.1% | -0.2% |
| Enc Time[%] | 88% |  |  | 87% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 |  |  |  |  |  |  |
| YCbCr 4:4:4 |  |  |  |  |  |  |
| YCbCr 4:2:2 | 0.2% | -0.4% | -0.4% | 0.3% | -0.4% | -0.6% |
| Overall | 0.2% | -0.4% | -0.4% | 0.3% | -0.4% | -0.6% |
|  | 0.2% | -0.4% | -0.5% | 0.3% | -0.6% | -0.6% |
| Enc Time[%] | 89% |  |  | 88% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# References

- V. Kolesnikov, C. Rosewarne, M. Maeda, “Non-CE1: Test 3 - 32x32 transform in chroma for 4:2:2”, JCTVC-L0151, 12th Meeting, Geneva, Switzerland, Jan. 2013

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1006, 12th Meeting, Geneva, Switzerland, Jan. 2013

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).