| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0100 |
| --- | --- |

| Title: | SCE1 : 3.6 Inter-layer Intra Mode assignment for EL in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Email: | jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of inter layer intra mode assignment for EL which was previously proposed as JCTVC-L0239. It proposes to use the corresponding intra prediction modes in base layer of neighbouring blocks instead of DC when they are predicted as intra BL for most probable mode derivation. It shows -0.2%, -0.1%, -0.1% for AI 2x and -0.1%, -0.2%, -0.2% for AI 1.5x with mode dependent coefficient scanning and 0.0%, 0.1%, 0.1% for AI 2x and -0.1%, 0.0%, 0.0% for AI 1.5x without mode dependent coefficient scanning. The proposed method is also basically included in several most probable mode derivation methods proposed in this and last meeting.

# Introduction

Most probable modes for intra prediction are derived from intra prediction modes of neighbouring blocks. If those blocks are inter layer texture predicted, currently in SHVC, the prediction modes are considered as INTRA_DC. We proposed to use the corresponding intra prediction modes in base layer instead of INTRA_DC.

# Experimental Results

The proposed method is implemented on SHM1.0 under SHVC common test conditions in [2].

## Mdcs on

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | 0.1% |  |  |  |
| Class B | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% |
| Overall (Test vs Ref) | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% |
| Overall (Test vs single layer) | 12.3% | 13.6% | 13.1% | 10.2% | 9.9% | 9.0% |
| EL only (Test vs Ref) | -0.3% | -0.2% | -0.2% | -0.1% | -0.2% | -0.3% |
| Enc Time[%] | 101.4% |  |  | 97.4% |  |  |
| Dec Time[%] | 100.7% |  |  | 96.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

## Mdcs off

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.1% | 0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.5% | 13.9% | 13.5% | 10.2% | 10.2% | 9.3% |
| EL only (Test vs Ref) | 0.0% | 0.1% | 0.1% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100.3% |  |  | 100.0% |  |  |
| Dec Time[%] | 99.0% |  |  | 99.1% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# References

- J. Kim, J. Park, B. Jeon (LG), S. Liu, M. Gui, S. Lei (MediaTek), “Assigning intra prediction mode to inter layer intra predicted blocks in enhance layer”, JCTVC-L0239, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013.

- Ali Tabatabai, Krishna Rapaka, Ankur Saxena, Shan Liu , “Description of Core Experiment SCE 1: Intra Prediction Improvements in SHVC”, JCTVC-L1101, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).