| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0104 |
| --- | --- |

| Title: | AHG 9: On order of inter-layer reference pictures in reference picture lists |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes to modify the order of inter layer reference pictures in RefPicListTempX in descending order of their layer id, that is, inter layer reference picture from higher layer shall be located in lower index in RefPicListTempX. It is argued, inter-layer reference picture from nearer layer may be used more often compared to inter-layer reference picture from farther layer. Thus, for initial reference picture list, inter-layer reference pictures should be ordered in descending order of their layer id so that inter-layer reference picture from nearer layer shall occupy lower index in RefPicListTempX.

# Problem Description

RefIdx approach for Scalable extension of HEVC (SHVC) applies only high level syntax change to the HEVC specification to achieve scalability. One of the changes in RefIdx approach involves constructing reference picture list by including reference pictures from reference layers for inter-layer prediction during reference picture list construction process. The construction process can be briefly described as follow. During reference picture list construction process, reference pictures for both inter prediction and inter-layer prediction are listed in RefPicListTempX, where X can be replaced by 0 or 1. Then, the final reference picture list, which is called RefPicListX, is constructed from the RefPicListTempX after incorporating any modification guided by reference picture list modification.

According to the SHVC draft text [1], inter layer reference pictures are inserted into RefPicListTempX in ascending order according to their layer id in direct dependency list called RefLayerId. Consequently, inter-layer reference picture from lower layer will occupy lower index in RefPicListTempX and if there is no particular reference picture list modification instruction, it will occupy lower index in the final reference picture list RefPicListX.

In our observation, inter-layer reference picture from nearer layer may be used more often compared to inter-layer reference picture from farther layer. Thus, for initial reference picture list, inter-layer reference pictures should be ordered in descending order of their layer id so that inter-layer reference picture from nearer layer shall occupy lower index in RefPicListTempX. Figure 1 illustrates situation where there are three layers in which each higher layer has direct dependency to all layer below. As illustrated, rather than having Pic A0 be put ahead of Pic A1 in RefPicListTempX, it may be better to put Pic A1 first then followed by Pic A0.

Figure 1 – Scenario example of inter-layer reference picture in RefPicListTempX

# Proposal

We propose that for RefIdx approach of SHVC, in initial reference picture list, inter-layer reference pictures are ordered in descending order of their layer id. This can be achieved by one of the following options.

Option 1

For the first option, we suggest reversing the list of direct dependent layer id in RefLayerId right after they are constructed. The required text update to cover this suggestion is as shown by yellow highlighted text below.

Snipped from Section F.7.4.3.1 of [1].

The variables NumDirectRefLayers[ i ] and RefLayerId[ i ][ j ] are derived as follows:

RefLayerIdTemp = 0

for( i = 1; i <= vps_max_layers_minus1; i++ ) {
	for( j = 0, NumDirectRefLayers[ i ] = 0; j < i; j++ )
		if( direct_dependency_flag[ i ][ j ]  = =  1 )
			RefLayerId[ i ][ NumDirectRefLayers[ i ]++ ] = layer_id_in_nuh[ j ]

for(j = 0, k = NumDirectRefLayers[ i ] - 1; j < k; j++, k--) {
		tmpLayerId = RefLayerId[ i ][ j ]
		RefLayerId[ i ][ j ] = RefLayerId[ i ][ k ]
		RefLayerId[ i ][ k ] = tmpLayerId
	}

}

Option 2

For the second option, we suggest inserting inter-layer reference pictures in RefPicSetInterLayer into RefPicListTempX in reverse order starting from the highest index of RefPicsetInterLayer. The required text update to cover this suggestion is as shown by yellow highlighted text below.

At the beginning of the decoding process for each slice, the reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(Error: Reference source not found-1)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] – 1; i >= 0; rIdx++, i-- )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ] 

}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			(Error: Reference source not found-2)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(Error: Reference source not found-3)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	for(i = NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] – 1; i >= 0; rIdx++, i-- )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(Error: Reference source not found-4)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

# References

- J. Chen, J. Boyce, Y. Ye, M. Hannuksela, “SHVC Draft Text 1,” Document JCTVC-L1008, January 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).