| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0105 |
| --- | --- |

| Title: | AHG 9: On position of inter-layer reference pictures in reference picture lists |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes to allow inter-layer reference pictures to be inserted into RefPicListTempX in front of reference pictures for inter prediction depending on the value of TemporalId of current picture. It is asserted that the proposed idea would work well to support the observation reported in JCTVC-L0278 that when time distance between the current and the reference pictures is far, which is the case when current picture is in lower temporal sub-layer, inter-layer prediction may be more preferred than inter prediction. Thus, having inter-layer preference pictures located in earlier position in RefPicListTempX would be more advantageous since they can be referred with smaller index value.

# Problem Description

For extensions of HEVC (e.g., scalable extension and / or multi-view extension), a picture in non-base layer may have two choices for prediction, they are: (1) inter prediction by using reference pictures in the same scalability layer and (2) inter layer prediction by using reference layer in lower layer. As reported in [2], for pictures in non-base layer and higher temporal sub-layer, inter prediction from reference picture in the same layer may be more preferred / efficient as they are closer which makes the correlation high. On the other hand, for pictures in non-base layer and lower temporal sub-layer, inter-layer prediction from reference picture in lower layer may be more preferred / efficient as reference pictures from the same layer are further away which makes their correlation low. Figure 1 illustrates an example of the above observation. Pictures in temporal sub-layer 0 are separated by at least 4 pictures away from their reference pictures thus their correlation is most likely to be low. On the other hand, pictures in temporal sub-layer 2 are always next to their reference pictures so that their correlation is most likely high.

Figure 1 – Relationship between temporal sub-layer and distance to reference pictures.

From the observation that for pictures in non-base layer and higher temporal sub-layer, inter prediction from reference picture in the same layer may be more preferred and for pictures in non-base layer and lower temporal sub-layer, inter-layer prediction from reference picture in lower layer may be more preferred. Such observation can be exploited to arrange position of inter prediction reference pictures and inter-layer reference pictures within reference picture list based on the TemporalId of current picture.

# Proposal

We propose to signal an element to describe the maximum temporal sub-layer value for which pictures with TemporalId equals to or less than the value of that element shall have their inter-layer reference pictures be placed in front of short-term and long-term reference pictures within RefPicListTempX.

The following signalling and text modification are proposed to realized the idea.

Signalling method

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| max_sub_layer_ref_layer_at_front_plus1 | u(3) |
| … |  |
| } |  |

max_sub_layer_ref_layer_at_front_plus1 equals 0 specifies that for picture with nuh_layer_id greater than 0, inter-layer reference pictures shall be inserted into RefPicListTempX after short-term reference pictures and long-term reference pictures. Otherwise, max_sub_layer_ref_layer_at_front_plus1 greater than 0 specifies that for picture with nuh_layer_id greater than 0 and TemporalId less than max_sub_layer_ref_layer_at_front_plus1, inter-layer reference pictures shall be inserted into RefPicListTempX before short-term reference pictures and long-term reference pictures; for picture with nuh_layer_id greater than 0 and TemporalId greater than or equal to max_sub_layer_ref_layer_at_front_plus1, inter-layer reference pictures shall be inserted into RefPicListTempX after short-term reference pictures and long-term reference pictures. The value of max_sub_layer_ref_layer_at_front_plus1 shall never exceed the value of vps_max_sub_layers_minus1 plus 1.

Text modification of reference picture list construction process

- The following text is snipped from Section G.8.3.4 with added texts are yellow highlighted.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
   if (max_sub_layer_ref_layer_at_front_plus1 > 0 && TemporalId < max_sub_layer_ref_layer_at_front_plus1)
		for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
         RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ]
	for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(Error: Reference source not found-1)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
   if (max_sub_layer_ref_layer_at_front_plus1 == 0 || TemporalId >= max_sub_layer_ref_layer_at_front_plus1)
		for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
         RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ]
}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			(Error: Reference source not found-2)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
   if (max_sub_layer_ref_layer_at_front_plus1 > 0 && TemporalId < max_sub_layer_ref_layer_at_front_plus1)
		for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
         RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(Error: Reference source not found-3)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
   if (max_sub_layer_ref_layer_at_front_plus1 == 0 || TemporalId >= max_sub_layer_ref_layer_at_front_plus1)
		for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
         RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(Error: Reference source not found-4)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

# References

- J. Chen, J. Boyce, Y. Ye, M. Hannuksela, “SHVC Draft Text 1,” Document JCTVC-L1008, January 2013.

- K. Sato, “SHVC: On Inter-layer Prediction,” JCTVC-L0278, the 12th JCTVC meeting, January 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).