| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0107 |
| --- | --- |

| Title: | AHG5: Chroma intra prediction in extended chroma formats |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghye Min Seungsoo Jeong Sunil Lee Chanyul Kim | Email: | jh643.min@samsung.com ss00.jeong@samsung.com sunil.lee@samsung.com dionism@samsung.com |
| Source: | Samsung |  |  |

_____________________________

# Abstract

In extended chroma formats, effect of prediction accuracy of chroma components is large for both coding efficiency and subjective visual quality. In this proposal, new designs of chroma intra prediction optimized for extended color formats are proposed. In the common software for range extension support, only five intra prediction modes are used for chroma intra prediction. In the proposed methods, 7 and 11 modes were tested with appropriate changes in mode coding methods. With 7 intra prediction modes, 0.0%,-0.1%,-0.2% gains were obtained where gains of 0.0%,-0.4%,-0.6% were observed with 11 modes for each color component. Additional tests were performed with chroma reference sample filtering method[2]. The proposed methods with combination of chroma reference sample filtering approach provide higher gains which are -0.1%, -0.4%,-0.5% and -0.1%, -0.7%, -0.9% for 7 and 11 prediction modes..

# Introduction

In the common software for range extension support, five intra prediction modes are used for chroma intra prediction. In HEVC fidelity range extension (FrExt), chroma format is extended to 4:4:4 and 4:2:2 format which leads to a need for better prediction accuracy for chroma.

In the common software with (HM10.0_RExt2.0), 35 and 5 prediction modes are used in luma and chroma intra prediction, respectively. Description of 35 prediction modes is presented in the figure 1.

Figure 1. prediction modes used in intra chroma prediction mode

5 prediction modes which are used in chroma intra prediction include DM ( Derived mode ), Planar, Vertical, Horizontal and DC mode, which are represented with a black lines in Figure 1. In case that DM mode is equal to one of four other modes( Planar, Vertical, Horizontal and DC mode ), DM mode is replaced with 34 angular mode which is represented with a red dotted line in Figure 1.

Specification of intra_chroma_pred_mode and binarization method are presented following two tables.

Table 2. Specification of IntraPredModeC

| intra_chroma_pred_mode[ xB ][ yB ] | IntraPredModeY[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 26 | 10 | 1 | X ( 0 <= X <= 34 ) |
| 0 | 34 | 0 | 0 | 0 | 0 |
| 1 | 26 | 34 | 26 | 26 | 26 |
| 2 | 10 | 10 | 34 | 10 | 10 |
| 3 | 1 | 1 | 1 | 34 | 1 |
| 4 | 0 | 26 | 10 | 1 | X |

Table 2.  chroma mode coding

| Value of intra_chroma_pred_mode | prefix | suffix |
| --- | --- | --- |
| 4 | 0 | n/a |
| 0 | 1 | 00 |
| 1 | 1 | 01 |
| 2 | 1 | 10 |
| 3 | 1 | 11 |

# Description of the proposed change

The proposed methods employ 7 or 11 chroma intra prediction modes. As can be observed in Figure 2 and Figure 3, additional prediction modes employed to these methods have same or similar directions with 9 directions used in H.264 standardization.

## Method 1: 7 intra modes

Angular mode 34 and 18 were added to have 7 intra modes as described with black lines in Figure 2. In case that DM mode is equal to one of other modes, DM mode is replaced with 2 angular mode, which is represented with a red dotted line in Figure 2.

Figure 2. prediction modes used in the method 1

Specification of intra_chroma_pred_mode and binarization method are presented following two tables.

Table 3. Specification of IntraPredModeC ( 7 directions )

| intra_chroma_pred_mode[ xB ][ yB ] | IntraPredModeY[ xB ][ yB ] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 226 | 110 | 11 | 334 | 118 | X ( 0 <= X <= 34 ) |
| 0 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 26 | 2 | 26 | 26 | 26 | 26 | 26 |
| 2 | 10 | 10 | 2 | 10 | 10 | 10 | 10 |
| 3 | 1 | 1 | 1 | 2 | 1 | 1 | 1 |
| 4 | 34 | 34 | 34 | 34 | 2 | 34 | 34 |
| 5 | 18 | 18 | 18 | 18 | 18 | 2 | 18 |
| 6 | 0 | 26 | 10 | 1 | 34 | 18 | X |

Table 4. chroma mode coding ( 7 directions )

| Value of intra_chroma_pred_mode | binarization |
| --- | --- |
| 6 | 0 |
| 0 | 100 |
| 1 | 101 |
| 2 | 1100 |
| 3 | 1101 |
| 4 | 1110 |
| 5 | 1111 |

## Method 2: 11 intra modes

Angular mode 34,18,2,22,30 and 14 were added to have 11 intra modes as described with black lines in Figure 3. In case that DM mode is equal to one of other modes, DM mode is replaced with 6 angular mode, which is represented with a red dotted line in Figure 2.

Figure 3. prediction modes used in the method 2

Table 5.  Specification of IntraPredModeC ( 11 directions )

Table 6. chroma mode coding ( 11 directions )

| Value of intra_chroma_pred_mode | binaryzation |
| --- | --- |
| 10 | 0 |
| 0 | 100 |
| 1 | 101 |
| 2 | 11000 |
| 3 | 11001 |
| 4 | 11010 |
| 5 | 11011 |
| 6 | 11100 |
| 7 | 11101 |
| 8 | 11110 |
| 9 | 11111 |

# Experiment Results

Simulations were performed under the common conditions [1]. As can be seen the following table, the method 1 with 7 intra prediction modes provides performance gains 0.0%, -0.1%, -0.2% .

Table 7. Test results with 7 chroma modes

Method 2 (11 modes) provides gains 0.0%, -0.4%,-0.6%  as in following table.

Table 7 Test results with 11hroma modes

For additional information, proposed methods were implemented on top of chroma reference sample filtering method (method 1 in the proposal) [2].

The method 1 provides performance of -0.1%,-0.4%,-0.5% with combination of reference sample filtering.

Table 8. Test results with 7 chroma modes with reference filtering

Performance gains of -0.1%, -0.7%,-0.9% were obtained from method 2 with combination of reference sample filtering  where maximum performance reaches up to -0.6%,-1.5%,-1.7% for “EBUGraphics_1920x1080_50_10bit_422”  sequence.

Table 9. Test results with 11 chroma modes with reference filtering

# Conclusion

Optimized designs of chroma intra prediction for extended color formats were proposed. Proposed designs provide  0.0%,-0.1% ,-0.2% and 0.0%,-0.4%, -0.6%  performance gains by employing 7 and 11 intra predictions modes for chroma. When reference filtering methods were employed together, maximum gains are up to -0.6%,-1.5%,-1.7% ,which proves potentials of proposed approaches. We recommend adopting  proposed method for common s/w with range extension support considering encoder complexity.

# References

[1] David Flynn, Chris Rosewarne , " Common test conditions and software reference configurations for HEVC range extensions ", JCTVC-L1006, 12th Meeting: Geneva, CH, 14–23 Jan. 2013

[2] J.Min, S.Lee, C.Kim, " AHG5: Reference sample filtering in intra prediction for extended color format ", JCTVC-M0106, 13th Meeting, Incheon, KR, 18–26 Apr. 2013

.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).