| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0109r1 |
| --- | --- |

| Title: | SCE3.5: Simplification of Generalized Residual Inter-Layer Prediction for spatial scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François Jonathan Taquet Christophe Gisquet Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a simplification of the Generalized Residual Inter-Layer Prediction (GRP), which is an inter mode comprising a second order prediction based on the BL temporal residual. The proposed simplification consists in directly accessing the temporal reference BL data instead of the upsampled temporal reference BL data to generate the second order residual. In the version with one single GRP mode, average BDR gains over the SHM1.0 of 1.5% Y, 3.4% U and 3.8% V for RA, of 2.2% Y, 2.5% U and 2.5% V for LDP, and of 2.3% Y, 3.2% U and 3.3% V for LDB, are reported. In the version with two GRP modes, average BDR gains over the SHM1.0 of 1.6% Y, 3.5% U and 4.0% V for RA, of 3.6% Y, 3.3% U and 2.8% V for LDP, and of 2.6% Y, 3.6% U and 3.7% V for LDB, are reported.  The encoding run time increase is reportedly around 10%, while the decoding run time increase is around 5%. It is reported that the worst case complexity of the proposed tool (evaluated according to the methodology described in JCTVC-L0440) is in the same order as the SHM worst case complexity.

# Introduction

This contribution proposes a simplification of the Generalized Residual Inter-Layer Prediction (GRP), initially proposed in [1].  The GRP mode consists in applying a second order prediction, using the Base Layer temporal residual derived from the scaled EL motion vectors. The prediction signal is derived as follows (shown for the mono-prediction case, the concept is easily generalized for the bi-prediction case):

PREDEL = MC[REFEL, MVEL] + w. ( UP{RECBL} - MC[UP{REFBL}, MVEL] )

where PREDEL is the prediction signal of the enhancement layer, REFEL and REFBL are the temporal reference pictures in the Enhancement and Base layers, RECBL is the base layer reconstructed picture corresponding to the current enhancement layer picture, MVEL is the motion vector selected, UP{.} is the up-sampling operator, MC[I,MV] is the motion compensation step of the current picture using I as reference picture and MV for the motion vector. w is a weighting factor possibly applied to the second order residual.

The principle of the GRP mode can be summarized by the following Figure 1.

Figure 1: Schematic principle of the GRP mode

# Proposed GRP simplification

The goal of the proposed simplification is to reduce the memory needs in case of spatial scalability, by directly accessing the temporal reference BL data instead of applying the motion compensation step to the upsampled temporal reference BL data. The simplification also enables reducing the number of required upsampling or motion compensation operations.

## Combined Upsampling and Motion compensation of the reference BL

To this end, the Upsampling and Motion Compensation steps applied to the temporal reference BL data are combined into one single operation, noted MCoUP in the following equation:

PREDEL = MC[REFEL, MVEL] + w. ( UP{RECBL} - MCoUP{REFBL, MVEL/ratio] )

where MVEL/ratio is the EL motion vector rescaled to the BL resolution.

The modified GRP mode is schematically represented in the Figure 2.

Figure 2: Schematic principle of the proposed GRP mode simplification

The accuracy of the rescaled motion vector for the MCoUP operation is defined as follows:

- In dyadic spatial scalability (ratio 2x)

  - 1/8th pixel from luma samples

  - 1/16th pixel from chroma samples

- In spatial scalability with inter-layer ratio of 3/2 (ratio 1.5x)

  - 1/6th pixel from luma samples

  - 1/12th pixel from chroma samples

For luma, 8-taps 6-bit filters are used. For chroma, 4-taps 6-bit filters are used. Filters for phases 0 and ½ are the DCT-IF based filters specified in the Core HEVC specification. For the other phases, a bilinear interpolation from these 2 filters applies. These filters turned out to give the best BD-rate results.

The corresponding filters, specified on 6-bits, are provided in annex.

## Motion compensation of the reference EL

The motion compensation of the EL (noted as MC[REFEL, MVEL] in the equation above) uses a bilinear filter instead of the HEVC 8-tap (luma) and 4-tap (chroma) filters.

## SNR scalability case

The proposal applies for spatial scalability cases. For the SNR case, the motion compensation achieved to the BL and EL are both based on bilinear filters.

## Minimum PU size limitations

The GRP mode applies to min PU size of 8x8 in the Uni-Pred case, and to min PU size of 16x8/8x16 in the Bi-Pred case.

# Results

The testing conditions are those defined by SCE3 description document Error: Reference source not found.

Full results are provided in the 2 following attached xls files:

- JCTVC-M0xxx_CE3_5_Results_1Mode.xls

  - results when GRP is implemented as 1 additional mode with weight w=1

- JCTVC-M0xxx_CE3_5_Results_2Modes.xls

  - results when GRP is implemented as 2 additional modes with weights w=1 and 1/2

Complexity analysis, estimated according to the methodology described in JCTVC-L0440, is provided in the attached excel files. 4 files are provided:

- JCTVC-M0xxx_CE3_5_Complexity_1Mode_AllRatios.xls

  - Complexity assessment data for all spatial / SNR scalability cases with GRP 1 mode

- JCTVC-M0xxx_CE3_5_Complexity_1Mode_Ratio1.5.xls

  - Complexity assessment data for spatial scalability ratio 1.5x only with GRP 1 mode

- JCTVC-M0xxx_CE3_5_Complexity_1Mode_Ratio2.xls

  - Complexity assessment data for spatial scalability ratio 2x only with GRP 1 mode

- JCTVC-M0xxx_CE3_5_Complexity_2Modes_AllRatios.xls

  - Complexity assessment data for all spatial / SNR scalability cases with GRP 2 modes

The following sections summarize the coding efficiency and complexity analysis results.

## Coding performance - GRP with w=1

In this evaluation, only one GRP mode is used, with weight w=1 applied to the second order residual. The average BDR gain is of

- RA: 1.5% Y, 3.4% U and 3.8% V

- LDP: 2.2% Y, 2.5% U and 2.5% V

- LDB: 2.3% Y, 3.2% U and 3.3% V

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.3% | -3.8% | -3.9% |  |  |  | -1.2% | -3.9% | -4.3% |
| Class B | -1.4% | -2.3% | -2.6% | -1.5% | -3.4% | -3.9% | -2.0% | -4.0% | -4.8% |
| Overall (Test vs Ref) | -1.3% | -2.7% | -3.0% | -1.5% | -3.4% | -3.9% | -1.7% | -4.0% | -4.6% |
| Enc Time[%] | 114.4% |  |  | 108.6% |  |  | 108.8% |  |  |
| Dec Time[%] | 105.1% |  |  | 109.0% |  |  | 103.8% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.8% | -2.9% | -2.5% |  |  |  | -1.6% | -3.4% | -3.4% |
| Class B | -2.3% | -0.8% | -0.6% | -2.2% | -2.2% | -2.0% | -2.6% | -4.2% | -4.8% |
| Overall (Test vs Ref) | -2.2% | -1.4% | -1.1% | -2.2% | -2.2% | -2.0% | -2.3% | -4.0% | -4.4% |
| Enc Time[%] | 118.0% |  |  | 110.2% |  |  | 112.8% |  |  |
| Dec Time[%] | 104.7% |  |  | 107.9% |  |  | 105.4% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.8% | -3.5% | -3.2% |  |  |  | -1.8% | -4.1% | -4.0% |
| Class B | -2.1% | -1.7% | -1.8% | -2.6% | -3.8% | -4.0% | -2.5% | -3.5% | -3.8% |
| Overall (Test vs Ref) | -2.0% | -2.2% | -2.2% | -2.6% | -3.8% | -4.0% | -2.3% | -3.6% | -3.9% |
| Enc Time[%] | 116.0% |  |  | 108.9% |  |  | 109.6% |  |  |
| Dec Time[%] | 105.0% |  |  | 108.0% |  |  | 105.9% |  |  |

## Coding performance - GRP with w=1 and w=1/2

In this evaluation, 2 GRP modes are used, with weight w=1 and with weight w=½ applied to the second order residual. The average BDR gain is of

- RA: 1.6% Y, 3.5% U and 4.0% V

- LDP: 3.6% Y, 3.3% U and 2.8% V

- LDB: 2.6% Y, 3.6% U and 3.7% V

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.3% | -3.9% | -3.8% |  |  |  | -1.3% | -4.2% | -4.5% |
| Class B | -1.4% | -2.3% | -2.6% | -1.5% | -3.4% | -4.0% | -2.3% | -4.6% | -5.3% |
| Overall (Test vs Ref) | -1.4% | -2.7% | -3.0% | -1.5% | -3.4% | -4.0% | -2.0% | -4.5% | -5.1% |
| Enc Time[%] | 125.2% |  |  | 118.0% |  |  | 114.4% |  |  |
| Dec Time[%] | 103.5% |  |  | 106.1% |  |  | 101.3% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.7% | -3.8% | -2.8% |  |  |  | -3.6% | -4.6% | -3.8% |
| Class B | -3.4% | -1.0% | -0.4% | -3.0% | -2.5% | -1.8% | -4.8% | -6.0% | -6.1% |
| Overall (Test vs Ref) | -3.2% | -1.8% | -1.1% | -3.0% | -2.5% | -1.8% | -4.5% | -5.6% | -5.4% |
| Enc Time[%] | 127.8% |  |  | 118.7% |  |  | 120.0% |  |  |
| Dec Time[%] | 105.3% |  |  | 106.2% |  |  | 102.7% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.0% | -3.8% | -3.4% |  |  |  | -2.3% | -4.9% | -4.6% |
| Class B | -2.3% | -1.9% | -2.0% | -2.6% | -3.8% | -4.0% | -3.3% | -4.6% | -4.9% |
| Overall (Test vs Ref) | -2.2% | -2.4% | -2.4% | -2.6% | -3.8% | -4.0% | -3.0% | -4.7% | -4.8% |
| Enc Time[%] | 125.6% |  |  | 116.7% |  |  | 115.1% |  |  |
| Dec Time[%] | 105.3% |  |  | 107.1% |  |  | 106.7% |  |  |

## Worst case complexity assessment

The following table summarizes the worst case complexity assessment for the different configurations. We explicitly provide the cases of spatial scalability (ratios 1.5 and 2) since our proposal specifically applies for these 2 cases.

It can be observed that the worst case estimated complexity for the proposed tool stays in a similar order as the SHM worst case, except in terms of the number of frames accessed.

|  | 1 mode ratio 2x | 1 mode ratio 1.5x | 1 mode all ratios | 2 modes all ratios |
| --- | --- | --- | --- | --- |
| Mult | 115% | 115% | 115% | 115% |
| Adds | 113% | 113% | 113% | 113% |
| MemBand(2D:4x2) | 83% | 99% | 99% | 99% |
| MemBand(2D:8x2) | 81% | 100% | 100% | 100% |
| Number of Ref Frames | 120% | 136% | 180% | 180% |
| Tables Size | 200% | 200% | 200% | 200% |

# Conclusion

The proposed tool provides a noticeable coding gain, (in average 2.1%  with GRP 1 mode, 2.6% with GRP 2 modes), with worst case complexity (evaluated according to the methodology described in JCTVC-L0440) similar to the worst case SHM.

It is therefore recommended to adopt this tool in the SHVC WD.

# References

- E. François, J. Taquet, C. Gisquet, G. Laroche, P. Onno, “Non-TE3: Simplification of Generalized Residual Inter-Layer Prediction (GRILP) in SHVC”, JCTVC-L0104, Geneva, Switzerland, 14–23 Jan. 2013.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex – interpolation filters

The interpolation filters used in the proposal are given below. They are based on the Core HEVC DCT-IF filters of phases 0 and 0.5. The other phases are generated from these 2 given filters by bilinear interpolation, constrained to be defined on 6-bits. Default DCT-IF based filters are indicated in bold red font.

Interpolation filters Luma ratio 2x

{  0, 0,   0, 64,  0,   0, 0,  0 },

{  0, 1,  -3, 58, 10,  -3, 1,  0 },

{ -1, 2,  -6, 53, 21,  -6, 2, -1 },

{ -1, 3,  -8, 46, 30,  -8, 3, -1 },

{ -1, 4, -11, 40, 40, -11, 4, -1 },

{ -1, 3,  -8, 30, 46,  -8, 3, -1 },

{ -1, 2,  -6, 21, 53,  -6, 2, -1 },

{  0, 1,  -3, 10, 58,  -3, 1,  0 }

Interpolation filters Chroma ratio 2x

{  0, 64,  0,  0},

{ -1, 61,  5, -1},

{ -1, 57,  9, -1},

{ -2, 54, 14, -2},

{ -2, 50, 18, -2},

{ -3, 47, 23, -3},

{ -3, 43, 27, -3},

{ -4, 40, 32, -4},

{ -4, 36, 36, -4},

{ -4, 32, 40, -4},

{ -3, 27, 43, -3},

{ -3, 23, 47, -3},

{ -2, 18, 50, -2},

{ -2, 14, 54, -2},

{ -1,  9, 57, -1},

{ -1,  5, 61, -1}

Interpolation filters Luma ratio 1.5x

{ 0, 0,   0, 64,  0,   0, 0,  0},

{ 0, 1,  -4, 57, 13,  -4, 1,  0},

{-1, 3,  -7, 48, 26,  -7, 3, -1},

{-1, 4, -11, 40, 40, -11, 4, -1},

{-1, 3,  -7, 26, 48,  -7, 3, -1},

{ 0, 1,  -4, 13, 57,  -4, 1,  0}

Interpolation filters Chroma ratio 1.5x

{  0, 64,  0,  0},

{ -1, 60,  6, -1},

{ -1, 54, 12, -1},

{ -2, 50, 18, -2},

{ -3, 46, 24, -3},

{ -3, 40, 30, -3},

{ -4, 36, 36, -4},

{ -3, 30, 40, -3},

{ -3, 24, 46, -3},

{ -2, 18, 50, -2},

{ -1, 12, 54, -1},

{ -1,  6, 60, -1}