| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0110r1 |
| --- | --- |

| Title: | SCE3.9: Base Mode with Generalized Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François Jonathan Taquet Christophe Gisquet Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This document presents the implementation of the Base Mode using Generalized Residual Prediction (GRP). In the Base Mode, the syntax of each NxN block of a CU is derived from the collocated BL CU. It is reported that the experiments have shown that using systematically GRP, instead of a switchable solution, provides the best coding performance. Average BDR gains of 1.3% Y, 1.9% U and 2.8% V for RA, of 2.0% Y, 1.7% U and 1.6% V for LDP, and of 1.9% Y, 1.7% U and 1.8 V for LDB are reported when N=8. Average BDR gains of 1.1% Y, 2.1% U and 2.4% V for RA, of 1.6% Y, 1.9% U and 1.8% V for LP, and of 1.4% Y, 2.1% U and 2.1% V for LDB are reported when N=16. The encoding and decoding runtime are reportedly slightly increased (by 2-3%). It is reported that the worst case complexity of the proposed tool (evaluated according to the methodology described in JCTVC-L0440) is in the same order as the SHM worst case complexity for the N=16 version, while for the N=8 version, multiplications, and memory access bandwidths worst cases are increased by around 60-65%.

# Proposal description

This contribution proposes a mode, called base mode, consisting in inheriting the syntax from the base layer on a CU basis. In addition, second order prediction applies using the Generalized Residual Inter-Layer Prediction (GRP). This base mode was proposed in [1].

## Base mode with GRP

The Base Mode consists in splitting the current CU in the enhancement layer into NxN blocks, and each block applies a motion compensation using the motion information (motion vector, reference list, reference index, …) of its collocated base layer CU. Motion vectors are scaled to match the spatial resolution change. In this Base Mode, we are also considering the addition of a residual data. The process applied to each NxN block of the CU can be summarized by the following formula:

PREEL = MC{ REFEL , SP_ratio*MVBL } + w.RES,

where

- MC{I, MV} corresponds to the motion compensation with motion vector MV  and using as reference picture the picture I.

- REFEL is the reference picture for the enhancement layer

- MVBL is the motion vector of the base layer

- SP_ratio is the spatial ratio between the base layer and the enhancement layer

- w is a weighted factor

- RES is the residual of the corresponding CU in the base layer and is computed as follows:

RES = UPS{RECBL} – MC{UPS{REFBL}, SP_ratio*MVBL }}

where

- UPS{.} is the up-sampling operator

- RECBL is the reconstructed base layer CU.

Contrary to the SVC specifications, the temporal residual texture coded and decoded in the base layer is not inherited from the base picture. For the proposed “Base Mode”, the residual RES is based on the GRP concept (see SCE3.3 and SCE3.5 for more details) and is used for the prediction of the enhancement layer.

## Upsampling and Motion compensation for Base Mode

The BL upsampling uses the SHM1.0 upsampling filters.

The motion compensation used both for the BL and EL blocks is based on bilinear filters.

# Results

The testing conditions are those defined by SCE3 description document Error: Reference source not found.

Full results are provided in the attached xls files.

## Test of Base Mode with switchable weights

We have first evaluated on a limited number of frames (33) of all class B sequences, the performance of the Base Mode using several possible weights. The block granularity is 8x8. The results show that using systematically a weight of 1 gives the best trade-off coding efficiency/complexity.

| Modes | Y | U |  | V | Enc Time[%] | Dec Time[%] |
| --- | --- | --- | --- | --- | --- | --- |
| 0: No GRP | -0.10% |  | 0.11% | 0.08% | 103.26% | 96.56% |
| 1: GRP w=1 | -1.42% | -2.26% |  | -2.99% | 102.29% | 102.18% |
| 2: GRP w=1/2 | -0.72% | -0.60% |  | -0.92% | 103.03% | 101.31% |
| 0 + 1 | -1.43% | -2.16% |  | -2.85% | 104.85% | 102.31% |
| 1 + 2 | -1.58% | -2.19% |  | -2.94% | 104.77% | 103.72% |
| 0 + 1 + 2 | -1.55% | -2.15% |  | -2.87% | 107.59% | 103.26% |

## Base Mode with block 8x8 and weight w=1

In this version, the PU block granularity for the base mode is of 8x8. The weight is always equal to 1.

The average BDR gains are of

- RA: 1.3% Y, 1.9% U and 2.4% V

- LDP: 2.0% Y, 1.7% U and 1.6% V

- LDB: 1.9% Y, 1.7% U and 1.8% V

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.8% | -2.2% | -2.1% |  |  |  | -1.1% | -2.9% | -2.9% |
| Class B | -1.0% | -1.6% | -1.9% | -1.5% | -2.4% | -3.0% | -1.6% | -1.1% | -2.2% |
| Overall (Test vs Ref) | -1.0% | -1.7% | -1.9% | -1.5% | -2.4% | -3.0% | -1.5% | -1.6% | -2.4% |
| Enc Time[%] | 103.1% |  |  | 102.2% |  |  | 102.3% |  |  |
| Dec Time[%] | 105.7% |  |  | 109.4% |  |  | 107.2% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.2% | -1.9% | -1.5% |  |  |  | -1.4% | -2.1% | -1.7% |
| Class B | -1.7% | -0.8% | -0.7% | -2.4% | -2.1% | -2.0% | -2.4% | -1.6% | -2.2% |
| Overall (Test vs Ref) | -1.5% | -1.1% | -0.9% | -2.4% | -2.1% | -2.0% | -2.1% | -1.8% | -2.0% |
| Enc Time[%] | 103.3% |  |  | 101.6% |  |  | 102.7% |  |  |
| Dec Time[%] | 105.4% |  |  | 111.1% |  |  | 109.5% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.1% | -2.1% | -1.7% |  |  |  | -1.4% | -2.4% | -2.0% |
| Class B | -1.5% | -1.1% | -1.1% | -2.4% | -2.6% | -2.8% | -1.9% | -0.5% | -1.0% |
| Overall (Test vs Ref) | -1.4% | -1.4% | -1.3% | -2.4% | -2.6% | -2.8% | -1.8% | -1.0% | -1.3% |

## Base Mode with block 16x16 and weight w=1

In this version, the PU block granularity for the base mode is of 16x8/8x16. The weight is always equal to 1.

The average BDR gains are of

- RA: 1.1% Y, 2.1% U and 2.4% V

- LDP: 1.6% Y, 1.9% U and 1.8% V

- LDB: 1.4% Y, 2.1% U and 2.1% V

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.9% | -1.8% |  |  |  | -0.9% | -2.6% | -2.5% |
| Class B | -0.8% | -1.8% | -2.1% | -1.1% | -2.4% | -2.8% | -1.4% | -1.7% | -2.5% |
| Overall (Test vs Ref) | -0.8% | -1.8% | -2.0% | -1.1% | -2.4% | -2.8% | -1.3% | -2.0% | -2.5% |
| Enc Time[%] | 102.6% |  |  | 101.5% |  |  | 101.5% |  |  |
| Dec Time[%] | 105.0% |  |  | 107.3% |  |  | 104.9% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.8% | -1.7% | -1.4% |  |  |  | -1.1% | -1.9% | -1.5% |
| Class B | -1.4% | -1.3% | -1.1% | -1.8% | -2.2% | -2.1% | -1.9% | -2.1% | -2.5% |
| Overall (Test vs Ref) | -1.2% | -1.4% | -1.2% | -1.8% | -2.2% | -2.1% | -1.7% | -2.1% | -2.2% |
| Enc Time[%] | 102.9% |  |  | 101.3% |  |  | 102.3% |  |  |
| Dec Time[%] | 105.0% |  |  | 107.7% |  |  | 106.2% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.8% | -1.9% | -1.6% |  |  |  | -1.1% | -2.3% | -1.8% |
| Class B | -1.2% | -1.6% | -1.6% | -1.8% | -2.9% | -3.0% | -1.5% | -1.5% | -1.8% |
| Overall (Test vs Ref) | -1.1% | -1.7% | -1.6% | -1.8% | -2.9% | -3.0% | -1.4% | -1.7% | -1.8% |

## Worst case complexity assessment

The following table summarizes the worst case complexity assessment for the different configurations.

It can be observed that the worst case estimated complexity for the proposed tool in case of 16x16 granularity stays in a lower or of similar order than the SHM worst case, except in terms of the number of frames accessed. For the 8x8 granularity, multiplications, and memory access bandwidths worst cases are increased by around 60-65%.

|  | Base mode 8x8 | Base mode 16x16 |
| --- | --- | --- |
| Mult | 169% | 115% |
| Adds | 116% | 80% |
| MemBand(2D:4x2) | 160% | 99% |
| MemBand(2D:8x2) | 163% | 100% |
| Number of Ref Frames | 180% | 180% |
| Tables Size | 108% | 108% |

## Additional results using weighted approach proposed in JCTVC-M0189

We have tested the weighting solution proposed in document JCTVC-M0189, that was only designed for the RefIdx framework. Here we have used the same approach for the IntraBL framework, using the 16x16 block granularity for the baseMode. The results are summarized in the table below. A noticeable gain is observed compared to results reported in section 2.3.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | -3.4% | -3.2% |  |  |  | -1.1% | -4.8% | -5.1% |
| Class B | -0.8% | -2.1% | -2.5% | -1.5% | -4.2% | -4.9% | -1.7% | -4.9% | -5.9% |
| Overall (Test vs Ref) | -0.8% | -2.5% | -2.7% | -1.5% | -4.2% | -4.9% | -1.5% | -4.9% | -5.7% |
| Enc Time[%] | 103.0% |  |  | 101.9% |  |  | 102.1% |  |  |
| Dec Time[%] | 129.3% |  |  | 125.5% |  |  | 124.4% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | -4.0% | -3.6% |  |  |  | -1.7% | -5.0% | -5.1% |
| Class B | -2.1% | -2.1% | -1.9% | -2.8% | -5.0% | -4.9% | -3.1% | -6.6% | -7.6% |
| Overall (Test vs Ref) | -1.9% | -2.6% | -2.4% | -2.8% | -5.0% | -4.9% | -2.7% | -6.1% | -6.9% |
| Enc Time[%] | 103.5% |  |  | 102.5% |  |  | 103.3% |  |  |
| Dec Time[%] | 122.6% |  |  | 121.8% |  |  | 122.3% |  |  |

# Conclusion

The proposed tool provides a noticeable coding gain, (around 1.5% for the simplest version), with worst case complexity (evaluated according to the methodology described in JCTVC-L0440) similar to or lower than the worst case SHM. The encoding and decoding run times increase is limited to 2-3%.

It is therefore recommended to adopt this tool in the SHVC WD.

# References

- C. Gisquet, E. François, G. Laroche, P. Onno, “Non-TE5.3: Inter-layer inferred prediction mode and motion compensation restrictions in SHVC”, JCTVC-L0103, Geneva, Switzerland, 14–23 Jan. 2013.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).