| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0111r1 |
| --- | --- |

| Title: | AHG5: on chroma QP for HEVC RExt |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François Christophe Gisquet Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a slight modification related to the chroma QP in the Range Extension of HEVC (HEVC RExt). In the current HEVC RExt Draft, one chroma QP table, linking the chroma QP (QPC) to the luma QP (QPY), is specified for each one of the three chroma formats 4:2:0, 4:2:2 and 4:4:4. In this contribution, two changes are proposed. The first change consists in removing one of the three tables, namely the 4:2:2 table. This change reportedly simplifies the design, since only the 4:2:0 table, already present in the HEVC V1 specification, and the 4:4:4 table, which is actually based on a straightforward QPC derivation, are kept. The second change consists in adding one syntax element, preferably specified in the SPS, that indicates which table must be used in case of 4:2:2 or 4:4:4 content.

The coding efficiency impact of removing the 4:2:2 table is reportedly small, mainly resulting in a slightly different balance between the luma and chroma quality. The proposed added syntax element is reported to provide a simple mechanism to easily and flexibly control this repartition.

# Introduction

In the current HEVC RExt Draft [1], the QP used for chroma is deduced from the QP used for luma and from the chroma format. The following process applies in case of non-monochrome format:

The variables qPCb and qPCr are set equal to the value of QpC as specified in Table 8-7 based on the index qPi equal to qPiCb and qPiCr, respectively, and qPiCb and qPiCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 57, QpY + pps_cb_qp_offset + slice_cb_qp_offset )	(8-1)

qPiCr = Clip3( −QpBdOffsetC, 57, QpY + pps_cr_qp_offset + slice_cr_qp_offset )	(8-2)

The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, are derived as follows:

Qp′Cb = qPCb + QpBdOffsetC		(8-3)

Qp′Cr = qPCr + QpBdOffsetC		(8-4)

Table 8-1 – Specification of QpC as a function of qPi and ChromaArrayType

| ChromaArrayType = 1 | qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

| ChromaArrayType = 2 | qPi | < 33 | 33 ≤ qPi < 39 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | > 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | = (qPi – 1) | 37 | 38 | 39 | 40 | 40 | 41 | 42 | 42 | 43 | 44 | 44 | 45 | = qPi − 6 |

| ChromaArrayType = 3 | qPi | < 51 | ≥ 51 |
| --- | --- | --- | --- |
|  | QPC | = qPi | 51 |

The figure below depicts QPC as a function of qPi, for the 3 tables.

Hence three different tables are needed:

- The 4:2:0 table, already specified in the HEVC V1 specification.

- The 4:4:4 table, which is very straightforward since it just corresponds to the clipped value of qPi.

- The 4:2:2 table, which includes specific values that cannot be deduced from a simple equation.

# Performance analysis

We have evaluated the usage of each one of these 3 tables for each type of chroma format, using the AHG7 test conditions. In this section, we are focusing on the YUV 4:2:2 and YUV 4:4:4 content cases. Results for RGB 4:4:4 are provided in Annex. Results on Super-High tier are not impacted by our proposal, so they are not provided. Full results are provided in attached excel files.

For information, we measure an average YUV BDR measure, computed using the following average YUV PSNR:

- YUV 4:4:4 – PSNRYUV = (6.PSNRY + 4.PSNRU + 4.PSNRV) / 14

- YUV 4:2:2 – PSNRYUV = (6.PSNRY + 2.PSNRU + 2.PSNRV) / 10

The weights have been derived from the YUV 4:2:0 weighting proposed in contribution JCTVC-G399, corresponding to:

- YUV 4:2:0 – PSNRYUV = (6.PSNRY + PSNRU + PSNRV) / 8

As in YUV 4:2:2 case, the chroma components are twice higher than in 4:2:0, the weights for U and V are doubled compared to the YUV 4:2:0 case. In YUV 4:4:4 case, the chroma components are twice higher than in 4:2:2, therefore the weights for U and V are doubled compared to the YUV 4:2:2 case.

In addition, we also provide BDR performance, evaluated using separate pre-channel bit-rate measures.

## Results with YUV 4:2:2 content

The tables below summarize the results obtained for the YUV 4:2:2 sequences. Table 2 reports the BDR variations using the total bit-rate measures, while table 3 reports the BDR variations using per-channel bit-rate measures.

From these 2 tables, the following points can be observed:

- The 4:2:0 chroma QP table may lead to a non-negligible loss of in luma, but this loss is well balanced by the gain in chroma (the indicative average YUV BDR shows a gain over 2% in Main-tier).

- The 4:4:4 chroma QP table leads to a slight gain of in luma, balanced by a loss in chroma (the indicative average YUV BDR shows a limited loss, around 1% in average for the 3 main-tier configurations).

Conclusion

For the YUV 4:2:2 content, it would be possible to switch to another table than the 4:2:2 table. Both 4:2:0 and 4:4:4 tables could be used. The 4:2:2 table does not look very useful.

Table 2: BDR results for YUV 4:2:2 sequences, using non-separate channel BDR

| YUV 4:2:2 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 1.8% | -3.7% | -4.9% | -2.1% | 0.3% | -0.5% | -0.7% | -0.3% |
| table 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | -0.3% | 1.0% | 1.1% | 0.5% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 1.3% | -5.4% | -5.8% | -2.6% | 0.4% | -0.9% | -1.3% | -0.5% |
| table 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | -0.9% | 3.6% | 3.6% | 1.5% | -0.2% | 0.4% | 0.5% | 0.2% |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 0.9% | -4.2% | -4.5% | -2.2% | 0.2% | -0.6% | -0.9% | -0.3% |
| table 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | -1.0% | 5.3% | 5.3% | 2.2% | -0.3% | 0.7% | 0.8% | 0.3% |

Table 3: BDR results for YUV 4:2:2 sequences, using separate channel BDR

| YUV 4:2:2 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |  |
| table 4:2:0 | 0.0% | -0.2% | -0.4% |  | 0.0% | -0.1% | 0.0% |  |
| table 4:2:2 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| table 4:4:4 | 0.0% | 0.1% | 0.1% |  | 0.0% | 0.0% | 0.0% |  |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V |  | Y | U | V |  |
| table 4:2:0 | 0.2% | -1.7% | -1.9% |  | 0.1% | -0.3% | -0.3% |  |
| table 4:2:2 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| table 4:4:4 | -0.2% | -0.4% | -0.4% |  | 0.0% | -0.2% | -0.3% |  |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V |  | Y | U | V |  |
| table 4:2:0 | 0.2% | -1.9% | -2.1% |  | 0.1% | -0.2% | -0.3% |  |
| table 4:2:2 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| table 4:4:4 | -0.2% | -0.2% | -0.3% |  | 0.0% | -0.2% | -0.3% |  |

## Results with YUV 4:4:4 content

The tables below summarize the results obtained for the YUV 4:4:4 sequences. Table 4 reports the BDR variations using the total bit-rate measures, while table 5 reports the BDR variations using per-channel bit-rate measures.

In both cases, the following points can be observed:

- The 4:2:0 chroma QP table leads to a noticeable gain in chroma, that compensates well the luma loss (the indicative average YUV BDR shows a gain around 2% in Main-tier).

- The 4:2:2 chroma QP table leads to a slight loss of in luma, well balanced by a slightly higher gain in chroma;

Conclusion

For the YUV 4:4:4 content, it would be possible to switch to another table than the 4:4:4 table. In particular switching to the 4:2:0 table would permit a different repartition of the luma and chroma quality.

Table 4: BDR results for YUV 4:4:4 sequences, using non-separate channel BDR

| YUV 4:4:4 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 3.7% | -3.5% | -4.8% | -1.5% | 0.6% | -0.6% | -0.8% | -0.3% |
| table 4:2:2 | 0.5% | -0.8% | -0.9% | -0.4% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 4.5% | -6.0% | -6.5% | -1.9% | 1.3% | -1.2% | -1.5% | -0.4% |
| table 4:2:2 | 2.0% | -3.0% | -3.0% | -0.7% | 0.5% | -0.5% | -0.5% | -0.1% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 4.1% | -6.0% | -7.3% | -2.4% | 1.1% | -1.2% | -1.7% | -0.6% |
| table 4:2:2 | 2.3% | -4.0% | -4.9% | -1.4% | 0.6% | -0.7% | -1.0% | -0.3% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 5: BDR results for YUV 4:4:4 sequences, using separate channel BDR

| YUV 4:4:4 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |  |
| table 4:2:0 | 0.1% | -0.1% | -0.3% |  | 0.0% | 0.0% | 0.0% |  |
| table 4:2:2 | 0.0% | -0.1% | -0.1% |  | 0.0% | 0.0% | 0.0% |  |
| table 4:4:4 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V |  | Y | U | V |  |
| table 4:2:0 | 1.0% | -1.4% | -0.3% |  | 0.4% | -0.3% | 0.1% |  |
| table 4:2:2 | 0.4% | 0.3% | 0.7% |  | 0.1% | 0.1% | 0.3% |  |
| table 4:4:4 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V |  | Y | U | V |  |
| table 4:2:0 | 1.3% | -1.7% | -0.9% |  | 0.4% | -0.5% | -0.1% |  |
| table 4:2:2 | 0.6% | -0.2% | 0.4% |  | 0.2% | 0.0% | 0.2% |  |
| table 4:4:4 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |

# Proposed modifications

From these observations, it is proposed to simplify the current design while keeping some flexibility in the usage of the chroma QP tables.

Two solutions are proposed.

## Proposal 1

Proposal 1 consists in keeping only 2 chroma QP tables, namely the 4:2:0 and 4:4:4 tables.

By default, in case of 4:2:2 or 4:4:4 content, the 4:4:4 table is used.

## Proposal 2

Proposal 2 consists in adding a syntax element chroma_420_not_used_flag, preferably in the SPS, signaled in case of 4:4:4 or 4:2:2 chroma format, indicating which table is used.

The changes in the specification are provided in the annex section.

## Advantages

The proposed changes have the following advantages:

- Simplification: only the straightforward 4:4:4 table is added in the HEVC RExt version compared to the HEVC V1 version.

- Flexibility: the addition of the syntax element to select one table among the 4:2:0 and 4:4:4 tables gives more flexibility to control the chroma QP, depending on the content. This can be in particular of interest for YUV 4:2:2 or YUV 4:4:4 content, which may benefit of using the 4:2:0 table instead of the 4:4:4 one.

# Conclusion

In this contribution two proposals are made related to the chroma QP in the HEVC RExt:

- First it is proposed to remove the chroma QP table corresponding to the 4:2:2 case, and just keeping the 4:2:0 and 4:4:4 conversion tables; this results in a simplified design.

- Second it is proposed to add a syntax element, in case of 4:2:2 or 4:4:4 content, to select one table among the 4:2:0 and 4:4:4 chroma QP tables; this gives more flexibility to control the chroma QP.

The coding efficiency impact of removing the 4:2:2 table is small, mainly resulting in a slightly different balanced repartition between the luma and chroma quality. The proposed added syntax element is a simple mechanism to easily control this repartition.

# References

- D. Flynn, J. Sole, T. Suzuki, “HEVC Range Extensions Draft 2”, JCTVC-L1005, 12th meeting, Geneva, Jan 2013.

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1006, 12th meeting, Geneva, Jan 2013.

# Patent rights declaration

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex1 – syntax changes

Removed text is highlighted in purple, added text in yellow.

## Syntax changes in Proposal 1

8.6.1	Derivation process for quantization parameters

…

Table 8-6 – Specification of QPC as a function of qPi and ChromaArrayType

| ChromaArrayType = 1 | qPi | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | >43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

| ChromaArrayType = 2 | qPi | < 33 | 33 ≤ qPi < 39 |  | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | > 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | = (qPi – 1) | 37 |  | 38 | 39 | 40 | 40 | 41 | 42 | 42 | 43 | 44 | 44 | 45 | = qPi − 6 |

| ChromaArrayType = 3 > 1 | qPi | < 51 | ≥ 51 |
| --- | --- | --- | --- |
|  | QPC | = qPi | 51 |

…

## Syntax changes in Proposal 2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| if( ChromaArrayType > 1 ) |  |
| chroma_420_not_used_flag | u(1) |
| … |  |
| } |  |

7.4.2.2	Sequence parameter set RBSP semantics

…

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in subclause Error: Reference source not found. The value of chroma_format_idc shall be in the range of 0 to 3, inclusive.

chroma_420_not_used_flag specifies if the chroma look-up table used to derive the chroma quantization parameters from the luma quantization parameter is the 4:2:0 table or the 4:4:4 table. When chroma_420_not_used_flag is equal to 1, the 4:4:4 table is used. When chroma_420_not_used_flag is equal to 0, the 4:2:0 table is used. When not present, chroma_420_not_used_flag is set equal to 0.

…

8.6.1	Derivation process for quantization parameters

…

Table 8-7 – Specification of QPC as a function of qPi and ChromaArrayType chroma_table_idc

| ChromaArrayType = 1 chroma_420_not_used_flag = 0 | qPi | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | >43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

| ChromaArrayType = 2 | qPi | < 33 | 33 ≤ qPi < 39 |  | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | > 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | = (qPi – 1) | 37 |  | 38 | 39 | 40 | 40 | 41 | 42 | 42 | 43 | 44 | 44 | 45 | = qPi − 6 |

| ChromaArrayType = 3 chroma_420_not_used_flag = 1 | qPi | < 51 | ≥ 51 |
| --- | --- | --- | --- |
|  | QPC | = qPi | 51 |

…

# Annex2 – Results for the RGB 4:4:4 content

The tables below summarize the results obtained for the RGB 4:4:4 sequences.

We report an average YUV BDR, based on an average PSNR computed for the three components as follows:

- RGB 4:4:4 – PSNRRGB = (PSNRR + PSNRG + PSNRB) / 3

From the results in the tables below, it must be concluded that the table 4:4:4 is actually the best one. Using another table always leads to noticeable loss in the G component, not balanced by the gain in the R and B components.

Conclusion

Keeping the 4:4:4 table is therefore recommended for the RGB 4:4:4 content.

Table 8: BDR results for RGB 4:4:4 sequences, using non-separate channel BDR

| RGB 4:4:4 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 8.0% | -1.5% | -1.5% | 0.9% | 1.3% | -0.2% | -0.2% | 0.2% |
| table 4:2:2 | 1.1% | -0.2% | -0.3% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 15.6% | -1.8% | -0.2% | 3.3% | 3.7% | -0.6% | 0.2% | 0.8% |
| table 4:2:2 | 6.0% | -0.6% | -0.5% | 1.4% | 1.1% | -0.1% | -0.1% | 0.2% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 17.0% | -1.9% | 0.0% | 3.6% | 3.9% | -0.7% | 0.3% | 0.9% |
| table 4:2:2 | 8.7% | -0.5% | -0.5% | 2.3% | 1.8% | 0.0% | -0.1% | 0.4% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 9: BDR results for RGB 4:4:4 sequences, using separate channel BDR

| RGB 4:4:4 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |  |
| table 4:2:0 | 0.3% | 0.4% | 0.4% |  | 0.0% | 0.3% | 0.1% |  |
| table 4:2:2 | 0.0% | 0.1% | 0.1% |  | 0.0% | 0.0% | 0.0% |  |
| table 4:4:4 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V |  | Y | U | V |  |
| table 4:2:0 | 4.4% | 0.6% | 1.6% |  | 1.2% | 0.1% | 0.4% |  |
| table 4:2:2 | 1.6% | 0.5% | 1.2% |  | 0.3% | 0.1% | 0.2% |  |
| table 4:4:4 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V |  | Y | U | V |  |
| table 4:2:0 | 5.1% | 0.8% | 2.4% |  | 1.1% | 0.2% | 0.5% |  |
| table 4:2:2 | 2.6% | 1.1% | 2.1% |  | 0.5% | 0.3% | 0.4% |  |
| table 4:4:4 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |