| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0112 |
| --- | --- |

| Title: | Non-SCE5: Position derivation for using Base Layer motion in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Patrice Onno Edouard François Guillaume Laroche Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

Since HM2.0, memory compression has been adopted so as to reduce the storage requirement of motion data to be used in deriving the temporal MV predictor. Following this, the location of that predictor has been changed to the bottom-right position H of the PU. It is asserted in this contribution that a similar issue occurs when deriving the motion candidate from the base. It is therefore proposed to perform a different rounding on the colocated coordinates within the Reference Layer. The gains of this rounding over SHM 1.0 are reported to be respectively for the TextureRL approach, -0.8%(RA 2x), -0.2%(RA 1.5x), -0.6%(RA SNR) and -0.5%(LD-P 2x), -0.1%(LD-P 1.5x), -0.5%(LD-P SNR). For the reference frame index approach, only ratio 2.0x is reported to offer changes, with gains over SHM1.0 of -0.2% (RA 2x) and -0.1% (LD-P 2x).

# Introduction

Figure 1: motion derivation and access to compressed motion buffer of Base Layer

Currently, there are 2 approaches in SHVC to reuse motion information from a Base Layer by looking up the motion at the colocated position in the Base Layer of:

- For Reference frame index based approach, section G.8.1.4.2.1, “Derivation process for inter layer motion”: the center of an Enhancement Layer 16x16 block;

- For TextureEL approach: section H.8.5.3.2.11, “Derivation process for inter-layer luma motion vector prediction”: the center of a PU in Merge mode.

In case of Reference frame index approach, the BL motion vector is actually considered as Temporal Motion Vector predictor in the Merge and AMVP candidate MV lists.

In case of TextureEL approach, the BL motion vector is added as a new candidate in the Merge and AMVP candidate MV lists, in addition to the Spatial and Temporal Motion Vector predictors. .

In both cases, motion information is essentially retrieved from the compressed motion buffer. Given input coordinates xRL, yRL, the position that is used is found through the equations:

xRL  = ( xRL >> 4 ) << 4

yRL  = ( yRL >> 4 ) << 4

However, this corresponds to truncating said position: as depicted in Figure 1: above, we can imagine accessing the bottom-right CU of a 16x16 block. In that case, motion from CU numbered 1 is then used.

However, CU numbered 4 is closer and should yield a better candidate. This essentially raises the question of how to round the colocated position, e.g. to the nearest neighbour among CUs 1,2,3, and 4.

# Proposal

We can more generally define the rounding operation of this position as:

xRL  = ( ( xRL + R ) >> 4 ) << 4

yRL  = ( ( yRL + R ) >> 4 ) << 4

R = 0 is what is currently used. Given the position used by motion compression, R = 8 seems a more natural choice for rounding to nearest. We have determined that R = 4 in fact yields the best results.

This affects both approaches, but the impact should be smaller in the reference index based approach: indeed, the motion associated to the ILR frame is used to derive the TMVP, whose position is already biased by taking the Bottom-Right of the PU instead of its center.

# Results

We would like to thank Vadim Seregin and Qualcomm for cross-checking those results.

## TextureRL approach

Table 1: mandatory results for the TextureRL approach

## Reference index based approach

Table 2: mandatory results for the reference index based approach

While the modification is applied in all cases, only the ratio 2.0x sees an improvement.

# Conclusion

In this contribution, a simple modification to the rounding of the colocated position in the Base Layer, when used for motion derivation, is processed. Patch is attached and demonstrated the limited change needed. In light of the coding gain for such a negligible change, we recommend adoption of the proposed changes.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

## TextureRL approach

Section H.8.5.3.2.11 is modified as follows:

The variables mvLXRLand availableFlagLXRL with X being 0 or 1, are derived as follows:

- The center location (xPCtr, yPCtr) of the luma prediction block is derived as follows

xPCtr = xPb + nPbW / 2		(H-1)

yPCtr = yPb + nPbH / 2		(H-2)

- The reference layer motion vector is derived as follows:

- The reference layer sample location derivation process as specified in subclause G.6.1 is invoked with  xPCtr, yPCtr as inputs and the outputs being assigned to  xPCtrRL and yPCtrRL ;

- The variable blPb specifies the luma prediction block covering the modified location given by ( ( ( xPCtrRL + 4 ) >> 4 ) << 4, ( ( yPCtrRL + 4 ) >> 4 ) << 4 ) inside the reference layer picture specified by RlPic.

- The luma location ( xPRL, yPRL ) is set equal to the top-left sample of the collocated luma prediction block specified by blPb relative to the top-left luma sample of the reference layer picture specified by RlPic.

- The arrays predFlagLXRL[ x ][ y ], mvLXRL[ x ][ y ] and refIdxLXRL[ x ][ y ] are set equal to the corresponding arrays of the reference layer picture specified by RlPic, PredFlagLX[ x ][ y ], mvLX[ x ][ y ] and RefIdxLX[ x ][ y ], respectively with X being the value of X this process is invoked for. The variables mvLXRL, refIdxLXRL and availableFlagLXRL are derived as follows

        - If colPb is coded in an intra prediction mode, both components of mvLXRL are set equal to 0, availableFlagLXRL is set equal to 0, and refIdxLXRL is set equal to -1;

          - Otherwise, If predFlagLXRL[ xPRL ][ yPRL ] is equal to 0, both components of mvLXRL are set equal to 0, and refIdxLXRL is set equal to -1, and availableFlagLXRL is set equal to 0.

          - Ohterwise, refIdxLXRL is set equal to  RefIdxLXRL [ xPRL ][ yPRL ], and availableFlagLXRL is set equal to 1. The two components of mvLXRL are derived as follows

mvLXRL [0] =  (mvLXRL[ xPRL ][ yPRL ][0]* ScaledW+ offset[0])/ PicWRL

mvLXRL [1] =  (mvLXRL[ xPRL ][ yPRL ][1]* ScaledH+ offset[1]) / PicHRL

          - where

            - offset[0] is set to (PicWRL/2-1) if mvLXRL[ xPRL ][ yPRL ][0] >0, and (1- PicWRL/2) otherwise;

            - offset[1] is set to (PicHRL/2-1) if mvLXRL[ xPRL ][ yPRL ][1] >0, and  (1- PicHRL/2) otherwise.

## reference index based approach

Section G.8.1.4.2.1 is modified as follows:

The variables predMode[ xP ][ yP ], mvLX[ xP ][ yP ], refIdxLX[ xP ][ yP ] and refPicOrderCntLX[ xP ][ yP ] are derived as follows.

- The center location (xPCtr, yPCtr) of the luma prediction block is derived as follows

xPCtr = xP + 8		(G-3)

yPCtr = yP + 8		(G-4)

- The derivation process for reference layer luma sample location specified in subclause G.6.1Error: Reference source not found is invoked with luma location ( xPCtr , yPCtr ) given as the inputs and ( xRef , yRef ) as output.

- The collocated position (xRL, yRL) is derived as follows

xRL = ( ( xRef + 4 ) >> 4 ) << 4		(G-5)

yRL = ( ( yRef + 4 ) >> 4 ) << 4		(G-6)