| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0113 |
| --- | --- |

| Title: | Non-SCE5: Availability of Base Layer motion in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Patrice Onno Edouard François Guillaume Laroche Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports the issue of availability of motion information in a Base Layer when the Base Layer temporal MVP has been disabled at the sequence level.

# Introduction

## Base Layer Motion Vector predictor

Currently, as described in document JCTVC-L1007, there are 2 approaches in SHVC to reuse motion information from a Base Layer by looking up the motion at the colocated position in the Base Layer of:

- For Reference frame index based approach, section G.8.1.4.2.1, “Derivation process for inter layer motion”: the center of an Enhancement Layer 16x16 block.

- For TextureEL approach: section H.8.5.3.2.11, “Derivation process for inter-layer luma motion vector prediction”: the center of a PU in Merge or Skip mode.

In case of Reference frame index approach, the BL motion vector may be considered as Temporal Motion Vector predictor in the Merge and AMVP candidate MV lists, if the reference frame from which it originates is the ILR frame.

In case of TextureEL approach, the BL motion vector is added as a new candidate in the Merge candidate MV list, in addition to the Spatial and Temporal Motion Vector predictors.

## TMVP activation in the Base Layer

In all cases (temporal motion vector predictor, or BL motion vector predictor), motion information is essentially retrieved from the compressed motion buffer.

This buffer is used for determination of the temporal MVP in the Merge and AMVP lists. However, this predictor can be disabled on 2 levels:

- At the SPS level, through flag sps_temporal_mvp_enable_flag;

- At the slice level, through flag enable_temporal_mvp_flag.

The later can be seen as a way to optimize coding efficiency for the motion field: it only affects the current slice, and mandates nothing about the use of the motion of this slice. Indeed, it cannot be known in advance whether any slice from a later frame in coding order may use its motion without further information.

This can be seen to be the role of the former flag: at the sequence level, it is known before decoding any frame whether temporal MVP will be used. This is useful for several reasons:

- It lowers the encoder complexity, and possibly the decoder, by not having to determine a specific additional candidate;

- It saves on memory bandwidth by not accessing an additional buffer;

- It may save on memory by not allocating said buffer.

Consequently, a codec designed for this specific behaviour (of not using nor allocating said buffer) may not provide such a buffer in the Base Layer.

Under these circumstances, use of the Base Layer motion, when the sps_temporal_mvp_enable_flag in the BL is 0, is questionable.

# Proposal

The following solution is proposed when the sps_temporal_mvp_enable_flag of the BL is equal to 0:

- For Reference frame index based approach:

  - Flag sps_inter_layer_mfm_enable_flag is set equal to 0 (section G.7.4.3.2.1);

  - Alternate solution: set the predModeRL buffer to “all intra” (section G.8.1.4.2).

- For TextureRL approach:

  - Flag InterLayerMotionPredictionEnableFlag (section H.8.5.3.2.1) is set equal to 0;

  - Alternate solution: definition of availableFlagRL (same section as well as H.8.5.3.2.2) , set equal to 0.

Concerning the flag InterLayerMotionPredictionEnableFlag, it can be noted that it depends on a SPS-level syntax element. As the position of InterLayerMotionPredictionEnableFlag is not yet defined, it seems required to place it in the SPS rather than the VPS.

Currently, it seems neither flag is implemented in SHM 1.0, so no code change is attached to this contribution.

# Conclusion

In this contribution, it is suggested to take into account whether TMVP is enabled in the Base Layer at the SPS level when deriving motion from the Base Layer. Some solutions have been proposed. We recommend further study of the issue in high-level syntax for HEVC (AHG9).

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).