| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0114 |
| --- | --- |

| Title: | Non SCE4: On Interlayer SAO filtering in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Patrice Onno Jonathan Taquet Edouard François Christophe Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a simplification of the Interlayer SAO contribution initially proposed in JCTVC-L0234. The simplification consists in directly applying the principle of the high-pass pre-filtering step of JCTVC-L0234 directly at the time of the determination of the edge index in the SAO process.  An average BDR gain of -0.8%(Y), -0.6%(U), -0.6%(V) over the SHM1.0 is reported for the Intra_BL approach. An average BDR gain of -0.8%(Y), -0.7%(U), -0.6%(V) over the SHM1.0 is reported for the Ref_Idx approach. In terms of complexity, it is reported that the SHM1.0 decoder runtime is reduced from 193% in SCE4.2.1 (JCTVC-L0234) to108% in this contribution.

# Introduction

The goal of the proposed simplification is to reduce the complexity of the experiment 4.2.1 of the core experiment SCE4 on Interlayer filtering. The major differences between SAO in HEVC depicted in Figure 1 and the HF-IL-SAO proposed in [2], [3] and presented in Figure 1 are the following:

- HF-IL-SAO is applied in picture basis while SAO in HEVC is CTB based;

- HF-IL-SAO is applied to the high frequency band only while SAO in HEVC applies to the whole frequency band.

In that particular Interlayer SAO experiment of [3], the base layer reconstructed picture (after up-sampling for spatial scalability case) is first low-pass filtered to get its low-frequency band picture. The low-pass filter used in [2] is a 3-tap smooth filter ([5 6 5]/16) applied both in horizontal and vertical directions. Secondly, the filtered picture is then subtracted to the base layer picture to get the high-frequency band picture. Then, the SAO process is performed to the high-frequency band picture. In the final stage, the high frequency band (on which the SAO process has been applied) and low-frequency band are merged together to get the base layer reference picture. The process of the HF-IL-SAO is depicted in Figure 2.

Figure 1: Classical Interlayer SAO

Figure 2: Decoder architecture of SCE4.2.1 (JCTVC-L0234) - regarding the HF-IL-SAO

# Proposed SAO simplification

The proposed simplification is to re-use the HF-IL-SAO principle in which the SAO filtering process is applied on the high frequencies of the base layer reconstructed pixel.

In the proposed method and in order to still apply the SAO filtering on the high frequencies of the reconstructed base layer, the SAO process of HEVC has been only modified when determining the Edge Index value for the Edge Offset. The SAO type, SAO class and the Band Offset process remain unchanged. The proposed modification results is just extending the number of neighbours for the SAO Edge Offset process to determine which of the 4 offsets values has to be used. The high-pass filtering process presented in Figure 2 is removed and is inserted at the time of the Edge Index determination. The new proposed Inter-Layer SAO filtering is presented in Figure 3.

Figure 3: Modification of the HF-IL-SAO to be applied during the SAO Edge Index determination

## Modification of the SAO Edge Index determination

As presented above the only change proposed in this contribution consists in modifying the computation of the Edge Index to then determine the offset value to be applied of the current pixel.

In the HEVC, the Edge Index of the current pixel c for the SAO Edge is determined as follows:

EdgeIndex(c) = sign (c - cn2) - sign (cn1- c) +2,                                                         (1)

where the definition of the function sign(.) is given by the following relationships

sign(x) = 1, when x>0

sign(x) = -1, when x<0

sign(x) = 0, when x=0

and where the neighboring pixel “cn1” and “cn2” are presented in Figure 4.

In the proposed method, the Edge Index is determined by the following formula by using an extended pixel neighborhood (cn3 and cn4) depicted in Figure 4:

EdgeIndex(c) = sign[((2c-cn1-cn2+2)>>2)-((2cn2-c-cn4+2)>>2)]

- sign[((2cn1-cn3-c+2)>>2)-((2c-cn1-cn2+2)>>2)]+2                             (2)

By comparing the two formulas (1) and (2), the new EdgeIndex function of (2) is changed by adding a simple high-pass filtering on each term for the sign(.) function in equation (1).

Figure 4: Neighboring pixels for the SAO process

# Results

This section present the results of the proposed method of SAO to filter the base layer reconstructed pixel by using the SHM1.0 testing conditions defined in the document [1]. The results are presented both for the Intra_BL and the Ref_idx approaches. In the following results, the SAO processing presented above is applied twice as for the HF-IL-SAO in SCE4.2.1 [3].

## Intra_BL case

The simplification presented in the proposed method brings an average BDR gain of -0.8%(Y), -0.6%(U), -0.6%(V) for mandatory tests.

|  |  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |  |  |
| Class A |  | -1.0% | -1.3% | -1.4% |  |  |  |  |  |  |
| Class B |  | -0.2% | -0.3% | -0.3% | -0.2% | -0.4% | -0.5% |  |  |  |
| Overall (Test vs Ref) |  | -0.4% | -0.6% | -0.6% | -0.2% | -0.4% | -0.5% |  |  |  |
| Overall (Test vs single layer) |  | 12.0% | 13.0% | 12.7% | 10.1% | 9.9% | 9.2% |  |  |  |
| EL only (Test vs Ref) |  | -0.8% | -1.0% | -1.0% | -0.2% | -0.4% | -0.5% |  |  |  |
| Enc Time[%] |  | 100.7% |  |  | 100.9% |  |  |  |  |  |
| Dec Time[%] |  | 104.0% |  |  | 105.1% |  |  |  |  |  |
| Enc Mem[%] |  | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match |  | Matched |  |  | Matched |  |  |  |  |  |
|  |  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  | -1.0% | -0.6% | -0.8% |  |  |  | -2.4% | -1.1% | -1.3% |
| Class B |  | -0.2% | -0.2% | -0.2% | -0.3% | -0.5% | -0.5% | -1.0% | -0.3% | -0.2% |
| Overall (Test vs Ref) |  | -0.5% | -0.3% | -0.4% | -0.3% | -0.5% | -0.5% | -1.4% | -0.6% | -0.5% |
| Overall (Test vs single layer) |  | 18.8% | 31.6% | 32.6% | 16.2% | 28.0% | 30.3% | 13.2% | 28.2% | 32.1% |
| EL only (Test vs Ref) |  | -0.9% | -0.7% | -0.8% | -0.4% | -0.5% | -0.5% | -2.3% | -1.3% | -1.3% |
| Enc Time[%] |  | 101.1% |  |  | 100.7% |  |  | 100.5% |  |  |
| Dec Time[%] |  | 106.4% |  |  | 107.0% |  |  | 110.7% |  |  |
| Enc Mem[%] |  | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match |  | Matched |  |  | Matched |  |  | Matched |  |  |
|  |  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  | -1.1% | -0.8% | -0.8% |  |  |  | -3.4% | -1.7% | -1.6% |
| Class B |  | -0.5% | -0.5% | -0.4% | -0.8% | -0.9% | -0.9% | -1.7% | -0.6% | -0.4% |
| Overall (Test vs Ref) |  | -0.7% | -0.6% | -0.5% | -0.8% | -0.9% | -0.9% | -2.2% | -0.9% | -0.8% |
| Overall (Test vs single layer) |  | 25.2% | 35.8% | 37.5% | 21.5% | 32.0% | 34.8% | 19.6% | 33.2% | 38.1% |
| EL only (Test vs Ref) |  | -1.1% | -0.9% | -0.9% | -1.0% | -1.1% | -1.1% | -3.1% | -1.8% | -1.6% |
| Enc Time[%] |  | 101.0% |  |  | 100.7% |  |  | 101.5% |  |  |
| Dec Time[%] |  | 108.8% |  |  | 109.8% |  |  | 113.9% |  |  |
| Enc Mem[%] |  | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match |  | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y |  | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% |  | -0.6% | -0.8% |  |  |  | -2.2% | -1.1% | -1.2% |
| Class B | -0.3% |  | -0.3% | -0.3% | -0.4% | -0.6% | -0.6% | -0.9% | -0.6% | -0.2% |
| Overall (Test vs Ref) | -0.5% |  | -0.4% | -0.5% | -0.4% | -0.6% | -0.6% | -1.2% | -0.7% | -0.5% |
| Overall (Test vs single layer) | 27.7% |  | 36.8% | 38.0% | 24.2% | 32.2% | 34.4% | 22.2% | 32.6% | 36.5% |
| EL only (Test vs Ref) | -0.8% |  | -0.7% | -0.8% | -0.3% | -0.5% | -0.5% | -1.8% | -1.2% | -1.0% |
| Enc Time[%] | 100.3% |  |  |  | 100.3% |  |  | 100.5% |  |  |
| Dec Time[%] | 111.5% |  |  |  | 111.3% |  |  | 114.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  |  | Matched |  |  | Matched |  |  |

## Ref_idx case

For Ref_idx case, the proposed simplification brings BDR gain of -0.8%(Y), -0.7%(U), -0,6%(V) for mandatory tests.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |  |  |  |
| Class A | -1.0% | -1.2% | -1.3% |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.2% | -0.2% | -0.4% | -0.5% |  |  |  |  |  |  |
| Overall (Test vs Ref) | -0.4% | -0.5% | -0.5% | -0.2% | -0.4% | -0.5% |  |  |  |  |  |  |
| Overall (Test vs single layer) | 12.4% | 14.5% | 14.2% | 10.3% | 9.5% | 9.0% |  |  |  |  |  |  |
| EL only (Test vs Ref) | -0.8% | -0.9% | -1.0% | -0.2% | -0.4% | -0.5% |  |  |  |  |  |  |
| Enc Time[%] | 100.5% |  |  | 99.8% |  |  |  |  |  |  |  |  |
| Dec Time[%] | 106.9% |  |  | 107.2% |  |  |  |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |  | RA HEVC SNR |  |  |  |  |
|  | Y | U | V | Y | U | V |  | Y |  | U |  | V |
| Class A | -1.0% | -0.7% | -0.7% |  |  |  |  | -2.4% |  | -1.9% |  | -2.0% |
| Class B | -0.2% | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% |  | -1.0% |  | -0.8% |  | -0.7% |
| Overall (Test vs Ref) | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% | -0.3% |  | -1.4% |  | -1.1% |  | -1.1% |
| Overall (Test vs single layer) | 19.6% | 32.7% | 33.8% | 17.0% | 27.7% | 30.2% |  | 14.2% |  | 28.6% |  | 33.3% |
| EL only (Test vs Ref) | -0.8% | -0.8% | -0.7% | -0.3% | -0.3% | -0.3% |  | -2.2% |  | -1.9% |  | -1.9% |
| Enc Time[%] | 100.6% |  |  | 100.6% |  |  |  | 101.0% |  |  |  |  |
| Dec Time[%] | 107.0% |  |  | 107.2% |  |  |  | 111.6% |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  | #DIV/0! |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  | Matched |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |  | LD-P HEVC SNR |  |  |  |  |
|  | Y | U | V | Y | U | V |  | Y | U |  |  | V |
| Class A | -1.0% | -0.7% | -0.7% |  |  |  |  | -3.5% | -2.2% |  |  | -2.2% |
| Class B | -0.5% | -0.5% | -0.2% | -0.8% | -0.8% | -0.8% |  | -1.7% | -1.3% |  |  | -1.0% |
| Overall (Test vs Ref) | -0.7% | -0.5% | -0.3% | -0.8% | -0.8% | -0.8% |  | -2.2% | -1.5% |  |  | -1.3% |
| Overall (Test vs single layer) | 25.8% | 37.1% | 38.7% | 22.0% | 31.8% | 34.6% |  | 20.6% | 32.6% |  |  | 37.6% |
| EL only (Test vs Ref) | -1.0% | -0.9% | -0.7% | -0.9% | -1.0% | -0.9% |  | -3.2% | -2.5% |  |  | -2.3% |
| Enc Time[%] | 100.7% |  |  | 101.0% |  |  |  | 100.9% |  |  |  |  |
| Dec Time[%] | 111.2% |  |  | 111.3% |  |  |  | 114.4% |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  | #DIV/0! |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  | Matched |  |  |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y |  | U |  |  | V |
| Class A | -0.9% | -0.6% | -0.5% |  |  |  | -2.1% |  | -1.5% |  |  | -1.6% |
| Class B | -0.2% | -0.2% | -0.3% | -0.3% | -0.5% | -0.5% | -0.8% |  | -0.9% |  |  | -0.8% |
| Overall (Test vs Ref) | -0.4% | -0.3% | -0.4% | -0.3% | -0.5% | -0.5% | -1.2% |  | -1.0% |  |  | -1.0% |
| Overall (Test vs single layer) | 28.0% | 38.3% | 39.7% | 24.4% | 32.5% | 35.0% | 22.8% |  | 33.3% |  |  | 38.1% |
| EL only (Test vs Ref) | -0.7% | -0.6% | -0.7% | -0.2% | -0.5% | -0.4% | -1.7% |  | -1.6% |  |  | -1.6% |
| Enc Time[%] | 100.3% |  |  | 100.0% |  |  | 100.2% |  |  |  |  |  |
| Dec Time[%] | 114.3% |  |  | 112.8% |  |  | 116.9% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |  |  |  |

# Result comparison with experiment 4.2.1 of SCE4 [3]

The picture-based HF-IL-SAO presented in [3] is implemented on SHVC base software SHM1.0 where a 3-tap smooth filter ([5 6 5]/16) I applied to create a low-pass picture. This picture is then subtracted to the base layer reconstructed picture. According to the results presented in Table 1 for the HF-IL-SAO, the  decoding time is quite high with a big increase of the decoding time compared to SHM1.0 (on average 193%). Please note that we have run SCE4.2.1 software on homogenous cluster to measure encoder and decoder run time.

|  | BDR |  |  | Run Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | ENC | DEC |
| AI HEVC 2x | 0.0% | -0.4% | -0.4% | 101.1% | 151.4% |
| AI HEVC 1.5x | -0.4% | -0.6% | -0.7% | 100.9% | 155.5% |
| RA HEVC 2x | -0.3% | -0.5% | -0.6% | 100.9% | 210.4% |
| RA HEVC 1.5x | -0.6% | -0.8% | -0.8% | 100.6% | 202.2% |
| RA HEVC SNR | -1.3% | -0.7% | -0.7% | 100.3% | 221.8% |
| LD-P HEVC 2x | -0.7% | -1.2% | -1.2% | 101.3% | 198.6% |
| LD-P HEVC 1.5x | -1.0% | -2.0% | -1.7% | 101.0% | 193.1% |
| LD-P HEVC SNR | -2.2% | -1.6% | -1.7% | 100.7% | 213.6% |
| Average | -0.8% | -1.0% | -1.0% | 100.9% | 193.3% |

Table 1: Summary of the results of the HF-IL-SAO presented in SCE4.2.1[3] for Intra BL case.

The results presented in this contribution shows a very limited increase of the decoding time with and average decoding time of 108% and with a similar Y-BDR performance as shown in the following table.

|  | BDR |  |  | Run Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | ENC | DEC |
| AI HEVC 2x | -0.4% | -0.6% | -0.6% | 100.7% | 104.0% |
| AI HEVC 1.5x | -0.2% | -0.4% | -0.5% | 100.9% | 105.1% |
| RA HEVC 2x | -0.5% | -0.3% | -0.4% | 101.1% | 106.4% |
| RA HEVC 1.5x | -0.3% | -0.5% | -0.5% | 100.7% | 107.0% |
| RA HEVC SNR | -1.4% | -0.6% | -0.5% | 100.5% | 110.7% |
| LD-P HEVC 2x | -0.7% | -0.6% | -0.5% | 101.0% | 108.8% |
| LD-P HEVC 1.5x | -0.8% | -0.9% | -0.9% | 100.7% | 109.8% |
| LD-P HEVC SNR | -2.2% | -0.9% | -0.8% | 101.5% | 113.9% |
| Average | -0.8% | -0.6% | -0.6% | 100.9% | 108.8% |

Table 2: Summary of the results of the modified SAO presented in that current contribution for Intra BL case.

Please note that similar results in terms of encoding and decoding time are observed for the Ref_idx case.

# Complexity measures

Complexity analysis, estimated according to the methodology described in JCTVC-L0440, is provided in the attached excel files. Three files are provided: the first one corresponds to the IntraBL case and the two other ones to the Refidx approach using either a picture base up-sampling approach or a CU-based up-sampling approach.

These measures show that the proposed method introduce new additions and multiplications for determining the edge index but it does not affect at all the worst case.

# Conclusion

This contribution proposes a simplification of SAO interlayer in order to significantly reduce the complexity at the SHM decoder while preserving the coding efficiency of the HF-IL-SAO approach.

The BDR performance of the proposed solution respectively gives -0.8%(Y), -0.6%(U), -0.6%(V) over the SHM1.0 for the IntraBL case and gives -0.8%(Y), -0.7%(U), -0.6%(V) over the SHM1.0 for the Ref_Idx case.

# References

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, Joint Collaborative Team on Video Coding, JCTVC-L1109, January 2013.

- W. Pu, J. Chen, K. Rapaka, M. Karczewicz, “High Frequency Pass Inter Layer Sample Adaptive Offset Filter”, Joint Collaborative Team on Video Coding, JCTVC-L0234, January 2013.

- W. Pu, J. Chen, X. Li, M. Karczewicz, “SCE4: High Frequency Pass Inter Layer Sample Adaptive Offset Filter”, Joint Collaborative Team on Video Coding, JCTVC-M0265, April 2013

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).