| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0115 |
| --- | --- |

| Title: | Non-SCE1: simplification of intra mode coding in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François ShaSha Shi Christophe Gisquet Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a simplification of the intra mode coding for the Enhancement Layer, by reducing the number of remaining modes to a limited set of M=2 or 4 modes instead of 32 as in the current SHM design. Thanks to this modes number reduction, the number of modes to be checked is significantly reduced. The change can be normative (remaining modes are coded using less bits) or non-normative. The reported results for the All-Intra configurations are as follows: 1) non-normative change, with M=2, encoding time reduction of 19% with an average BDR-Y variation of 0.10% in luma; 2) non-normative change, with M=4, encoding time reduction of 13% with an average BDR-Y variation of 0.06% in luma; 3) normative change, with M=2, encoding time reduction of 19% with an average BDR-Y variation of -0.11% in luma; 4) normative change, with M=4, encoding time reduction of 13% with an average BDR-Y variation of -0.10% in luma. The impact for chroma and in inter configurations is reportedly negligible.

# Introduction

For intra-coded EL CUs, it turns out that the intra prediction mode is often equal or very close to the intra prediction mode from the co-located BL CU (noted BLmode), when this BL CU is intra coded. This property can be exploited for a more efficient intra mode coding process in the EL.

The contribution proposes a simplified process for the intra mode coding, by reducing the number of remaining modes (currently equal to 32) to a limited set of M modes, M being set to 2 or 4 in our experiments. This leads to a noticeable complexity reduction.

# Proposal

## Non-normative proposal

In the non-normative proposal, the number of possible remaining modes rem[k], k=0..M-1, is limited to M=2 or 4 when the co-located BL CU is intra. In that case, the set of possible remaining modes depends on the mode of the co-located BL CU or on the MPM0. The selection algorithm of the set of remaining modes works as follows:

- If BLmode is not intra, the normal process applies with 35 possible intra modes.

- Otherwise (BLmode is intra), the following process applies:

  - BLmode is inserted into the set of remaining modes if it is not equal to any of the MPMs.

  - If BLmode is angular, the set is completed by the M neighboring modes closest to BLmode and which are not equal to any of the MPMs.

  - Otherwise if MPM0 is angular, the set is completed by the M neighboring modes closest to the MPM0 and which are not equal to any of the MPMs.

  - Otherwise (BLmode and MPM0 are not angular), the set corresponds to the M first neighboring modes which are not equal to any of the MPMs from the set {26, 10, 18, 34, 2, 22, 14}.

As in the Core HEVC, the neighboring modes of a mode m are computed as follows:

Lower mode: 2 + ( ( m + 29 ) % 32 )
Higher mode: 2 + ( ( m − 2 + 1 ) % 32 )

The process to derive this set applies only at the encoder (non-normative change).

Modifications of the modes checking process

The modes checking process is slightly modified. In the SHM1 (as in the HM), the intra mode checking process works in two steps. First a fast HAD research is achieved on all available (35) modes; then the N best candidates, N=3 or 8, depending on the CU size, plus eventually missing MPMs, are checked in the full RDO loop to select the optimal intra mode.

In this proposal, the maximum number of available modes is 5 (3 MPMs + M=2 remaining modes) or 7 (3 MPMs + M=4 remaining modes). Therefore, when the number N of candidates selected by the fast HAD search is 8, this fast HAD search is not required and the full RDO search can directly apply to the 5 or 7 available modes.

## Normative proposal

A normative version of the algorithm is also proposed. In this version, the modes reduction is always applied, whatever the mode of the collocated BL CU is (intra or not). So it is possible to reduce the number of bits required to code the remaining mode. The remaining mode is coded with 1 bit if M=2, 2 bits if M=4. This normative change enables to save bits to code the intra mode.

The selection algorithm of the set of remaining modes works as follows:

- If BLmode is not intra, the set corresponds to the M first neighboring modes which are not equal to any of the MPMs from the set {26, 10, 18, 34, 2, 22, 14}.

- Otherwise (BLmode is intra), the following process applies:

  - BLmode is inserted into the set of remaining modes if it is not equal to any of the MPMs.

  - If BLmode is angular, the set is completed by the M neighboring modes closest to BLmode and which are not equal to any of the MPMs.

  - Otherwise if MPM0 is angular, the set is completed by the M neighboring modes closest to the MPM0 and which are not equal to any of the MPMs.

  - Otherwise (BLmode and MPM0 are not angular), the set corresponds to the M first neighboring modes which are not equal to any of the MPMs from the set {26, 10, 18, 34, 2, 22, 14}.

# Results

The two next sub-sections present the results of the proposal in case of non-normative or normative change for the All-Intra 2x and 1.5x configurations. The full results, including the inter configurations, are provided in the attached xls files.

The table below summarizes the All Intra results averaged for the Intra BL and Ref Index frameworks and for the ratios 2x and 1.5x. It is observed a significant encoding time reduction of 19% when M=2, and of 13% when M=4. The decoding time can also be slightly decreased in several cases. An average BDR-Y loss below 0.1% is observed when the change is non-normative, and an average BDR-Y gain of 0.1% is observed when the change is normative. The impact for chroma and in inter configurations is negligible.

Table 1: average BDR and Enc/Dec time variations for All-Intra cases.

| Test | Y | U | V | Enc time [%] | Dec time [%] |
| --- | --- | --- | --- | --- | --- |
| Non-normative M=2 | 0.10% | 0.04% | 0.07% | 81% | 99% |
| Non-normative M=4 | 0.06% | 0.02% | 0.04% | 87% | 100% |
| Normative M=2 | -0.11% | -0.03% | -0.03% | 81% | 100% |
| Normative M=4 | -0.10% | -0.03% | -0.03% | 86% | 100% |

## Non-normative proposal

In this section the changes are made only at the encoder side and are therefore non-normative.

Non-normative proposal with M=2 remaining modes

The average BDR variations over the SHM1 for All-Intra configuration are of 0.10% Y, 0.04% U and 0.06% V with an average encoding run time of 81% and an average decoding run time of 99%.

For the inter configurations, the average BDR variations are negligible, with a maximum loss observed in RA-2x RefIdx framework of 0.09% Y, 0.19% U, 0.25% V. A slight encoding time decrease is observed.

Table 2: performance of non-normative proposal with M=2 remaining modes

| INTRA BL framework | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.08% | -0.03% | -0.05% |  |  |  |
| Class B | 0.21% | 0.10% | 0.13% | 0.03% | 0.02% | 0.04% |
| Overall (Test vs Ref) | 0.17% | 0.07% | 0.08% | 0.03% | 0.02% | 0.04% |
| Overall (Test vs single layer) | 12.6% | 13.7% | 13.5% | 10.3% | 10.3% | 9.8% |
| EL only (Test vs Ref) | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 80.4% |  |  | 82.3% |  |  |
| Dec Time[%] | 98.4% |  |  | 99.1% |  |  |

| REF IDX framework | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.07% | -0.04% | -0.05% |  |  |  |
| Class B | 0.19% | 0.12% | 0.18% | 0.03% | 0.00% | 0.03% |
| Overall (Test vs Ref) | 0.16% | 0.07% | 0.12% | 0.03% | 0.00% | 0.03% |
| Overall (Test vs single layer) | 13.0% | 15.1% | 14.9% | 10.6% | 9.9% | 9.5% |
| EL only (Test vs Ref) | 0.1% | 0.0% | 0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 80.5% |  |  | 82.2% |  |  |
| Dec Time[%] | 99.4% |  |  | 98.7% |  |  |

Non-normative proposal with M=4 remaining modes

The average BDR variations over the SHM1 are of 0.06% Y, 0.02% U and 0.04% V with an average encoding run time of 87% and an average decoding run time of 100%.

For the inter configurations, the average BDR variations are negligible, with a maximum loss observed in RA-2x RefIdx framework of 0.06% Y, 0.09% U, 0.14% V. A slight encoding time decrease is observed.

Table 3: performance of non-normative proposal with M=4 remaining modes

| INTRA BL framework | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.04% | -0.03% | -0.03% |  |  |  |
| Class B | 0.14% | 0.05% | 0.07% | 0.02% | 0.01% | 0.03% |
| Overall (Test vs Ref) | 0.11% | 0.03% | 0.04% | 0.02% | 0.01% | 0.03% |
| Overall (Test vs single layer) | 12.5% | 13.7% | 13.4% | 10.3% | 10.3% | 9.7% |
| EL only (Test vs Ref) | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 86.4% |  |  | 87.6% |  |  |
| Dec Time[%] | 99.7% |  |  | 100.0% |  |  |

| REF IDX framework | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.04% | -0.03% | -0.03% |  |  |  |
| Class B | 0.13% | 0.07% | 0.11% | 0.02% | 0.00% | 0.02% |
| Overall (Test vs Ref) | 0.10% | 0.04% | 0.07% | 0.02% | 0.00% | 0.02% |
| Overall (Test vs single layer) | 12.9% | 15.1% | 14.8% | 10.6% | 9.9% | 9.5% |
| EL only (Test vs Ref) | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 86.4% |  |  | 87.9% |  |  |
| Dec Time[%] | 100.2% |  |  | 99.8% |  |  |

## Normative proposal

In this section the changes are normative: the number of modes is always restricted to M, M being 2 or 4, and the number of bits for coding the remaining mode is set to 1 or 2 respectively.

Normative proposal with M=2 remaining modes

The average BDR variations over the SHM1 for All-Intra configuration are of -0.11% Y, -0.02% U and -0.01% V with an average encoding run time of 81% and an average decoding run time of 100%.

For the inter configurations, the average BDR variations are negligible, with a maximum loss observed in LDP-2x RefIdx framework of 0.13% Y, 0.40% U, 0.54% V. A slight encoding time decrease is observed.

Table 4: performance of normative proposal with M=2 remaining modes

| INTRA BL framework | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.18% | -0.11% | -0.04% |  |  |  |
| Class B | -0.17% | -0.12% | -0.13% | -0.05% | 0.04% | 0.03% |
| Overall (Test vs Ref) | -0.17% | -0.12% | -0.10% | -0.05% | 0.04% | 0.03% |
| Overall (Test vs single layer) | 12.2% | 13.5% | 13.3% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.3% | -0.2% | -0.2% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 79.8% |  |  | 82.0% |  |  |
| Dec Time[%] | 99.2% |  |  | 100.0% |  |  |

| REF IDX framework | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.17% | -0.09% | -0.04% |  |  |  |
| Class B | -0.16% | -0.12% | -0.11% | -0.05% | 0.06% | 0.06% |
| Overall (Test vs Ref) | -0.16% | -0.11% | -0.09% | -0.05% | 0.06% | 0.06% |
| Overall (Test vs single layer) | 12.6% | 14.9% | 14.6% | 10.5% | 10.0% | 9.6% |
| EL only (Test vs Ref) | -0.3% | -0.2% | -0.2% | -0.1% | 0.1% | 0.1% |
| Enc Time[%] | 80.6% |  |  | 83.0% |  |  |
| Dec Time[%] | 100.9% |  |  | 99.3% |  |  |

Normative proposal with M=4 remaining modes

The average BDR variations over the SHM1 are of -0.10% Y, -0.03% U and -0.03% V with an average encoding run time of 86% and an average decoding run time of 100%.

For the inter configurations, the average BDR variations are negligible, with a maximum loss observed in LDP-2x RefIdx framework of 0.11% Y, 0.37% U, 0.45% V.

Table 5: performance of normative proposal with M=4 remaining modes

| INTRA BL framework | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.16% | -0.06% | -0.01% |  |  |  |
| Class B | -0.16% | -0.10% | -0.11% | -0.04% | 0.03% | 0.02% |
| Overall (Test vs Ref) | -0.16% | -0.09% | -0.08% | -0.04% | 0.03% | 0.02% |
| Overall (Test vs single layer) | 12.2% | 13.5% | 13.3% | 10.3% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.2% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 85.2% |  |  | 86.8% |  |  |
| Dec Time[%] | 99.1% |  |  | 100.0% |  |  |

| REF IDX framework | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.16% | -0.05% | -0.01% |  |  |  |
| Class B | -0.15% | -0.10% | -0.11% | -0.04% | 0.05% | 0.04% |
| Overall (Test vs Ref) | -0.15% | -0.09% | -0.08% | -0.04% | 0.05% | 0.04% |
| Overall (Test vs single layer) | 12.7% | 14.9% | 14.7% | 10.5% | 10.0% | 9.5% |
| EL only (Test vs Ref) | -0.2% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 85.6% |  |  | 85.7% |  |  |
| Dec Time[%] | 100.2% |  |  | 100.3% |  |  |

# Conclusion

In this contribution a simplified process was proposed for the remaining mode coding, by reducing the number of remaining modes to a limited set of 2 or 4 modes instead of 32 as in the current SHM design.

The proposed modification leads to a noticeable encoding time decrease (around 15-20%) with also a slight decoding time decrease for the All-Intra configurations. The impact in Inter configurations is negligible.

When the change is non-normative (encoder only), a slight loss below 0.1% is observed in luma for the All-Intra configurations.

When the change is normative (encoder only), a slight gain around 0.1% is observed in luma for the All-Intra configurations.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).