| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0116 |
| --- | --- |

| Title: | Non-RCE1: Multiple LM chroma modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chih-Wei Hsu, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, cw.hsu, chiayang.tsai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, multiple LM chroma modes are implemented on top of JCTVC-M0097, where the prior LM chroma mode is extended for non-420 video formats. Three LM chroma modes are proposed. One is the prior LM chroma mode using above and left neighboring reconstructed samples to derive parameters of the LM linear model, and the other two modes only use left or above neighboring reconstructed samples. Simulation result shows that the average BD-rates of the proposed method for the three color components are -8.5%, -11.6%, and -11.7% for AI-Main-tier, -6.9%, -8.3%, and -9.4% for AI-High-tier, and -5.4%, -5.9%, and -6.8% for AI-SuperHigh-tier. For intra test cases, the encoding time is increased by 21%, and the decoding time is roughly unchanged.

# Proposed method

In the 12th JCT-VC meeting, the LM chroma mode, which predicts chroma samples by using luma samples with a linear model, is extended for non-420 video formats in JCTVC-L0240 [1], and a core experiment [2] is established to further investigate the performance of the LM chroma mode. In the LM chroma mode, parameters of a linear model are derived by using above and left neighboring reconstructed samples around the current block, as shown in Fig. 1(a), where the video format is YUV420, one square is one sample, black squares are samples in the current block, gray squares are reconstructed samples around the current block, and those samples used to derive parameters are represented as circles. However, when the current block is located at object boundaries, not all the neighboring samples are highly correlated to the current block. If all the neighboring samples are used to derive LM parameters in this case, it will degrade the performance of LM chroma mode. In order to solve this problem, two additional LM chroma modes are proposed with explicit mode signaling, and different neighboring samples are used to derive LM parameters in these two LM chroma modes. One new mode is a left-only LM chroma mode, which only uses left neighboring reconstructed samples around the current block to derive LM parameters, as shown in Fig. 1(b); the other is a above-only LM chroma mode, which only uses above neighboring reconstructed samples around the current block to derive LM parameters, as shown in Fig. 1(c). The proposed multiple LM chroma modes are implemented on top of JCTVC-M0097 [3], which is the software of RCE1.

(a)

(b)

(c)

Figure 1.  The proposed multiple LM chroma modes: (a) left and above LM chroma mode; (b) left-only LM chroma mode; (c) above-only LM chroma mode

# Experimental results

The experimental results are shown in Table 1. The software is developed based on the JCTVC-M0097 [3], and the test conditions are described in JCTVC-L1006 [4]. Compared to the anchor, the proposed multiple LM chroma modes can achieve 8.5%, 11.6%, 11.7% BD-rate reductions in AI-Main-tier, 6.9%, 8.3%, 9.4% BD-rate reductions in AI-High-tier, and 5.4%, 5.9%, 6.8% BD-rate reductions in AI-SuperHigh-tier, for the three color components, respectively. Compared to the traditional LM chroma mode, additional 0.9% and 1.3% chroma BD-rate gains for AI-Main-tier, 0.6% and 1.0% chroma BD-rate gains for AI-High-tier, and 0.5% and 0.7% chroma BD-rate gains for AI-SuperHigh-tier can be provided. For intra test cases, the encoding time increase is 21%, and the decoding time is roughly unchanged.

Table 1.  Results of the proposed multiple LM chroma modes

# Conclusions

In this proposal, multiple LM chroma modes are implemented on top of the extended LM chroma mode for non-420 video formats in JCTVC-M0097. It is proposed to adopt the extended LM chroma mode first and study the multiple LM chroma modes in a range extensions core experiment.

# References

- J. Kim, B. Jeon, “AHG7: The performance of extended intra chroma prediction for non 4:2:0 format,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0240, January, 2013.

- J. Sole, K. Kawamura, J. Kim, “HEVC Range Extensions Core Experiment 1 (RCE1): Inter component decorrelation methods,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1121, January, 2013.

- J. Kim, “RCE1: The performance of extended chroma mode for non 4:2:0 format,” Document of Joint Collaborative Team on Video Coding, JCTVC-M00997, April, 2013.

- D. Flynn, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1006, January, 2013.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).