| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0117 |
| --- | --- |

| Title: | Non-CE1: Weighted intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | [82-10-8893-4332] [chulkeun.kim@lge.com] |
| Source: | LG Electronics |  |  |

# Abstract

This contribution presents weighted intra prediction (WIP) that uses reconstructed base layer texture to improve enhancement layer sample predictor. It only applied on intra CU. CU level on/off flag is signalled for WIP. Simulation results reportedly show 0.2% and 0.1% BD-rate savings on average for AI-2x and AI-1.5x, respectively, compared with SHM 1.0 anchors.

# Proposed methods

## Weighted intra prediction

P(X,Y) represents prediction sample of the current layer produced via intra prediction while R(X,Y) represents reference sample which was used for intra prediction. Dh and DV represent horizontal and vertical distances between P and R respectively. As Dh and Dv increase, spatial correlation decreases so that accuracy of P is reduced.  In order to compensate this, in the present contribution, a method of using P, which is created by intra prediction of the current layer, and collocated sample B of the base layer at the same time is proposed according to Dh and Dv.

Figure 1: Weighted Intra Prediction

Equation (1) shows the process of how to obtain the final prediction value  P’x,y .

PVx,y  = (N – y)·Px,y  +  y·Bx,y

PHx,y  = (N – x)·Px,y  +  x·Bx,y

P’x,y    = (PVx,y + PHx,y  + N)  >>  (log2(N) + 1) 					(1)

In the proposed method, as a distance between P and R increases, weighted value of B also increases. Reduction of accuracy of P due to increase of distance can be compensated by using the information of the lower layer at the same position. In this proposal, an WIP_flag is signaled at coding unit (CU) level after the pred_mode. If the intra_BL_flag is false and pred_mode is intra, WIP_flag is signaled.

(a)                                     (b)

Figure 2. Flow chart of the RD-cost checking.

Figure 2 shows the RD-cost check flow chart to determine the application of the proposed method. Figure 2 (a) represents rd-cost check with regard to the proposed method once rd-cost of normal intra prediction is checked. This case is equivalent to performing intra rd-cost check twice with regard to the corresponding CU so that encoding complexity increases significantly. Figure 2 (b) represents a method of checking rd-cost once the proposed method is applied to the best mode of normal intra prediction, in order to reduce the complexity. Because of this, complexity can be reduced but coding efficiency also decreased.

# Simulation results

Table 1shows the RD-cost check as shown in Figure 2 (a). It shows that while BD-rate gain is 0.3%, complexity increases by 92.1%. Table 2 shows the case of RD-cost check as shown in Figure 2 (b), in which complexity increases by 9.6% while BD-rate gain shows 0.2%.

Table 1. Results of the proposed method(RD-cost check as shown in Figure 2 (a))

Table 2. Results of the proposed method(RD-cost check as shown in Figure 2 (b))

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).