| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0118 |
| --- | --- |

| Title: | Removal of the offset quantization in SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Ching-Yeh Chen, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, chingyeh.chen, chiayang.tsai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to remove the quantization process of offsets in SAO for sample bit depths larger than 10. When applied to HEVC range extensions common test conditions, the proposed method reportedly achieves 0.1%, 0.1%, 0.2%, x.x%, x.x%, x.x%, and x.x%, BD-rate reductions in AI Main-tier, AI High-tier, AI Super High-tier, RA Main-tier, RA High-tier, LB Main-tier, and LB High-tier, respectively, with unchanged run times.

# Proposed method

In HEVC, SAO is applied as one of the two in-loop filters, and SAO syntax elements are coded in the beginning of each CTU including merge flags, SAO types, offsets, signs and band position for BO or classes for EO. For the offsets, absolute values are coded, and a quantization process is applied to the absolute values when the sample bit depth is larger than 10. Therefore, when decoding the offsets, the parsed offset values need to be inversely quantized when the sample bit depth is larger than 10, as shown in the following equations specified in the HEVC FDIS [1].
  SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] =							(7-1)
		offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  ( bitDepth − Min( bitDepth, 10 ) )
In this contribution, the effect of the quantization process for the offsets in SAO is evaluated for HEVC range extensions (HEVC RExt) since higher sample bit depths may be used in HEVC RExt. After the evaluation, we propose to remove the quantization process for the offsets in SAO when the sample bit depth is larger than 10.

# Experimental results

The experimental results are shown in Table 1. The software is developed based on HM10.0_RExt2.0 branch, revision 3381, and the test condition is described in JCTVC-L1006 [2]. The internal sample bit depths for both the anchor and the test are set to 12, and the SAO quantization process is removed for the test. Compared to the anchor, the proposed method can achieve 0.1%, 0.1%, 0.2%, x.x%, x.x%, x.x%, and x.x%, BD-rate reductions in AI Main-tier, AI High-tier, AI Super High-tier, RA Main-tier, RA High-tier, LB Main-tier, and LB High-tier, respectively. Run times are roughly unchanged.

Table 1.  Results of the proposed method (to be updated)

# Conclusions

In this contribution, it is proposed to remove the quantization process of offsets in SAO for sample bit depths larger than 10. We recommend adopting this proposal in order to solve the performance degradation problem when SAO is used for sample bit depths more than 10.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent),” Document of Joint Collaborative Team on Video Coding, JCTVC-L1003, January, 2013.

- D. Flynn, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1006, January, 2013.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).