| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0119 |
| --- | --- |

| Title: | SCE3.1: Adaptive predictor compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang, Polin Lai, Shan Liu, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yuwen.huang, polin.lai, shan.liu, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, adaptive predictor compensation (APC) using reconstructed base layer (BL) texture is applied to refine enhancement layer (EL) sample predictors. For each inter-coded coding unit (CU) that has a partition mode equal to 2Nx2N and at least one non-zero residue, the proposed APC can be applied with an explicit APC enabling flag at CU level. If the APC is applied, the EL sample predictors are generated by averaging the EL original sample predictors and the BL reconstructed samples of the corresponding block. Simulation results reportedly show 0.4%, 0.5%, 0.7%, 1.4%, 1.4%, 2.4%, 0.6%, 0.4%, and 0.9% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, LDP-SNR, LDB-2x, LDB-1.5x, and LDB-SNR, respectively, compared with SHM-1.0 IntraBL mode anchors. The encoding time increase is 4%, and the decoding time increase is about zero. The average bandwidth increase is 4%. The average computation increases in terms of adders and multipliers are about zero. The worst case bandwidth increase is 33%. The worst case computation increases in terms of adders and multipliers are 41% and 44%, respectively.

# Introduction

In scalable video coding, the current base layer (BL) reconstructed picture may predict the current enhancement layer (EL) picture better than the EL reference picture for regions that newly appear or have light changes. If the BL quantization distortion is not large, the EL original samples should be close to the BL reconstructed samples. Therefore, in the EL inter prediction, the BL reconstructed texture can be used to refine the EL motion compensated predictors, which was proposed in JCTVC-L0072 [1] and are further investigated on top of SHM-1.0 IntraBL mode as a part of Scalable Extensions Core Experiment 3 (SCE3) [2]. The average and worst case bandwidth and computations are also investigated in this contribution. In this contribution, the proposed technique is termed adaptive predictor compensation (APC). The rest of this contribution is organized as follows. The proposed algorithm will be described in Section 2, simulation results, bandwidth and computation analyses will be shown in Section 3, and finally conclusions will be given in Section 4.

# Proposed method

In JCTVC-L0072, APC was proposed to use the BL reconstructed samples to refine the EL predictors. The refined EL sample predictors are generated by averaging the EL original sample predictors and the BL reconstructed samples of the corresponding block, as shown by the following equation.
	PredEL’ = (PredEL + RecBL) >> 1								(1)
The proposed APC is only applied to the luma component of each inter-coded coding unit (CU) with a partition mode equal to 2Nx2N and at least one non-zero residue. In addition, a CU-level APC_enable_flag is signaled to indicate whether the proposed APC is applied.

# Simulation results

## BD-rates and run times

Experiments were conducted according to the SCE3 description defined in JCTVC-L1103 [2], and the software is based on SHM-1.0 IntraBL mode. As shown in Table 1, the proposed APC reportedly achieves 0.4%, 0.5%, 0.7%, 1.4%, 1.4%, 2.4%, 0.6%, 0.4%, and 0.9% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, LDP-SNR, LDB-2x, LDB-1.5x, and LDB-SNR, respectively. The encoding time increase is 4%, and the decoding time is roughly unchanged.

## Bandwidths and computations

The bandwidths and computations of APC are also evaluated. Experiments were conducted according to the methodology defined in JCTVC-L0440 [3], and the AHG17 released software is used. The detailed results can be found in JCTVC-M0119_Bandwidths&Computations.xls. The average bandwidths and computations are shown in Table 2. Roughly speaking, the bandwidth increase is 4%, and the computation increases in terms of adders and multipliers are zero. The worst case bandwidths, computations, and memory usages are summarized in Table 3. The worst case bandwidth increase is 33%. The worst case computation increases in terms of adders and multipliers are 41% and 44%, respectively. The memory usages in terms of number of reference frames and table size are the same as SHM-1.0.

# Conclusions

The adaptive predictor compensation (APC) proposed in JCTVC-L0072 is investigated on SHM-1.0. Simulation results reportedly show 0.4%, 0.5%, 0.7%, 1.4%, 1.4%, 2.4%, 0.6%, 0.4%, and 0.9% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, LDP-SNR, LDB-2x, LDB-1.5x, and LDB-SNR, respectively. The encoding time is roughly increased by 4%, and the decoding time is roughly unchanged. The average bandwidth increase is 4%. The average computations in terms of adders and multipliers are roughly the same as the anchor. The worst case bandwidth increase is 33%. The worst case computation increases in terms of adders and multipliers are 41% and 44%, respectively.

# References

- T.-D. Chuang, Y.-W. Huang, P. Lai, S. Liu, S. Lei, “TE3-4.3.1: Adaptive predictor compensation by using reconstructed BL texture,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0072, January, 2013.

- X. Li, E. François, P. Lai, D. Kwon, A. Saxena, “Description of Scalable Extensions Core Experiment 3 (SCE3): Combined inter and inter-layer prediction in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1103, January, 2013.

- E. François, A. Tabatabai, E. Alshina, “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0440, January, 2013.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  BD-rates and run times of the APC

Table 2.  Average bandwidths and computations of the APC

Table 3.  Worst case bandwidths and computations of the proposed method

|  | The worst case computations |  | The worst case bandwidth |  | Memory usage |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Multipliers | Adders | MemBand (2D:4x2) | MemBand (2D:8x2) | Number of Ref. Frames | Tables Size |
| Proposed | 141% | 144% | 133% | 133% | 100% | 100% |