| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0120_r2 |
| --- | --- |

| Title: | On signaling the syntax ‘sps_inter_layer_mfm_enable_flag’ |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jung Won Kang Jinho Lee Jin Soo Choi Visual Media Research Section, ETRI 218 Gajeongno, Yuseong-gu Daejeon, South Korea | Tel: Email: | +82-42-860-6138 hanilee@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

In the reference index based SHVC framework, when the sps_inter_layer_mfm_enable_flag in SPS extension is equal to 1, motion field mapping process between the current layer and its reference layer is performed in order to use the inter-layer reference picture as a collocated picture for TMVP derivation. In this proposal, it is asserted that sps_inter_layer_mfm_enable_flag should be sent only when sps_temporal_mvp_enabled_flag is equal to 1 since, if TMVP is not used, sps_inter_layer_mfm_enable_ flag should always be equal to 0. In addition, it is proposed to add constraint on collocated_ref_idx for TMVP not to indicate inter-layer reference picture when sps_inter_layer_mfm_enable_flag is equal to 0. This document also provides approaches for adding constraint on collocated picture for TMVP derivation for reducing the memory requirement of motion information of reference picture in the reference index based SHVC framework.

# Problems statements

In the reference index based SHVC framework [1], a flag called ‘sps_inter_layer_mfm_enable_flag’ is signaled in the sequence parameter sets (SPS) extension to indicate whether motion field mapping (MFM) process is applied or not. When this flag is equal to 1, the motion field mapping applied as part of the resampling process for reference pictures. With MFM process, inter-layer motion parameters prediction can be enabled by set the inter-layer reference picture as the collocated picture for temporal motion vector prediction (TMVP) derivation [2]. It is a good feature since it gives a chance to enable inter-layer motion parameter prediction without any low-level modification. However, when TMVP is disabled, there is no way to enable the inter-layer motion parameter prediction in the current reference index based SHVC framework. This means that when TMVP is disabled, MFM process is unnecessary so that ‘sps_inter_layer_mfm_enable_flag’ usually be set to 0. In such case, ‘sps_inter_layer_mfm_enable_flag’ cannot be signaled.

Additionally, collocated picture for TMVP is indicated by ‘collocated_ref_idx’ syntax signaled in the slice segment header. In current semantic of ‘collocated_ref_idx’ specifies the reference index of the collocated picture used for TMVP and the value of it shall be in the range of 0 to num_ref_idx_lx_active_minus1. This means that any of reference pictures in the reference picture list can be specified as collocated picture of TMVP according to the semantic. However, when the MFM is disabled, current semantic can incur the situation in which ‘collocated_ref_idx’ can indicate the inter-layer reference picture as collocated picture for TMVP even though inter-layer reference picture does not have any motion parameters. In this situation, some unintelligent encoder cannot take advantage of possibility to use the TMVP as candidates of merge or motion vector prediction.

It is also observed that when ‘sps_inter_layer_mfm_enable_flag’ is equal to 1, the colloated picture for TMVP derivation can be both reference pictures with the same layer index as the current picture and the inter-layer reference pictures. So, the motion information of temoporal reference pictures always has to be stored. However, if we constrain the collocatd picture for TMVP derivation to be only inter-layer reference picture, the memory requirement for storing the motion information of reference pictures can be reduced.

Proposals

## Proposal 1

To deal with the above problems, we propose to the following modification.

- The flag ‘sps_inter_layer_mfm_enable_flag’ is signaled when ‘sps_temporal_mvp_enabled_flag’ is equal to 1.

- With the above point, the syntax table for SPS extension should be modified as below

| sps_extension( ) { | Descriptor |
| --- | --- |
| if ( sps_temporal_mvp_enabled_flag ) |  |
| sps_inter_layer_mfm_enable_flag | u(1) |
| } |  |

- To prevent the situation in which inter-layer reference picture without motion parameters is specified as collocated picture for TMVP, we clarifies the semantic of ‘sps_inter_layer_mfm_enable_flag’ in the SPS extension and put the note in the semantic of ‘collocated_ref_idx’ syntax signaled in the slice segment header as follows.

sps_inter_layer_mfm_enable_flag equal to 1 specifies the motion field mapping process is applied as part of the resampling process for reference pictures specified in subclause Error: Reference source not found. sps_inter_layer_mfm_enable_flag equal to 0 specifies that the motion field mapping processing is not applied. When not present, the value of sps_inter_layer_mfm_enable_flag is inferred to be equal to 0.  When avc_base_layer_flag is equal to 1, sps_inter_layer_mfm_enable_flag shall be equal to 0. When sps_inter_layer_mfm_enable_flag is equal to 0, the inter-layer reference picture shall not be specified as collocated picture for TMVP derivation.

collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction. When slice_type is equal to P or when slice_type is equal to B and collocated_from_l0 is equal to 1, collocated_ref_idx refers to a picture in list 0, and the value of collocated_ref_idx shall be in the range of 0 to num_ref_idx_l0_active_minus1, inclusive.When slice_type is equal to B and collocated_from_l0 is equal to 0, collocated_ref_idx refers to a picture in list 1, and the value of collocated_ref_idx shall be in the range of 0 to num_ref_idx_l1_active_minus1, inclusive.It is a requirement of bitstream conformance that the picture referred to by collocated_ref_idx shall be the same for all slices of a coded picture.

NOTE- when ‘sps_inter_layer_mfm_flag’ in the SPS extension is equal to 0, the picture referred to by collocated_ref_idx shall not be inter-layer reference picture.

## Proposal 2

In order to reduce the storage of motion information of reference pictures for TMVP derivation, we propose two following approaches.

### Approach 1

We propose to add constraint on the semantic of sps_inter_layer_mfm_enable_flag to use only inter-layer reference pictures as the collocated pictures for TMVP derivation when motion field mapping process is enabled. As a results, when sps_inter_layer_mfm_enable_flag is equal to 1, the motion information of temporal reference pictures are no longer stored.

| sps_extension( ) { | Descriptor |
| --- | --- |
| if ( sps_temporal_mvp_enabled_flag ) |  |
| sps_inter_layer_mfm_enable_flag | u(1) |
| } |  |

sps_inter_layer_mfm_enable_flag equal to 1 specifies the motion field mapping process is applied as part of the resampling process for reference pictures specified in subclause Error: Reference source not found. sps_inter_layer_mfm_enable_flag equal to 0 specifies that the motion field mapping processing is not applied. When not present, the value of sps_inter_layer_mfm_enable_flag is inferred to be equal to 0.  When avc_base_layer_flag is equal to 1, sps_inter_layer_mfm_enable_flag shall be equal to 0. When sps_inter_layer_mfm_enable_flag is equal to 0, the inter-layer reference picture shall not be specified as collocated picture for TMVP derivation. When sps_inter_layer_mfm_enable_flag is equal to 1, only inter-layer reference pictures shall be used as collocated picture for TMVP derivation.

### Approach 2

We proposed to a flag in the SPS extension to constrain the collocated picture for TMVP derivation when motion field mapping is enabled. As a results, when this flag is equal to 1, the motion information of temporal reference pictures are no longer stored.

| sps_extension( ) { | Descriptor |
| --- | --- |
| if ( sps_temporal_mvp_enabled_flag ) { |  |
| sps_inter_layer_mfm_enable_flag | u(1) |
| if ( sps_inter_layer_mfm_enable_flag ) |  |
| sps_temporal_mvp_constraint_flag | u(1) |
| } |  |
| } |  |

sps_inter_layer_mfm_enable_flag equal to 1 specifies the motion field mapping process is applied as part of the resampling process for reference pictures specified in subclause subclause Error: Reference source not found. sps_inter_layer_mfm_enable_flag equal to 0 specifies that the motion field mapping processing is not applied. When not present, the value of sps_inter_layer_mfm_enable_flag is inferred to be equal to 0.  When avc_base_layer_flag is equal to 1, sps_inter_layer_mfm_enable_flag shall be equal to 0. When sps_inter_layer_mfm_enable_flag is equal to 0, the inter-layer reference picture shall not be specified as collocated picture for TMVP derivation.

sps_temporal_mvp_constraint_flag equal to 1 specifies that collocated picture for TMVP derivation is constrained in the CVS. When sps_temporal_mvp_constraint_flag is equal to 1, only inter-layer reference pictures shall be used as the collocated picture for TMVP derivation. When sps_temporal_mvp_constraint_flag is equal to 0, both reference pictures with the same layer index as the current picture and the inter-layer reference pictures can be used as the collocated picture for TMVP derivation. When sps_temporal_mvp_constraint_flag is not present, it is inferred to be equal to 0.

# References

[1] J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, "SHVC Working Draft 1", JCTVC-L1008, 12th Meeting: Geneva, CH, 14–23 Jan. 2013

[2] J. Chen, V. Seregin, L. Guo, M. Karczewicz, “Non-TE5: on motion mapping in SHVC”, JCTVC-L0336, 12th Meeting: Geneva, CH, 14–23 Jan. 2013

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).