| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0123 |
| --- | --- |

| Title: | Non-CE 1: Constrained intra prediction at enhancement layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | [82-10-8893-4332] [chulkeun.kim@lge.com] |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents constrained intra prediction (CIP) at enhancement layer (EL). In the SHM 1.0, when CIP is enabled, Intra BL cannot be used in intra prediction. In the proposed technique, when CIP is enabled, an intra BL CU at the enhancement layer that refers to an intra coded CU at the base layer should be used in intra prediction. Simulation results reportedly show 1.3%, 0.2% BD-rate savings on average for AI-2x, AI-1.5x respectively, compared with SHM 1.0 anchors(CIP On).

# Introduction

Currently, when constrained intra prediction (CIP) is enabled, only intra coded neighbouring PUs can be used in intra prediction. That is, neighbouring PUs coded by an intra BL mode cannot be used in intra prediction. Hence, coding efficiency decreases when CIP is enabled.

# Proposed method

In JCTVC-L0134, we suggested that when CIP is enabled, if slice type of base layer is Intra, an intra BL CU at the enhancement layer should be used in intra prediction. Figure 1 represents proposed method in JCTVC-L0134.

Figure 1. Proposed CIP process in JCTVC-L0134

In this contribution, when CIP is enabled at both enhancement layer and base layer , an intra BL CU at the enhancement layer that refers to an intra coded CU at the base layer should be used in intra prediction.

Figure 2. Proposed CIP process

In SHM 1.0, it will check the prediction modes of the neighboring 4x4 blocks. In case of non-dyadic scalability, an enhancement layer PU may cover inter and intra PUs from base layer. For example, ‘A’ covers inter and intra PUs from base layer. ‘A’ represents neighboring 4 pixels of current PU. To solve this problem, prediction mode of collocated BL PU is checked by a pixel.

Figure 3. Problem of prediction modes checking of collocated BL PU in non-dyadic scalability.

# Experimental results

The proposed method was implemented on the SHM 1.0 and the performance was measured under both of intra and inter configuration. Table 1 shows the experimental results of proposed method. Simulation results reportedly show 1.3%, 0.2% BD-rate savings on average for AI-2x, AI-1.5x, respectively, compared with SHM 1.0 anchors(CIP On).

Table 1. Experimental results

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).