- H.8.5.3.2.1 Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag [ xPb ][  yPb ] is equal to 1, where ( xPb, yPb ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are:

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The location ( xOrigP, yOrigP ) and the variables nOrigPbW and nOrigPbH are derived to store the values of ( xPb, yPb ) , nPbW and nPbH as follows:

( xOrigP, yOrigP ) is set equal to ( xPb, yPb )		(H-1)

nOrigPbW = nPbW		(H-2)

nOrigPbH = nPbH		(H-3)

When Log2ParMrgLevel is greater than 2 and nCbS is equal to 8, ( xPb, yPb ), nPbW, nPbH and partIdx are modified as follows:

( xPb, yPb ) = ( xCb, yCb )		(H-4)

nPbW = nCbS		(H-5)

nPbH = nCbS		(H-6)

partIdx = 0		(H-7)

NOTE – When Log2ParMrgLevel is greater than 2 and nCbS is equal to 8, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

- The value of availableFlagRL is set equal to 0. When InterLayerMotionPredictionEnableFlag equal to 1, the derivation process for merging candidates from co-located reference layer prediction unit partitions in subclause  is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output being the availability flags availableFlagRL, the reference indices refIdxLXRL, the prediction list utilization flags predFlagLXRL, and the motion vectors mvLXRL (with X being 0 or 1, respectively). The value of availableFlagRLH is set equal to 0. When InterLayerMotionPredictionEnableFlag equal to 1, the derivation process for merging candidates from the bottom-right prediction unit of the co-located reference layer prediction unit partitions in subclause  is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output being the availability flags availableFlagRLH, the reference indices refIdxLXRLH, the prediction list utilization flags predFlagLXRLH, and the motion vectors mvLXRLH (with X being 0 or 1, respectively).

- The derivation process for merging candidates from neighbouring prediction unit partitions in subclause  is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, ), the luma prediction block height nPbH and the partition index partIdx as well as availableFlagRL, mvLXRL, refIdxLXRL, availableFlagRLH, mvLXRLH, and refIdxLXRLH as inputs and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1, predFlagLXB2 and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1, mvLXB2 (with X being 0 or 1, respectively).

- The reference indices for the temporal merging candidate, refIdxLXCol, with X being 0 or 1, are set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the variable refIdxLXCol as inputs and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol and predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col 		(H-8)

predFlagL0Col = availableFlagL0Col		(H-9)

predFlagL1Col = 0		(H-10)

- When slice_type is equal to B, the derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(H-11)

predFlagL1Col = availableFlagL1Col		(H-12)

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagRL )
	mergeCandList[ i++ ] = RL
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagRLH )
	mergeCandList[ i++ ] = RLH
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(H-13)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

- The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numMergeCand and numCurrMergeCand as inputs and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added into mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and the numCurrMergeCand as the inputs and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, and the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numCurrMergeCand − numOrigMergeCand − numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xOrigP][ yOrigP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xOrigP ][ yOrigP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(H-14)

mvLX[ 1 ] = mvLXN[ 1 ]	(H-15)

refIdxLX = refIdxLXN	(H-16)

predFlagLX = predFlagLXN	(H-17)

- When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12, the following applies.

refIdxL1 = −1		(H-18)

predFlagL1 = 0		(H-19)

- H.8.5.3.2.2 Derivation process for spatial merging candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

- a variables availableFlagRL, predFlagLXRL, mvLXRL and refIdxLXRL

Outputs of this process are as follows, with X being 0 or 1

- the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, and availableFlagB2 of the neighbouring prediction units,

- the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, and refIdxLXB2 of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1, and predFlagLXB2 of the neighbouring prediction units,

- the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1, and mvLXB2 of the neighbouring prediction units.

For the derivation of availableFlagA1, refIdxLXA1, predFlagLXA1, and mvLXA1 the following applies:

–	The luma location ( xNbA1, yNbA1 ) inside the neighbouring luma coding block is set equal to ( xP − 1,  yP + nPbH − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xNbA1, yNbA1 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableA1.

–	When one or more of the following conditions are true, availableA1 is set equal to FALSE

- xPb  >>  Log2ParMrgLevel is equal to xNbA1  >>  Log2ParMrgLevel and  yPb  >>  Log2ParMrgLevel is equal to yNbA1  >>  Log2ParMrgLevel.

- PartMode of the current prediction unit is equal to PART_Nx2N, PART_nLx2N or PART_nRx2N, and partIdx is equal to 1.

- if PredFlagLX[ xA1 ][ yA1 ] is equal to 1 and MvLX[ xA1 ][ yA1 ] is equal to mvLXRL and RefIdxLX[ xA1 ][ yA1 ] is equal to refIdxLXRL for X being (0..1)

–	The variables availableFlagA1, refIdxLXA1, predFlagLXA1, and mvLXA1 are derived as follows:

- If availableA1 is equal to FALSE, availableFlagA1 is set equal to 0, both components of mvLXA1 are set equal to 0, refIdxLXA1 is set equal to −1 and predFlagLXA1 is set equal to 0, with X being 0 or 1.

- Otherwise, availableFlagA1 is set equal to 1 and the following assignments are made:

mvLXA1 = MvLX[ xNbA1 ][ yNbA1 ]		(H-20)

refIdxLXA1 = RefIdxLX[ xNbA1 ][ yNbA1 ]		(H-21)

predFlagLXA1 = PredFlagLX[ xNbA1 ][ yNbA1 ]	(H-22)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXA1, mvLXA1 is equal to mvLXRL and refIdxLXA1 is equal to refIdxLXRL for X being (0..1) availableFlagA1 is set to 0.

For the derivation of availableFlagB1, refIdxLXB1, predFlagLXB1, and mvLXB1 the following applies:

–	The luma location ( xNbB1, yNbB1 ) inside the neighbouring luma coding block is set equal to ( xPb + nPbW − 1,  yPb − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xNbB1, yNbB1 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableB1.

–	When one or more of the following conditions are true, availableB1 is set equal to FALSE

- xPb  >>  Log2ParMrgLevel is equal to xNbB1  >>  Log2ParMrgLevel and yPb  >>  Log2ParMrgLevel is equal to yNbB1  >>  Log2ParMrgLevel.

- PartMode of the current prediction unit is equal to PART_2NxN, PART_2NxnU or PART_2NxnD, and partIdx is equal to 1.

–	The variables availableFlagB1, refIdxLXB1, predFlagLXB1, and mvLXB1 are derived as follows.

- If one or more of the following conditions are true, availableFlagB1 is set equal to 0, both components of mvLXB1 are set equal to 0, refIdxLXB1 is set equal to −1 and predFlagLXB1 is set equal to 0, with X being 0 or 1:

    - availableB1 is equal to FALSE

    - availableA1 is equal to TRUE and the prediction units covering the luma locations ( xNbA1, yNbA1 ) and ( xNbB1, yNbB1 ) have the same motion vectors and the same reference indices.

- Otherwise, availableFlagB1 is set equal to 1 and the following assignments are made.

mvLXB1 = MvLX[ xNbB1 ][ yNbB1 ]		(H-23)

refIdxLXB1 = RefIdxLX[ xNbB1 ][ yNbB1 ]		(H-24)

predFlagLXB1 = PredFlagLX[ xNbB1 ][ yNbB1 ]		(H-25)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXB1, mvLXB1 is equal to mvLXRL and refIdxLXB1 is equal to refIdxLXRL for X being (0..1) availableFlagB1 is set to 0.

For the derivation of availableFlagB0, refIdxLXB0, predFlagLXB0, and mvLXB0 the following applies.

–	The luma location ( xNbB0, yNbB0 ) inside the neighbouring luma coding block is set equal to ( xPb + nPbW,  yPb − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xNbB0, yNbB0 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableB0.

–	When xPb  >>  Log2ParMrgLevel is equal to xNbB0  >>  Log2ParMrgLevel and yPb  >>  Log2ParMrgLevel is equal to yNbB0  >>  Log2ParMrgLevel,  availableB0 is set equal to FALSE.

–	The variables availableFlagB0, refIdxLXB0, predFlagLXB0, and mvLXB0 are derived as follows.

- If one or more of the following conditions are true, availableFlagB0 is set equal to 0, both components of mvLXB0 are set equal to 0, refIdxLXB0 is set equal to −1 and predFlagLXB0 is set equal to 0, with X being 0 or 1:

    - availableB0 is equal to FALSE.

    - availableB1 is equal to TRUE and the prediction units covering the luma locations ( xNbB1, yNbB1 ) and ( xNbB0, yNbB0 ) have the same motion vectors and the same reference indices.

    - availableFlagRL + availableFlagA1 + availableFlagBa + availableFlagRLH is equal to or larger than 4.

- Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made:

mvLXB0 = MvLX[ xNbB0 ][ yNbB0 ]		(H-26)

refIdxLXB0 = RefIdxLX[ xNbB0 ][ yNbB0 ]		(H-27)

predFlagLXB0 = PredFlagLX[ xNbB0 ][ yNbB0 ]		(H-28)

For the derivation of availableFlagA0, refIdxLXA0, predFlagLXA0, and mvLXA0 the following applies:

–	The luma location ( xNbA0, yNbA0 ) inside the neighbouring luma coding block is set equal to ( xPb − 1,  yPb + nPbH ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma location ( xPb, yPb ), the luma prediction block width nPbW and the luma prediction block height nPbH, the luma location ( xNbA0, yNbA0 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableA0.

–	When xPb  >>  Log2ParMrgLevel is equal to xNbA0  >>  Log2ParMrgLevel and yPb  >>  Log2ParMrgLevel is equal to yA0 >> >>  Log2ParMrgLevel, availableA0 is set equal to FALSE.

–	The variables availableFlagA0, refIdxLXA0, predFlagLXA0, and mvLXA0 are derived as follows:

- If one or more of the following conditions are true, availableFlagA0 is set equal to 0, both components of mvLXA0 are set equal to 0, refIdxLXA0 is set equal to −1 and predFlagLXA0 is set equal to 0, with X being 0 or 1:

    - availableA0 is equal to FALSE.

    - availableA1 is equal to TRUE and the prediction units covering the luma locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) have the same motion vectors and the same reference indices.

    - availableFlagRL + availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagRLH is equal to or larger than 4.

- Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made:

mvLXA0 = MvLX[ xNbA0 ][ yNbA0 ]		(H-29)

refIdxLXA0 = RefIdxLX[ xNbA0 ][ yNbA0 ]		(H-30)

predFlagLXA0 = PredFlagLX[ xNbA0 ][ yNbA0 ]	(H-31)

For the derivation of availableFlagB2, refIdxLXB2, predFlagLXB2, and mvLXB2 the following applies:

–	The luma location ( xNbB2, yNbB2 ) inside the neighbouring luma coding block is set equal to ( xPb − 1, yPb − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xNbB2, yNbB2 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableB2.

–	When xPb >>  Log2ParMrgLevel is equal to xNbB2  >>  Log2ParMrgLevel and yPb  >>  Log2ParMrgLevel is equal to yNbB2  >>  Log2ParMrgLevel, availableB2 is set equal to FALSE.

–	The variables availableFlagB2, refIdxLXB2, predFlagLXB2, and mvLXB2 are derived as follows.

- If one or more of the following conditions are true, availableFlagB2 is set equal to 0, both components of mvLXB2 are set equal to 0, refIdxLXB2 is set equal to −1 and predFlagLXB2 is set equal to 0, with X being 0 or 1:

    - availableB2 is equal to FALSE.

    - availableA1 is equal to TRUE and prediction units covering luma locations ( xNbA1, yNbA1 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices.

    - availableB1 is equal to TRUE and the prediction units covering luma locations ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices.

    - availableFlagRL + availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 + availableFlagRLH is equal to or larger than 4.

- Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made.

mvLXB2 = MvLX[ xNbB2 ][ yNbB2 ]		(H-32)

refIdxLXB2 = RefIdxLX[ xNbB2 ][ yNbB2 ]		(H-33)

predFlagLXB2 = PredFlagLX[ xNbB2 ][ yNbB2 ]		(H-34)

- H.8.5.3.2.10 Derivation process for co-located reference layer merging candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are as follows, with X being 0 or 1:

- the availability flags availableFlagRL of the co-located prediction units,

- the reference indices refIdxLXRL of the co-located prediction units,

- the prediction list utilization flags predFlagLXRL of the co-located prediction units,

- the motion vectors mvLXRL of the co-located prediction units.

- the availability flags availableFlagRLH of the bottom-right prediction unit of the co-located prediction units,

- the reference indices refIdxLXRLH of the bottom-right prediction unit of the co-located prediction units,

- the prediction list utilization flags predFlagLXRLH of the bottom-right prediction unit of the co-located prediction units,

- the motion vectors mvLXRLH of the bottom-right prediction unit of the co-located prediction units.

For the derivation of availableFlagRL, refIdxLXRL, predFlagLXRL, and mvLXRL the following applies.

- The availability derivation process for a prediction block as specified in subclause  is invoked with the luma location ( xPb, yPb ), the luma prediction block width nPbW and the luma prediction block height nPbH as inputs and the output is assigned to the prediction block availability flag availableFlagRL, refIdxLXRL, and mvLXRL.

- The validity check is performed for derived refIdxLXRL from the co-located predicted unit of the reference layer and the following applies.

  - If refIdxLXRL is greater than num_ref_idx_lX_active_minus1 the AvaillableFlagRL is set to 0 for X being  0 and1.

- The variables availableFlagRL and predFlagLXRL, with X being 0 or 1,  are derived as specified below.

availableFlagRL = availableFlagL0RL | | availableFlagL1RL	(H-35)

predFlagLXRL = availableFlagLXRL		(H-36)

For the derivation of availableFlagRLH, refIdxLXRLH, predFlagLXRLH, and mvLXRLH the following applies:

- The availability derivation process for a prediction block as specified in subclause  is invoked with the luma location ( xPb, yPb ), the luma prediction block width nPbW and the luma prediction block height nPbH as inputs and the output is assigned to the prediction block availability flag availableFlagRLH, refIdxLXRLH, and mvLXRLH.

- The validity check is performed for derived refIdxLXRL from the bottom-right predicted unit of the co-located predicted unit of the reference layer and the following applies.

  - If refIdxLXRLH is greater than num_ref_idx_lX_active_minus1, the availlableFlagRLH is set to 0 for X being  0 and1.

  - If availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXRLH, mvLXRLH is equal to mvLXRL and refIdxLXRLH is equal to refIdxLXRL for X being (0..1) availableFlagRLH is set to 0.

  - predFlagLXRLH = availableFlagLXRLH

- H.8.5.3.2.11 Derivation process for inter-layer luma motion vector prediction

Inputs to this process are:

- a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block ,

Outputs of this process are as follows, with X being 0 or 1:

- the availability flag availableFlagLXRL,

- the reference index refIdxLXRL,

- the motion vector prediction mvLXRL.

- the availability flag availableFlagLXRLH,

- the reference index refIdxLXRLH,

- the motion vector prediction mvLXRLH.

The variable currPb specifies the current luma prediction block at luma location ( xPb, yPb ).

The variables mvLXRLand availableFlagLXRL with X being 0 or 1, are derived as follows:

- The center location (xPCtr, yPCtr) of the luma prediction block is derived as follows

xPCtr = xPb + nPbW / 2		(H-37)

yPCtr = yPb + nPbH / 2		(H-38)

- The reference layer motion vector is derived as follows:

- The reference layer sample location derivation process as specified in subclause G.6.1is invoked with  xPCtr, yPCtr as inputs and the outputs being assigned to  xPCtrRL and yPCtrRL ;

- The variable blPb specifies the luma prediction block covering the modified location given by ( ( xPCtrRL >> 4 ) << 4, ( yPCtrRL >> 4 ) << 4 ) inside the reference layer picture specified by RlPic.

- The luma location ( xPRL, yPRL ) is set equal to the top-left sample of the collocated luma prediction block specified by blPb relative to the top-left luma sample of the reference layer picture specified by RlPic.

- The arrays predFlagLXRL[ x ][ y ], mvLXRL[ x ][ y ] and refIdxLXRL[ x ][ y ] are set equal to the corresponding arrays of the reference layer picture specified by RlPic, PredFlagLX[ x ][ y ], mvLX[ x ][ y ] and RefIdxLX[ x ][ y ], respectively with X being the value of X this process is invoked for. The variables mvLXRL, refIdxLXRL and availableFlagLXRL are derived as follows

        - If blPb is coded in an intra prediction mode, both components of mvLXRL are set equal to 0, availableFlagLXRL is set equal to 0, and refIdxLXRL is set equal to -1;

          - Otherwise, If predFlagLXRL[ xPRL ][ yPRL ] is equal to 0, both components of mvLXRL are set equal to 0, and refIdxLXRL is set equal to -1, and availableFlagLXRL is set equal to 0.

          - Ohterwise, refIdxLXRL is set equal to  RefIdxLXRL [ xPRL ][ yPRL ], and availableFlagLXRL is set equal to 1. The two components of mvLXRL are derived as follows

mvLXRL [0] =  (mvLXRL[ xPRL ][ yPRL ][0]* ScaledW+ offset[0])/ PicWRL

mvLXRL [1] =  (mvLXRL[ xPRL ][ yPRL ][1]* ScaledH+ offset[1]) / PicHRL

          - where

            - offset[0] is set to (PicWRL/2-1) if mvLXRL[ xPRL ][ yPRL ][0] >0, and (1- PicWRL/2) otherwise;

            - offset[1] is set to (PicHRL/2-1) if mvLXRL[ xPRL ][ yPRL ][1] >0, and  (1- PicHRL/2) otherwise.

The variables mvLXRLH and availableFlagLXRLH with X being 0 or 1, are derived as follows:

- The bottom-right location (xPBr, yPBr) of the luma prediction block is derived as follows

xPBr = xPb + nPbW		(H-41)

yPBr = yPb + nPbH 		(H-39)

- The reference layer motion vector is derived as follows:

- The reference layer sample location derivation process as specified in subclause G.6.1is invoked with  xPBr, yPBr as inputs and the outputs being assigned to xPBrRL and yPBrRL ;

- The variable blBrPb specifies the luma prediction block covering the modified location given by ( ( xPBrRL >> 4 ) << 4, ( yPBrRL >> 4 ) << 4 ) inside the reference layer picture specified by RlPic.

- The luma location ( xPRLH, yPRLH ) is set equal to the top-left sample of the collocated luma prediction block specified by blBrPb relative to the top-left luma sample of the reference layer picture specified by RlPic.

- The arrays predFlagLXRLH[ x ][ y ], mvLXRLH[ x ][ y ] and refIdxLXRLH[ x ][ y ] are set equal to the corresponding arrays of the reference layer picture specified by RlPic, PredFlagLX[ x ][ y ], mvLX[ x ][ y ] and RefIdxLX[ x ][ y ], respectively with X being the value of X this process is invoked for. The variables mvLXRLH, refIdxLXRLH and availableFlagLXRLH are derived as follows

        - If blBrPb is coded in an intra prediction mode, both components of mvLXRLH are set equal to 0, availableFlagLXRLH is set equal to 0, and refIdxLXRLH is set equal to -1;

          - Otherwise, If predFlagLXRLH[ xPRL ][ yPRL ] is equal to 0, both components of mvLXRLH are set equal to 0, and refIdxLXRLH is set equal to -1, and availableFlagLXRLH is set equal to 0.

          - Ohterwise, refIdxLXRLH is set equal to  RefIdxLXRLH [ xPRLH ][ yPRLH ], and availableFlagLXRLH is set equal to 1. The two components of mvLXRLH are derived as follows

mvLXRLH [0] =  (mvLXRLH[ xPRLH ][ yPRLH ][0]* ScaledW+ offset[0])/ PicWRL

mvLXRLH [1] =  (mvLXRLH[ xPRLH ][ yPRLH ][1]* ScaledH+ offset[1]) / PicHRL

          - where

            - offset[0] is set to (PicWRL/2-1) if mvLXRLH[ xPRLH ][ yPRLH ][0] >0, and (1- PicWRL/2) otherwise;

            - offset[1] is set to (PicHRLH/2-1) if mvLXRLH[ xPRLH ][ yPRLH ][1] >0, and  (1- PicHRL/2) otherwise.