| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0124 |
| --- | --- |

| Title: | SCE5.1.5: Inter-layer motion information prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, the scaled center and the bottom-right motion vectors (MVs) of the corresponding block in base layer (BL) are inserted in the 1st and the 4th positions of the enhancement layer (EL) merging candidate list. In the pruning process, only the left, above, and BL bottom-right candidates are checked to see if they are the same as with BL center candidate for redundancy removal. The number of redundancy checks with BL center candidate is reduced from 5 to 3. The total number of merging candidates is 5, which is unchanged. Simulation results reportedly show 0.6%, 0.2%, 0.6%, 0.4%, 0.2%, and 0.5% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively, compared with SHM-1.0 IntraBL mode anchors. The encoding time and the decoding time are roughly unchanged. It is also shown that the average additional gain of the proposed SCE5.1.5 on top of SCE2.1.2 or SCE5.1.1 is around 0.4%.

# Introduction

In scalable video coding, the scaled base layer (BL) motion vectors (MVs) can be used for enhancement layer (EL) coding. In JCTVC-L0070 [1], the scaled BL MVs of the corresponding block are put into the merging candidate list to improve the EL motion vector coding. These techniques were decided to be investigated on top of SHM-1.0IntraBL mode as a part of Scalable Extensions Core Experiment 5 (SCE5). The rest of this contribution is organized as follows. The proposed algorithm will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed methods

## Inter-layer merging candidate list

In this contribution, two corresponding positions, BL center (BL-C) and BL bottom-right (BL-H), are used for deriving merging candidates, as shown in Figure 1. The BL-C candidate is placed in the first position of the merging candidate list, and the BL-H is placed in the 4th position of the merging candidate list, which is after the B1 candidate. The number of merging candidates is still kept as 5. If the resolutions of the BL and the EL are different, the motion vectors of BL-C and BL-H are scaled accordingly.

Figure 1.  Illustration of inter-layer merging candidates

## Simplified pruning

To reduce redundancy, in SHM-1.0, any spatial candidate will be removed if it is the same as the BL-C candidate, as shown in Fig.2 (a). Compared with HEVC version 1, 5 extra redundancy checks are required. To reduce the complexity, we proposed to remove 3 redundancy checks in SHM-1.0 as shown in Fig.2 (b). The redundancy checks with BL-C are performed only for left (A1) and above (B1) candidates. For the BL-H candidate, the redundancy check with BL-C is performed. The number of redundancy checks with BL-C candidate is reduced from 5 to 3 in this contribution.

(a)                  (b)                       (c)

Figure 2.  Redundancy checks; (a) SHM-1.0; (b) proposed simplified pruning; (c) proposed simplified pruning with BL-H

## Combining SCE5.1.5 with SCE2.1.2 or SCE5.1.1

In the combined test with SCE2.1.2 [2], the BL-H candidate is placed in the 4th position in the merging candidate list. The redundancy checks with BL-C are performed only for left, above, and BL-H candidates.

In the combined test with SCE5.1.1 [3], the BL-H candidate is placed in the 2nd position in the merging candidate list. The redundancy checks with BL-C are performed only for BL-H candidates.

# Simulation results

## Results of the proposed methods

Experiments were conducted according to the SCE5 description defined in JCTVC-L1105 [4], and the software is based on SHM-1.0 IntraBL mode. As shown in Table 1, the proposed inter-layer motion information prediction in SCE5.1.5 can achieve 0.6%, 0.2%, 0.6%, 0.4%, 0.2%, and 0.5% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively. The encoding time and the decoding time are roughly unchanged. The results of the proposed simplified pruning in SHM-1.0, as shown in Fig.2 (b), are shown in Table 2. No BD-rate change is observed. The encoding time and the decoding time are roughly unchanged.

## Results of the supplementary tests

The results of combining SCE5.1.5 with SCE2.1.2 are shown in Table 3. Simulation results reportedly show 0.8%, 0.4%, 0.8%, 0.8%, 0.5%, and 0.8% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively. The results of combining SCE5.1.5 with SCE5.1.1 are shown in Table 4. The BD-rates are -0.3%, 0.0%, -0.3%, 0.0%, 0.1%, and -0.2% for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively. The summarized results are shown in Table 5. From Table 5, it shows that the average additional gain of the proposed method in SCE5.1.5 on top of SCE 2.1.2 or SCE 5.1.1 is around 0.4%.

# Conclusions

In SCE5.1.5, the BL-C and BL-H MVs are inserted in the 1st and the 4th positions of the EL merging candidate list. The redundancy checks with BL-C are performed only for left, above, and BL-H candidates. The number of redundancy checks with BL-C candidate is reduced from 5 to 3. Simulation results reportedly show 0.4% BD-rate savings on average. The combined results with SCE2.1.2 or SCE5.1.1 are also provided. The average additional gain of the proposed SCE5.1.5 on top of SCE2.1.2 and SCE5.1.1 is also around 0.4%.

# References

- T.-D. Chuang, Y.-W. Huang, S. Lei, “TE5-5.2.1: Inter-layer motion vector prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0070, January, 2013.

- D.-K. Kwon, M. Budagavi, “SCE2: Results of test 1.2 and test 1.3 on inter-layer texture prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0217, April, 2013.

- V. Seregin, J. Chen, M. Karczewicz, “SCE5: Simplified base layer MV candidate for merge mode (test 5.1.1),” Document of Joint Collaborative Team on Video Coding, JCTVC-M0295, April, 2013.

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, “Description of Scalable Extensions Core Experiment 5 (SCE5): Inter-layer syntax prediction using HEVC base layer,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1105, January, 2013.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the proposed SCE5.1.5

Table 2.  Results of the proposed simplified pruning in SHM-1.0

Table 3. Results of combining SCE5.1.5 with SCE2.1.2

Table 4.  Results of combining SCE5.1.5 with SCE5.1.1

Table 5.  Summary of experimental results

|  | RA-2x | RA-1.5x | RA-SNR | LDP-2x | LDP-1.5x | LDP-SNR | Average BD-rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCE2.1.2 | -0.2% | -0.2% | -0.1% | -0.4% | -0.4% | -0.3% | -0.3% |
| SCE2.1.2 + SCE5.1.5 | -0.8% | -0.4% | -0.8% | -0.8% | -0.5% | -0.8% | -0.7% |
| SCE5.1.1 | 0.3% | 0.2% | 0.3% | 0.3% | 0.2% | 0.3% | 0.3% |
| SCE5.1.1 + SCE5.1.5 | -0.3% | 0.0% | -0.3% | 0.0% | 0.1% | -0.2% | -0.1% |