| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0126 |
| --- | --- |

| Title: | Spatial substream definition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Paola Sunna Marco Arena Centro Ricerche e Innovazione Tecnologica RAI, corso Giambone 68, 10135 Torino, ITALY | Tel: Email: | +39 011 8103189 paola.sunna@rai.it marco.arena@rai.it |
|  | Giovanni Ballocca Sisvel Technology, Via Castagnole 59, 10060 None Torinese (TO), ITALY | Tel: Email: | +39 011 9904770 giovanni.ballocca@sisveltech.com |
| Source: | RAI, Sisvel Technology |  |  |

_____________________________

# Abstract

The partitioning of pictures into tiles in HEVC provides a new way to define regions of interest (ROIs) within coded pictures. Those regions can be used to compose a clustering of video streams within a picture in a video sequence. Streams associated with one or more tiles can be independently decoded to provide a set of adaptive services on the basis of the receiver capabilities and/or user’s selection, without the need to decode the entire binary stream.

This contribution proposes a modification to the current sintax of HEVC in order to allow the signaling of an independent decodable region with its corresponding profile and level.

# Introduction

Tiles enable independent coding and decoding of portions of the HEVC bitstream.

In [2], [3], [4] and [5] it has already been proposed to use tiles so as to provide independently decodable video substreams from a single bitstream so as to be able to feed decoders having different decoding capabilities (e.g. decoders of different levels).

As shown in Figure 1, a UHD-1 (3840x2160) video might be partitioned in 4 HD (1920x1080) tiles so that a level 5.2 decoder might decode the whole picture, while an HD (level 4.1) decoder might be enabled to decode a substream corresponding to a single tile

Figure 1

This application requires several steps:

- Identification of spatial substreams at the VPS/SPS level

- Association of one or more tiles to each spatial substream

- Association of profile and level to each spatial substream

This contribution proposes a modification to the current sintax in order to implement the above illustrated application.

# Proposed solution

The use-case described in the previous section can be sketched as in Figure 2.

Figure 2

#### To enable it, during the encoding process, it is necessary to set the following syntax element of the Picture Parameter Set RBSP

- tiles_enabled_flag = 1

- entropy_coding_sync_enabled_flag = 0

- loop_filter_across_tiles_enabled_flag = 0

- sample_adaptive_offset_enabled_flag = 0

It is also necessary to force the temporal prediction to not use reference data from other tiles.

# Text modification

- 7.3.2.1	Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_max _layers_minus1 | u(6) |
| vps_max_temporal_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_0xffff_16bits | u(16) |
| profile_tier_level( vps_max_temporal_sub_layers_minus1 ) |  |
| vps_temporal_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_temporal_sub_layer_ordering_info_present_flag? 0 : vps_max_temporal_sub_layers_minus1 ); i <= vps_max_temporal_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering_minus1[i][0] | ue(v) |
| vps_max_num_reorder_pics[i] | ue(v) |
| vps_max_latency_increase_plus1[i] | ue(v) |
| } |  |
| vps_max_layer_id | u(6) |
| vps_num_layer_sets_minus1 | ue(v) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) |  |
| for( j = 0; j <= vps_max_layer_id; j++ ) |  |
| layer_id_included_flag[ i ][ j ] | u(1) |
| vps_timing_info_present_flag | u(1) |
| if( vps_timing_info_present_flag ) { |  |
| vps_num_units_in_tick | u(32) |
| vps_time_scale | u(32) |
| vps_poc_proportional_to_timing_flag | u(1) |
| if( vps_poc_proportional_to_timing_flag ) |  |
| vps_num_ticks_poc_diff_one_minus1 | ue(v) |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_layer_set_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_temporal_sub_layers_minus1 ) |  |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) { |  |
| vps_subspatial_layers_num | ue(v) |
| if(vps_subspatial_layers_num) { |  |
| for( i = 1; i < vps_subspatial_layers_num; i ++ ) { |  |
| vps_subspatial_layer_id_mask[ i ] | ue(v) |
| profile_tier_level( vps_max_temporal_sub_layers_minus1 ) |  |
| } |  |
| vps_subspatial_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_temporal_sub_layer_ordering_info_present_flag? 0 : vps_max_temporal_sub_layers_minus1 ); i <= vps_max_temporal_sub_layers_minus1; i++ ) { |  |
| for( j = ( vps_subspatial_layer_ordering_info_present_flag ? 1 : vps_subspatial_layers_num ); j <= vps_ subspatial_layers_num; j++ ) { |  |
| vps_max_dec_pic_buffering_minus1[i][j] | ue(v) |
| } |  |
| } |  |
| } |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- 7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| sps_max_temporal_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( sps_max_temporal_sub_layers_minus1 ) |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_temporal_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_temporal_sub_layer_ordering_info_present_flag? 0 : sps_max_temporal_sub_layers_minus1 ); i <= sps_max_temporal_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ][ 0 ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| sps_subspatial_layers_num | ue(v) |
| if(sps_subspatial_layers_num) { |  |
| for( i = 1; i < sps_subspatial_layers_num; i ++ ) { |  |
| sps_subspatial_layer_id_mask[ i ] | ue(v) |
| profile_tier_level( sps_max_temporal_sub_layers_minus1 ) |  |
| } |  |
| sps_subspatial_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_temporal_sub_layer_ordering_info_present_flag? 0 : sps_max_temporal_sub_layers_minus1 ); i <= sps_max_temporal_sub_layers_minus1; i++ ) { |  |
| for( j = ( sps_subspatial_layer_ordering_info_present_flag ? 1 : sps_subspatial_layers_num ); j <= sps_ subspatial_layers_num; j++ ) { |  |
| sps_max_dec_pic_buffering_minus1[i][j] | ue(v) |
| } |  |
| } |  |
| } |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- 7.4.2.1	Video parameter set RBSP semantics

[...]

vps_max_temporal_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the bitstream. The value of vps_max_temporal_sub_layers_minus1  shall be in the range of 0 to 6, inclusive.

vps_temporal_id_nesting_flag, when vps_max_temporal_sub_layers_minus1  is greater than 0,  specifies whether inter prediction is additionally restricted for CVSs referring to the VPS. When vps_max_temporal_sub_layers_minus1  is equal to 0, vps_temporal_id_nesting_flag shall be equal to 1.

NOTE 1 – The syntax element vps_temporal_id_nesting_flag is used to indicate that temporal sub-layer up-switching, i.e. switching from decoding of up to any TemporalId tIdN to decoding up to any TemporalId tIdM that is greater than tIdN, is always possible.

vps_reserved_0xffff_16bits shall be equal to 0xFFFF in bitstreams conforming to this version of this Specification. Other values for vps_reserved_0xffff_16bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of vps_reserved_0xffff_16bits.

NOTE 2 – It is anticipated that in future scalable or 3D video coding extensions of this Specification, this this syntax element will specify a byte offset to the next set of fixed-length coded information in the VPS RBSP, within the data currently specified as vps_extension_data_flag syntax elements. The byte offset would then help to locate and access such information in the VPS RBSP without the need for performing entropy decoding.

- vps_temporal_sub_layer_ordering_info_present_flag equal to 1 specifies that vps_max_dec_pic_buffering_minus1[ i ][ j ] vps_max_num_reorder_pics[ i ],and vps_max_latency_increase_plus1[ i ] are present for vps_max_temporal_sub_layers_minus1 + 1 sub-layers, vps_temporal_sub_layer_ordering_info_present_flag equal to 0 specifies that the value of vps_max_dec_pic_buffering_minus1[ vps_max_temporal_sub_layers_minus1 ][ j ] apply for all temporal sub-layer when SubspatialId is equal to j while the values of vps_max_num_reorder_pics[ vps_max_temporal_sub_layers_minus1 ], and vps_max_latency_increase_plus1[ vps_max_temporal_sub_layers_minus1 ] apply to all sub-layers.

- vps_max_dec_pic_buffering_minus1[ i ][ j ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i and SubspatialId is equal to j. The value of vps_max_dec_pic_buffering_minus1[ i ][ j ] shall be in the range of 0 to MaxDpbSize − 1 (as specified in subclause Error: Reference source not found), inclusive. When i is greater than 0, vps_max_dec_pic_buffering_minus1[ i ][ j ] shall be greater than or equal to vps_max_dec_pic_buffering_minus1[ i − 1 ][ j ]. When vps_max_dec_pic_buffering_minus1[ i ][ j ] is not present for i in the range of 0 to vps_max_temporal_sub_layers_minus1  − 1 due to vps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_dec_pic_buffering_minus1[ vps_max_temporal_sub_layers_minus1 ][ j ]. When vps_max_dec_pic_buffering_minus1[ i ][ j ] is not present for j in the range of 0 to vps_subspatial_layers_num_minus1  − 1 due to vps_subspatial_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_dec_pic_buffering_minus1[i][ vps_subspatial_layers_num  ].

- vps_max_num_reorder_pics[ i ] indicates the maximum allowed number of pictures that can precede any picture in the CVS in decoding order and follow that picture in output order when HighestTid is equal to i. The value of vps_max_num_reorder_pics[ i ] shall be in the range of 0 to vps_max_dec_pic_buffering_minus1[ i ], inclusive. When i is greater than 0, vps_max_num_reorder_pics[ i ] shall be greater than or equal to vps_max_num_reorder_pics[ i − 1 ]. When vps_max_num_reorder_pics[ i ] is not present for i in the range of 0 to vps_max_sub_layers_minus1 − 1 due to vps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_num_reorder_pics[ vps_max_sub_layers_minus1 ]. The value of vps_max_num_reorder_pics[i] is inferred to be the same for all subspatial layer when HighestTid is equal to i.

- vps_max_latency_increase_plus1[ i ] not equal to 0 is used to compute the value of VpsMaxLatencyPictures[ i ] as specified by setting VpsMaxLatencyPictures[ i ] to vps_max_num_reorder_pics[ i ] + vps_max_latency_increase_plus1[ i ] − 1. When vps_max_latency_increase_plus1[ i ] is not equal to 0, the value of VpsMaxLatencyPictures[ i ] specifies the maximum number of pictures that can precede any picture in the CVS in output order and follow that picture in decoding order when HighestTid is equal to i. When vps_max_latency_increase_plus1[ i ] is equal to 0, no corresponding limit is expressed. The value of vps_max_latency_increase_plus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. When vps_max_latency_increase_plus1[ i ] is not present for i in the range of 0 to vps_max_temporal_sub_layers_minus1  − 1 due to vps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_latency_increase_minus1[ vps_max_temporal_sub_layers_minus1  ]. The value of vps_max_latency_increase_plus1[i] is inferred to be the same for all subspatial layer when HighestTid is equal to i.

- [...]

vps_subspatial_layers_num specifies the number of subspatial layers that are present in the bitstream. The value of vps_ subspatial_layers_num  shall be in the range of 0 to (PicHeightInCtbsY – 1)*( PicHeightInCtbsX – 1), inclusive.

- vps_subspatial_layer_ordering_info_present_flag equal to 1 specifies that vps_max_dec_pic_buffering_minus1[ i ][ j ] is present for vps_subspatial_layers_num sub-layers, vps__subspatial_layer_ordering_info_present_flag equal to 0 specifies that the value of vps_max_dec_pic_buffering_minus1[ i ][ vps_subspatial_layers_num_minus1 ] apply to all subspatial layers when HighestTid is equal to i..

vps_subspatial_layer_id_mask[ i ] it is a bitmask that indicates the set of tiles representing a subspatial layer. A bit to 1 in the position corresponding to the tile index indicates that .the tile is included in the subspatial layer i.

NOTE X: for example a value of vps_subspatial_layer_id_mask[i] equal to 1100 indicates that the subspatial layer i is composed by tile 0 and tile 1.

- 7.4.2.2	Sequence parameter set RBSP semantics

sps_video_parameter_set_id specifies the value of the vps_video_parameter_set_id of the active VPS.

sps_max_temporal_sub_layers_minus1 plus 1 specifies the number of temporal sub-layers present in each CVS referring to the SPS. The value of sps_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive.

[...]

sps_temporal_sub_layer_ordering_info_present_flag equal to 1 specifies that sps_max_dec_pic_buffering_minus1[ i ][ j ], sps_max_num_reorder_pics[ i ], and sps_max_latency_increase_plus1[ i ] are present for sps_max_sub_layers_minus1 + 1 sub-layers, sps_sub_layer_ordering_info_present_flag equal to 0 specifies that the value of sps_max_dec_pic_buffering_minus1[ sps_max_temporal_sub_layers_minus1 ][ j ] apply for all temporal sub-layer when SubspatialId is equal to j while sps_max_num_reorder_pics[ sps_max_temporal_sub_layers_minus1 ], and sps_max_latency_increase_plus1[ sps_max_temporal_sub_layers_minus1 ] apply to all temporal sub-layers.

- sps_max_dec_pic_buffering_minus1[ i ][ j ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i. The value of sps_max_dec_pic_buffering_minus1[ i ][ j ] shall be in the range of 0 to MaxDpbSize − 1 (as specified in subclause Error: Reference source not found), inclusive. When i is greater than 0, sps_max_dec_pic_buffering_minus1[ i ][ j ] shall be greater than or equal to sps_max_dec_pic_buffering_minus1[ i − 1 ][ j ]. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be less than or equal to vps_max_dec_pic_buffering_minus1[ i ] for each value of i. When sps_max_dec_pic_buffering_minus1[ i ][ j ] is not present for i in the range of 0 to sps_max_temporal_sub_layers_minus1  − 1 due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_dec_pic_buffering_minus1[ sps_max_temporal_sub_layers_minus1 ][ j ].  When sps_max_dec_pic_buffering_minus1[ i ][ j ]  is not present for j in the range of 0 to sps_subspatial_layers_num_minus1  − 1 due to sps_subspatial_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_dec_pic_buffering_minus1[i][ sps_subspatial_layers_num  ].

- sps_max_num_reorder_pics[ i ] indicates the maximum allowed number of pictures that can precede any picture in the CVS in decoding order and follow that picture in output order when HighestTid is equal to i. The value of sps_max_num_reorder_pics[ i ] shall be in the range of 0 to sps_max_dec_pic_buffering_minus1[ i ], inclusive. When i is greater than 0, sps_max_num_reorder_pics[ i ] shall be greater than or equal to sps_max_num_reorder_pics[ i − 1 ]. The value of sps_max_num_reorder_pics[ i ] shall be less than or equal to vps_max_num_reorder_pics[ i ] for each value of i. When sps_max_num_reorder_pics[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1 due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_num_reorder_pics[ sps_max_sub_layers_minus1 ]. The value of sps_max_num_reorder_pics[i] is inferred to be the same for all subspatial layer when HighestTid is equal to i.

sps_max_latency_increase_plus1[ i ] not equal to 0 is used to compute the value of SpsMaxLatencyPictures[ i ] as specified by setting SpsMaxLatencyPictures[ i ] equal to sps_max_num_reorder_pics[ i ] + sps_max_latency_increase_plus1[ i ] − 1. When sps_max_latency_increase_plus1[ i ] is not equal to 0, the value of SpsMaxLatencyPictures[ i ] specifies the maximum number of pictures that can precede any picture in the CVS in output order and follow that picture in decoding order when HighestTid is equal to i. When sps_max_latency_increase_plus1[ i ] is equal to 0, no corresponding limit is expressed. The value of sps_max_latency_increase_plus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. When vps_max_latency_increase_plus1[ i ] is not equal to 0, the value of sps_max_latency_increase_plus1[ i ] shall not be equal to 0 and shall be less than or equal to vps_max_latency_increase_plus1[ i ] for each value of i. When sps_max_latency_increase_plus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1 due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_latency_increase_plus1[ sps_max_sub_layers_minus1 ]. The value of sps_max_latency_increase_plus1[i] is inferred to be the same for all subspatial layer when HighestTid is equal to i.

[...]

sps_subspatial_layers_num specifies the number of subspatial layers that are present in the bitstream. The value of sps_ subspatial_layers_num  shall be in the range of 0 to (PicHeightInCtbsY – 1)*( PicHeightInCtbsX – 1), inclusive.

- sps_subspatial_layer_ordering_info_present_flag equal to 1 specifies that sps_max_dec_pic_buffering_minus1[ i ][ j ] is present for sps_subspatial_layers_num sub-layers, sps__subspatial_layer_ordering_info_present_flag equal to 0 specifies that the value of sps_max_dec_pic_buffering_minus1[ i ][ sps_subspatial_layers_num_minus1 ] apply to all subspatial layers when HighestTid is equal to i.

sps_subspatial_layer_id_mask[ i ] it is a bitmask that indicates the set of tiles representing a subspatial layer. A bit to 1 in the position corresponding to the tile index indicates that .the tile is included in the subspatial layer i.

NOTE X: for example a value of sps_subspatial_layer_id_mask[i] equal to 1100 indicates that the subspatial layer i is composed by tile 0 and tile 1.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand, JCTVC-L1003, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent)”, Geneva, CH, Jan. 2013.

- X. Yang, P. Yue, Y. Zhang, JCTVC-K0116, “2D compatible frame packing stereo 3D video”, Shanghai, CN, Oct. 2012.

- Y. Ye, Y. He, Y. He, M. Horowitz, JCTVC-K0248, “ROI tile sections”, Shanghai, CN, Oct. 2012.

- R. Skupin, V. George, T. Schier, JCTVC-K0218, “Tile-based region-of-interest signalling with sub-picture SEI messages”, Shanghai, CN, Oct. 2012.

- Y. Ye, Y. He, Y. He, X. Yang, P. Yue, Y. Zhang, M. Horowitz, JCTVC L0049, “SEI message: independently decodable regions based on tiles”, Geneva, CH, Jan. 2013.

# Patent rights declaration(s)

RAI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sisvel Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).