- Working Draft of Proposal 1A

##### 8.4.4.2.6	Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nT − 1.

The variable predModeIntraAdj is derived as follows:

- When ChromaArrayType is equal to 2 and cIdx is not equal to 0, predModeIntraAdj is derived using the mapping table Table 8-x.

- Otherwise (ChromaArrayType is not equal to 2 or cIdx is equal to 0), predModeIntraAdj is set equal to predModeIntra.

Table 8-x Specification of predModeIntraAdj for 4:2:2 chroma

| predModeIntra | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| predModeIntraAdj for 4:2:2 | - | - | 2 | 2 | 2 | 2 | 3 | 5 | 7 | 8 | 10 | 12 | 13 | 15 | 17 | 18 | 19 | 20 |
| predModeIntra |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| predModeIntraAdj for 4:2:2 |  | 21 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 31 |

Figure 8-2 illustrates the total 33 intra angles and Table 8-4 specifies the mapping table between predModeIntraAdj and the angle parameter intraPredAngle.

Figure 8-1 – Intra prediction angle definition (informative)

Table 8-1 – Specification of intraPredAngle

| predModeIntraAdj | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 | −5 | −9 | −13 | −17 | −21 | −26 |
| predModeIntraAdj | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngle | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Table 8-5 further specifies the mapping table between predModeIntraAdj and the inverse angle parameter invAngle.

Table 8-2 – Specification of invAngle

| predModeIntraAdj | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | −4096 | −1638 | −910 | −630 | −482 | −390 | −315 | −256 |
| predModeIntraAdj | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| invAngle | −315 | −390 | −482 | −630 | −910 | −1638 | −4096 | - |

The variables intraPredAngleAdj and invAngleAdj are derived as follows:

–	When ChromaArrayType is equal to 2 and cIdx is not equal to 0, the following applies:

- If intraPredMode is greater than 17, intraPredAngleAdj is set equal to intraPredAngle  >>  1) and invAngleAdj is set equal to invAngle  <<  1,

- Otherwise (intraPredMode is less than 18), the variables intraPredAngleAdj and invAngleAdj are derived as follows:

  - If Abs( intraPredAngle ) is greater than 16, intraPredAngleAdj is set equal to Sign( intraPredAngle ) * 32 and invAngleAdj is set equal to −256,

  - Otherwise (Abs( intraPredAngle ) is less than 17), intraPredAngleAdj is  set equal to intraPredAngle  <<  1 and when nT * intraPredAngle  >>  5 is less than −1, invAngleAdj is set equal to invAngle  >>  1.

–	Otherwise (ChromaArrayType is not equal to 2 or cIdx is equal to 0), intraPredAngleAdj is set equal to intraPredAngle and when nT * intraPredAngle  >>  5 is less than −1, invAngleAdj is set equal to invAngle.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

–	If predModeIntraAdj is equal or greater than 18, the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT, is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nT		(8-1)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngleAdj + 128 )  >>  8 ) ],
	with x=( nT * intraPredAngleAdj )  >>  5..−1	(8-2)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nT + 1..2 * nT	(8-3)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngleAdj )  >>  5	(8-4)

iFact = ( ( y + 1 ) * intraPredAngleAdj ) & 31	(8-5)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-6)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-7)

  - When predModeIntraAdj is equal to 26 (vertical), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0, y = 0..nT − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-8)

–	Otherwise (predModeIntraAdj is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nT		(8-9)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],
	with x = ( nT * intraPredAngle )  >>  5..−1	(8-10)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nT + 1..2 * nT	(8-11)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8-12)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8-13)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-14)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-15)

- When predModeIntraAdj is equal to 10 (horizontal), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0..nT − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )

- Working Draft of Proposal 1B

### 8.4.3	Derivation process for chroma intra prediction mode

[Ed.: (WJ) this subclause may be moved to the semantics of intra_chroma_pred_mode syntax]

This process is only invoked when ChromaArrayType is not equal to 0.

Input to this process is a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture.

Output of this process is the variable IntraPredModeC.

The variable IntraPredModeC' is derived using intra_chroma_pred_mode[ xB ][ yB ] and IntraPredModeY[ xB ][ yB ] as specified in Table 8-3.

Table 8-3 – Specification of IntraPredModeC'

| intra_chroma_pred_mode[ xB ][ yB ] | IntraPredModeY[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 26 | 10 | 1 | X ( 0 <= X <= 34 ) |
| 0 | 34 | 0 | 0 | 0 | 0 |
| 1 | 26 | 34 | 26 | 26 | 26 |
| 2 | 10 | 10 | 34 | 10 | 10 |
| 3 | 1 | 1 | 1 | 34 | 1 |
| 4 | 0 | 26 | 10 | 1 | X |

The chroma intra prediction mode IntraPredModeC is derived as follows:

- When ChromaArrayType is equal to 2, IntraPredModeC is derived using the mapping table Table 8-3.

- Otherwise (ChromaArrayType is not equal to 2), IntraPredModeC is set equal to IntraPredModeC'.

Table 8-3 Specification of IntraPredModeC for 4:2:2 chroma format

| IntraPredModeC' | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraPredModeC for 4:2:2 | 0 | 1 | 2 | 2 | 2 | 2 | 3 | 5 | 7 | 8 | 10 | 12 | 13 | 15 | 17 | 18 | 19 | 20 |
| IntraPredModeC' |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| IntraPredModeC for 4:2:2 |  | 21 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 31 |

##### 8.4.4.2.6	Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nT − 1.

Figure 8-2 illustrates the total 33 intra angles and Table 8-4 specifies the mapping table between predModeIntra and the angle parameter intraPredAngle.

Figure 8-2 – Intra prediction angle definition (informative)

Table 8-4 – Specification of intraPredAngle

| predModeIntra | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 | −5 | −9 | −13 | −17 | −21 | −26 |
| predModeIntra | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngle | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Table 8-5 further specifies the mapping table between predModeIntra and the inverse angle parameter invAngle.

Table 8-5 – Specification of invAngle

| predModeIntra | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | −4096 | −1638 | −910 | −630 | −482 | −390 | −315 | −256 |
| predModeIntra | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| invAngle | −315 | −390 | −482 | −630 | −910 | −1638 | −4096 | - |

The variables intraPredAngleAdj and invAngleAdj are derived as follows:

–	When ChromaArrayType is equal to 2 and cIdx is not equal to 0, the following applies:

- If intraPredMode is greater than 17, intraPredAngleAdj is set equal to intraPredAngle  >>  1) and invAngleAdj is set equal to invAngle  <<  1,

- Otherwise (intraPredMode is less than 18), the variables intraPredAngleAdj and invAngleAdj are derived as follows:

  - If Abs( intraPredAngle ) is greater than 16, intraPredAngleAdj is set equal to Sign( intraPredAngle ) * 32 and invAngleAdj is set equal to −256,

  - Otherwise (Abs( intraPredAngle ) is less than 17), intraPredAngleAdj is  set equal to intraPredAngle  <<  1 and when nT * intraPredAngle  >>  5 is less than −1, invAngleAdj is set equal to invAngle  >>  1.

–	Otherwise (ChromaArrayType is not equal to 2 or cIdx is equal to 0), intraPredAngleAdj is set equal to intraPredAngle and when nT * intraPredAngle  >>  5 is less than −1, invAngleAdj is set equal to invAngle.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

–	If predModeIntra is equal or greater than 18, the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT, is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nT		(8-16)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngleAdj + 128 )  >>  8 ) ],
	with x=( nT * intraPredAngleAdj )  >>  5..−1	(8-17)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nT + 1..2 * nT	(8-18)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngleAdj )  >>  5	(8-19)

iFact = ( ( y + 1 ) * intraPredAngleAdj ) & 31	(8-20)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-21)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-22)

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0, y = 0..nT − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-23)

–	Otherwise (predModeIntra is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nT		(8-24)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],
	with x = ( nT * intraPredAngle )  >>  5..−1	(8-25)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nT + 1..2 * nT	(8-26)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8-27)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8-28)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-29)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-30)

- When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0..nT − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )

- Working Draft of Proposal 2A

##### 8.4.4.2.6	Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nT − 1.

The variable predModeIntraAdj is derived as follows:

- When ChromaArrayType is equal to 2 and cIdx is not equal to 0, predModeIntraAdj is derived using the mapping table Table 8-x.

- Otherwise (ChromaArrayType is not equal to 2 or cIdx is equal to 0), predModeIntraAdj is set equal to predModeIntra.

Table 8-x Specification of predModeIntraAdj for 4:2:2 chroma

| predModeIntra | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| predModeIntraAdj for 4:2:2 | - | - | 2 | 2 | 2 | 2 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 18 | 18 | 18 |
| predModeIntra |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| predModeIntraAdj for 4:2:2 |  | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |

Figure 8-2 illustrates the total 33 intra angles and Table 8-4 specifies the mapping table between predModeIntraAdj and the angle parameter intraPredAngle.

Figure 8-3 – Intra prediction angle definition (informative)

Table 8-6 – Specification of intraPredAngle

| predModeIntraAdj | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 | −5 | −9 | −13 | −17 | −21 | −26 |
| predModeIntraAdj | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngle | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Table 8-5 further specifies the mapping table between predModeIntraAdj and the inverse angle parameter invAngle.

Table 8-7 – Specification of invAngle

| predModeIntraAdj | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | −4096 | −1638 | −910 | −630 | −482 | −390 | −315 | −256 |
| predModeIntraAdj | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| invAngle | −315 | −390 | −482 | −630 | −910 | −1638 | −4096 | - |

The variables intraPredAngleAdj and invAngleAdj are derived as follows:

–	When ChromaArrayType is equal to 2 and cIdx is not equal to 0, the following applies:

- If intraPredMode is greater than 17, intraPredAngleAdj is set equal to intraPredAngle  >>  1) and invAngleAdj is set equal to invAngle  <<  1,

- Otherwise (intraPredMode is less than 18), the variables intraPredAngleAdj and invAngleAdj are derived as follows:

  - If Abs( intraPredAngle ) is greater than 16, intraPredAngleAdj is set equal to Sign( intraPredAngle ) * 32 and invAngleAdj is set equal to −256,

  - Otherwise (Abs( intraPredAngle ) is less than 17), intraPredAngleAdj is  set equal to intraPredAngle  <<  1 and when nT * intraPredAngle  >>  5 is less than −1, invAngleAdj is set equal to invAngle  >>  1.

–	Otherwise (ChromaArrayType is not equal to 2 or cIdx is equal to 0), intraPredAngleAdj is set equal to intraPredAngle and when nT * intraPredAngle  >>  5 is less than −1, invAngleAdj is set equal to invAngle.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

–	If predModeIntraAdj is equal or greater than 18, the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT, is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nT		(8-31)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngleAdj + 128 )  >>  8 ) ],
	with x=( nT * intraPredAngleAdj )  >>  5..−1	(8-32)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nT + 1..2 * nT	(8-33)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngleAdj )  >>  5	(8-34)

iFact = ( ( y + 1 ) * intraPredAngleAdj ) & 31	(8-35)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-36)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-37)

  - When predModeIntraAdj is equal to 26 (vertical), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0, y = 0..nT − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-38)

–	Otherwise (predModeIntraAdj is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nT		(8-39)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],
	with x = ( nT * intraPredAngle )  >>  5..−1	(8-40)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nT + 1..2 * nT	(8-41)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8-42)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8-43)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-44)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-45)

- When predModeIntraAdj is equal to 10 (horizontal), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0..nT − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )

- Working Draft of Proposal 2B

### 8.4.3	Derivation process for chroma intra prediction mode

[Ed.: (WJ) this subclause may be moved to the semantics of intra_chroma_pred_mode syntax]

This process is only invoked when ChromaArrayType is not equal to 0.

Input to this process is a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture.

Output of this process is the variable IntraPredModeC.

The variable IntraPredModeC' is derived using intra_chroma_pred_mode[ xB ][ yB ] and IntraPredModeY[ xB ][ yB ] as specified in Table 8-3.

Table 8-8 – Specification of IntraPredModeC'

| intra_chroma_pred_mode[ xB ][ yB ] | IntraPredModeY[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 26 | 10 | 1 | X ( 0 <= X <= 34 ) |
| 0 | 34 | 0 | 0 | 0 | 0 |
| 1 | 26 | 34 | 26 | 26 | 26 |
| 2 | 10 | 10 | 34 | 10 | 10 |
| 3 | 1 | 1 | 1 | 34 | 1 |
| 4 | 0 | 26 | 10 | 1 | X |

The chroma intra prediction mode IntraPredModeC is derived as follows:

- When ChromaArrayType is equal to 2 and cIdx is not equal to 0, IntraPredModeC is derived using the mapping table Table 8-3.

- Otherwise (ChromaArrayType is equal to 2 and cIdx is not equal to 0), IntraPredModeC is set equal to IntraPredModeC'.

Table 8-3 Specification of predModeIntra for 4:2:2 chroma

| IntraPredModeC' | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraPredModeC for 4:2:2 | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 18 | 18 | 18 |
| IntraPredModeC' |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| IntraPredModeC for 4:2:2 |  | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |

##### 8.4.2.2.6	Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

As the same as Proposal 1B with applying to scanning order for residual coding.

- Working Draft of Proposal 3A

##### 8.4.4.2.6	Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nT − 1.

The variable predModeIntraAdj is derived as follows:

- When ChromaArrayType is equal to 2 and cIdx is not equal to 0, predModeIntraAdj is derived using the mapping table Table 8-x.

- Otherwise (ChromaArrayType is not equal to 2 or cIdx is equal to 0), predModeIntraAdj is set equal to predModeIntra.

Table 8-x Specification of predModeIntraAdj for 4:2:2 chroma

| predModeIntra | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| predModeIntraAdj for 4:2:2 | - | - | 2 | 2 | 2 | 2 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 21 | 21 |
| predModeIntra |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| predModeIntraAdj for 4:2:2 |  | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |

Figure 8-2 illustrates the total 33 intra angles and Table 8-4 specifies the mapping table between predModeIntraAdj and the angle parameter intraPredAngle.

Figure 8-4 – Intra prediction angle definition (informative)

Table 8-9 – Specification of intraPredAngle

| predModeIntraAdj | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 | −5 | −9 | −13 | −17 | −21 | −26 |
| predModeIntraAdj | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngle | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Table 8-5 further specifies the mapping table between predModeIntraAdj and the inverse angle parameter invAngle.

Table 8-10 – Specification of invAngle

| predModeIntraAdj | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | −4096 | −1638 | −910 | −630 | −482 | −390 | −315 | −256 |
| predModeIntraAdj | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| invAngle | −315 | −390 | −482 | −630 | −910 | −1638 | −4096 | - |

The variables intraPredAngleAdj and invAngleAdj are derived as follows:

–	When ChromaArrayType is equal to 2 and cIdx is not equal to 0, the following applies:

- If intraPredMode is greater than 17, intraPredAngleAdj is set equal to intraPredAngle  >>  1) and invAngleAdj is set equal to invAngle  <<  1,

- Otherwise (intraPredMode is less than 18), the variables intraPredAngleAdj and invAngleAdj are derived as follows:

  - If Abs( intraPredAngle ) is greater than 16, intraPredAngleAdj is set equal to Sign( intraPredAngle ) * 32 and invAngleAdj is set equal to −256,

  - Otherwise (Abs( intraPredAngle ) is less than 17), intraPredAngleAdj is  set equal to intraPredAngle  <<  1 and when nT * intraPredAngle  >>  5 is less than −1, invAngleAdj is set equal to invAngle  >>  1.

–	Otherwise (ChromaArrayType is not equal to 2 or cIdx is equal to 0), intraPredAngleAdj is set equal to intraPredAngle and when nT * intraPredAngle  >>  5 is less than −1, invAngleAdj is set equal to invAngle.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

–	If predModeIntraAdj is equal or greater than 18, the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT, is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nT		(8-46)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngleAdj + 128 )  >>  8 ) ],
	with x=( nT * intraPredAngleAdj )  >>  5..−1	(8-47)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nT + 1..2 * nT	(8-48)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngleAdj )  >>  5	(8-49)

iFact = ( ( y + 1 ) * intraPredAngleAdj ) & 31	(8-50)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-51)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-52)

  - When predModeIntraAdj is equal to 26 (vertical), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0, y = 0..nT − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-53)

–	Otherwise (predModeIntraAdj is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nT		(8-54)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],
	with x = ( nT * intraPredAngle )  >>  5..−1	(8-55)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nT + 1..2 * nT	(8-56)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8-57)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8-58)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-59)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-60)

- When predModeIntraAdj is equal to 10 (horizontal), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0..nT − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )

- Working Draft of Proposal 3B

### 8.4.3	Derivation process for chroma intra prediction mode

[Ed.: (WJ) this subclause may be moved to the semantics of intra_chroma_pred_mode syntax]

This process is only invoked when ChromaArrayType is not equal to 0.

Input to this process is a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture.

Output of this process is the variable IntraPredModeC.

The variable IntraPredModeC' is derived using intra_chroma_pred_mode[ xB ][ yB ] and IntraPredModeY[ xB ][ yB ] as specified in Table 8-3.

Table 8-11 – Specification of IntraPredModeC'

| intra_chroma_pred_mode[ xB ][ yB ] | IntraPredModeY[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 26 | 10 | 1 | X ( 0 <= X <= 34 ) |
| 0 | 34 | 0 | 0 | 0 | 0 |
| 1 | 26 | 34 | 26 | 26 | 26 |
| 2 | 10 | 10 | 34 | 10 | 10 |
| 3 | 1 | 1 | 1 | 34 | 1 |
| 4 | 0 | 26 | 10 | 1 | X |

The chroma intra prediction mode IntraPredModeC is derived as follows:

- When ChromaArrayType is equal to 2 and cIdx is not equal to 0, IntraPredModeC is derived using the mapping table Table 8-3.

- Otherwise (ChromaArrayType is equal to 2 and cIdx is not equal to 0), IntraPredModeC is set equal to IntraPredModeC'.

Table 8-3 Specification of predModeIntra for 4:2:2 chroma

| IntraPredModeC' | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraPredModeC for 4:2:2 | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 21 | 21 |
| IntraPredModeC' |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| IntraPredModeC for 4:2:2 |  | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |

##### 8.4.2.2.6	Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

As the same as Proposal 1B with applying to scanning order for residual coding.