| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0127 |
| --- | --- |

| Title: | AHG5: Unified intra prediction angles for 4:2:2 chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Motoharu Ueda Shigeru Fukushima Toru Kumakura JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In the current draft of HEVC Range Extensions and the test model HM-10.0-RExt-2.0, some additional intra prediction angles for 4:2:2 chroma are calculated from intra prediction angle for luma and 4:2:0/4:4:4 chroma.

This contribution proposes the unification of chroma intra prediction angles between 4:2:2 chroma and the others without adding any intra prediction angles for 4:2:2 chroma.

# Introduction

In the current draft of HEVC Range Extensions [1] and the test model HM-10.0-RExt-2.0, each intra prediction angle for 4:2:2 chroma is adjusted from intra prediction angle for luma and 4:2:0/4:4:4 chroma. Therefore, some additional intra prediction angles for 4:2:2 chroma are required.

This contribution proposes the unification of chroma intra prediction angles between 4:2:2 chroma and the others without adding any intra prediction angles for 4:2:2 chroma.

# Algorithm description

## Current algorithm (HM-10.0-RExt-2.0)

In the current draft of HEVC Range Extensions [1] and the test model HM-10.0-RExt-2.0, intra prediction angles for luma and 4:2:0/4:4:4 chroma are defined. Figure 1 illustrates 33 intra prediction angles for luma and 4:2:0 chroma.

Figure 1 Intra prediction angle for luma and 4:2:0/4:4:4 chroma

In the HM-10.0-RExt-2.0, intra prediction angles for 4:2:2 chroma are calculated from intra prediction angle for luma and 4:2:0/4:4:4 chroma. Figure 2 illustrates intra prediction angles for 4:2:2 chroma.

Figure 2 Intra prediction angle for 4:2:2 chroma in HM-10.0-RExt-2.0

Table 1 specifies the mapping table between predModeIntra and the angle parameter intraPredAngle for 4:2:2 chroma of the HM-10.0-RExt-2.0.

Table 1 Specification of intraPredAngle for 4:2:2 chroma (HM-10.0-RExt-2.0)

| predModeIntra | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 | −5 | −9 | −13 | −17 | −21 | −26 |
| intraPredAngle for 4:2:2 chroma | - | 32 | 32 | 32 | 32 | 26 | 18 | 10 | 4 | 0 | -4 | -10 | -18 | -26 | -32 | -32 | -32 |
| predModeIntra | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngle | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |
| intraPredAngle for 4:2:2 chroma | -16 | -13 | -11 | -9 | -7 | -5 | -3 | -1 | 0 | 1 | 2 | 4 | 6 | 8 | 10 | 13 | 16 |

17 additional intra prediction angles for 4:2:2 chroma are required. The additional intra prediction angles are not used in the intra prediction for luma and 4:2:0/4:4:4 chroma.

## Proposed algorithm

In this proposal, intra prediction angles for 4:2:2 chroma are unified with intra prediction angles for luma and 4:2:0/4:4:4 chroma. No additional intra prediction angles for 4:2:2 chroma are required.

Each intra prediction mode for 4:2:2 chroma is derived from intra prediction mode for luma and 4:2:0/4:4:4 chroma using a mapping table before the decoding process of intra sample prediction. As a result, only existing intra prediction angles in HEVC version 1 are required as intra prediction angles for 4:2:2 choroma.

Method A and Method B are proposed.

- Method A: The application of the mapped intra prediction mode to intra prediction
In Method A, the mapping process of intra prediction mode is only applied to the decoding process of intra sample prediction, and the mapped intra prediction mode is not applied to the decision process of scanning order for residual coding.

- Method B: The application of the mapped intra prediction mode to intra prediction and MDCS
In Method B, the mapping process of intra prediction mode is applied to the derivation process for chroma intra prediction mode. The mapped intra prediction mode is applied to the decoding process of intra sample prediction. Furthermore, the mapped intra prediction mode is also applied to the decision process of scanning order for residual coding. Decoding process of intra sample prediction is used without changing from HEVC version 1.

The following mapping table of Proposal 1, 2 or 3 is applied to Method A or Method B.

### Proposal 1

In Proposal 1, intra prediction mode for 4:2:2 chroma is derived from intra prediction mode for luma and 4:2:0/4:4:4 chroma as specified in Table 2.

Table 2 Specification of intra prediction mode for 4:2:2 chroma (Proposal 1)

| intra pred mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intra pred mode for 4:2:2 chroma | 0 | 1 | 2 | 2 | 2 | 2 | 3 | 5 | 7 | 8 | 10 | 12 | 13 | 15 | 17 | 18 | 19 | 20 |
| intra pred mode |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intra pred mode for 4:2:2 chroma |  | 21 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 31 |

Figure 3 illustrates the intra prediction angles for 4:2:2 chroma in Proposal 1.

Figure 3 Intra prediction angle for 4:2:2 chroma in Proposal 1

### Proposal 2

In Proposal 2, intra prediction mode for 4:2:2 chroma is derived from intra prediction mode for luma and 4:2:0/4:4:4 chroma as specified in Table 3.

Table 3 Specification of intra prediction mode for 4:2:2 chroma (Proposal 2)

| intra pred mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intra pred mode for 4:2:2 chroma | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 18 | 18 | 18 |
| intra pred mode |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intra pred mode for 4:2:2 chroma |  | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |

The intra prediction modes for 4:2:2 chroma of the mapping table of Proposal 2 (Table 3) are derived as described in Appendix 8.1.

Figure 4 illustrates the intra prediction angles for 4:2:2 chroma in Proposal 2.

Figure 4 Intra prediction angle for 4:2:2 chroma in Proposal 2

### Proposal 3

In Proposal 3, intra prediction mode for 4:2:2 chroma is derived from intra prediction mode for luma and 4:2:0/4:4:4 chroma as specified in Table 4.

Table 4 Specification of intra prediction mode for 4:2:2 chroma (Proposal 3)

| intra pred mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intra pred mode for 4:2:2 chroma | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 21 | 21 |
| intra pred mode |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intra pred mode for 4:2:2 chroma |  | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |

The intra prediction modes for 4:2:2 chroma of the mapping table of Proposal 3 (Table 4) are derived as described in Appendix 8.2.

Figure 5 illustrates the intra prediction angles for 4:2:2 chroma in Proposal 3.

Figure 5 Intra prediction angle for 4:2:2 chroma in Proposal 3

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table 5.

Table 5 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The encoding and decoding time is measured with the writing of yuv file.

## Simulation results

### Simulation results of Proposal 1

Table 6 presents the simulation results of Proposal 1A. The mapped intra prediction mode is not applied to the decision process of scanning order for residual coding.

Table 6 Simulation results of Proposal 1A

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.2% | -0.2% | -0.3% | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |

Table 7 presents the simulation results of Proposal 1B. The mapped intra prediction mode is also applied to the decision process of scanning order for residual coding.

Table 7 Simulation results of Proposal 1B applying into MDCS

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.2% | -0.3% | -0.4% | -0.2% | -0.2% | -0.3% | -0.1% | -0.2% | -0.1% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

### Simulation results of Proposal 2

Table 8 presents the simulation results of Proposal 2A. The mapped intra prediction mode is not applied to the decision process of scanning order for residual coding.

Table 8 Simulation results of Proposal 2A

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 9 presents the simulation results of Proposal 2B. The mapped intra prediction mode is also applied to the decision process of scanning order for residual coding.

Table 9 Simulation results of Proposal 2B

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

### Simulation results of Proposal 3

Table 10 presents the simulation results of Proposal 3A. The mapped intra prediction mode is not applied to the decision process of scanning order for residual coding.

Table 10 Simulation results of Proposal 3A

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |

Table 11 presents the simulation results of Proposal 3B. The mapped intra prediction mode is also applied to the decision process of scanning order for residual coding.

Table 11 Simulation results of Proposal 3B

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  |  | All Intra Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |  | Y | U | V |  |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | -0.1% | -0.1% |  | 0.0% | 0.0% | -0.1% |  | 0.0% | 0.0% | 0.0% |  |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | 0.1% | 0.1% |  | 0.1% | 0.1% | 0.1% |  | 0.0% | 0.0% | 0.0% |  |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | -0.1% |  | 0.0% | 0.0% | -0.1% |  | 0.0% | 0.0% | 0.0% |  |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.0% | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |  |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |  |
| Overall | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  | 100% |  |  |  |  |

# Conclusions

In this proposal, intra prediction angles for 4:2:2 chroma are unified with intra prediction angles for luma and 4:2:0/4:4:4 chroma. No additional intra prediction angles for 4:2:2 chroma are required.

Each intra prediction mode for 4:2:2 chroma is derived from intra prediction mode for luma and 4:2:0/4:4:4 chroma using a mapping table before the decoding process of intra sample prediction. As a result, only existing intra prediction angles in HEVC version 1 are required as intra prediction angles for 4:2:2 choroma. In Method B, decoding process of intra sample prediction is used without changing from HEVC version 1.

It is recommended that this proposal be adopted into HEVC Range Extensions Text and HM-RExt.

# References

[1] “HEVC Range Extensions Draft 2”,
D. Flynn, J. Sole, T. Suzuki, JCTVC-L1005, Geneva, January 2013

[2] “Common test conditions and software reference configurations for HEVC range extensions”,
D. Flynn, K. Sharman, JCTVC-L1006, Geneva, January 2013

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working Draft changes

Working Draft changes are described in the attached file JCTVC-M0127_WD.doc.

# Appendix

## Derivation of the mapping table of Proposal 2

The intra prediction modes for 4:2:2 chroma of the mapping table of Proposal 2 (Table 3) are derived as follows:

void Mapping2(int intraPredModeC, int & intraPredModeC422) { int sign; if (intraPredModeC <= 1) { intraPredModeC422 = intraPredModeC; } else if (intraPredModeC < 18) { intraPredModeC422 = Clip3(2, 18, (intraPredModeC - 10) * 2 + 10); } else { sign = ((intraPredModeC - 26) < 0) ? -1 : 1; intraPredModeC422 = (abs(intraPredModeC - 26) + 1 >> 1) * sign + 26; } }

## Derivation of the mapping table of Proposal 3

The intra prediction modes for 4:2:2 chroma of the mapping table of Proposal 3 (Table 4) are derived as follows:

void Mapping3(int intraPredModeC, int & intraPredModeC422) { int sign; if (intraPredModeC <= 1) { intraPredModeC422 = intraPredModeC; } else if (intraPredModeC < 16) { intraPredModeC422 = Max(2, (intraPredModeC - 10) * 2 + 10); } else { sign = ((intraPredModeC - 26) < 0) ? -1 : 1; intraPredModeC422 = (abs(intraPredModeC - 26) + 1 >> 1) * sign + 26; } }