| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0128 |
| --- | --- |

| Title: | SCE5: Cross-check of SCE5 Test 5.2.1 on Effectiveness of temporal motion vector prediction (JCTVC-M0283) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

SCE5 Test 5.2.1 studies the restriction of temporal merging candidate in enhancement layer and SCE5 Test 5.2.2 studies the motion data compression for co-located reference layer merging candidates in enhancement layer.

This contribution reports the cross-check results of the Test.

The BD-rate results tested by the cross-checker perfectly match those provided by the proponent.

# Introduction

SCE5 Test 5.2.1 studies the restriction of temporal merging candidate in enhancement layer and SCE5 Test 5.2.2 studies the motion data compression for co-located reference layer merging candidates in enhancement layer [1].

This contribution reports the cross-check results of the Test.

# Experiments

## Tests and software

- Anchor (SHM-1.0)

On the derivation process for luma motion vectors for merge mode of the enhancement layer for SHM-1.0 [2], both a co-located reference layer merging candidate and a temporal merging candidate are derived.

- Tested: SCE5 Test 5.2.1

On the derivation process for luma motion vectors for merge mode of the enhancement layer in this Test, temporal merging candidates are not derived.

The following settings are tested.

Test 1 - Test 5.2.1
The motion data for co-located reference layer merging candidates is compressed by 1/16.
	(as the same as SHM1.0)

Test 2 - Test 5.2.1 with 5.2.2 (uncompressed MV)
The motion data for co-located reference layer merging candidates is not compressed.

Test 3 - Test 5.2.1 with 5.2.2 (half compressed MV)
The motion data for co-located reference layer merging candidates is compressed by 1/4.

## Test conditions

The common SHM test conditions [3] are used.

## Experimental environment

The experimental environment used is presented in Table 1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The encoding and decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 presents the simulation results for Test 1.

Table 2 Simulation results of Test 1

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.8% | 0.6% | 0.7% |  |  |  | 0.3% | 0.2% | 0.2% |
| Class B | 0.8% | 0.3% | 0.4% | 0.3% | 0.1% | 0.0% | 0.4% | 0.1% | 0.1% |
| Overall (Test vs Ref) | 0.8% | 0.4% | 0.5% | 0.3% | 0.1% | 0.0% | 0.4% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 20.2% | 32.5% | 33.7% | 16.9% | 28.6% | 31.0% | 15.3% | 29.0% | 32.9% |
| EL only (Test vs Ref) | 0.8% | 0.5% | 0.5% | 0.3% | 0.0% | 0.0% | 0.3% | 0.0% | 0.1% |
| Enc Time[%] | 99.4% |  |  | 99.5% |  |  | 99.8% |  |  |
| Dec Time[%] | 99.5% |  |  | 100.5% |  |  | 100.4% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.8% | 0.9% | 1.1% |  |  |  | 0.3% | 0.3% | 0.5% |
| Class B | 1.0% | 0.7% | 0.6% | 0.6% | 0.3% | 0.3% | 0.6% | 0.5% | 0.4% |
| Overall (Test vs Ref) | 0.9% | 0.7% | 0.7% | 0.6% | 0.3% | 0.3% | 0.5% | 0.4% | 0.4% |
| Overall (Test vs single layer) | 27.2% | 37.5% | 39.2% | 23.3% | 33.5% | 36.3% | 22.9% | 35.0% | 39.7% |
| EL only (Test vs Ref) | 1.2% | 1.0% | 1.0% | 0.7% | 0.4% | 0.4% | 0.6% | 0.5% | 0.5% |
| Enc Time[%] | 102.1% |  |  | 101.7% |  |  | 101.5% |  |  |
| Dec Time[%] | 100.2% |  |  | 100.5% |  |  | 101.0% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.7% | 0.9% | 1.0% |  |  |  | 0.2% | 0.3% | 0.4% |
| Class B | 0.9% | 0.3% | 0.3% | 0.5% | 0.1% | 0.2% | 0.6% | 0.1% | 0.3% |
| Overall (Test vs Ref) | 0.8% | 0.5% | 0.5% | 0.5% | 0.1% | 0.2% | 0.4% | 0.2% | 0.3% |
| Overall (Test vs single layer) | 29.3% | 38.0% | 39.3% | 25.3% | 33.1% | 35.4% | 24.3% | 33.7% | 37.5% |
| EL only (Test vs Ref) | 1.1% | 0.7% | 0.7% | 0.6% | 0.2% | 0.2% | 0.5% | 0.3% | 0.4% |
| Enc Time[%] | 100.8% |  |  | 100.5% |  |  | 100.6% |  |  |
| Dec Time[%] | 101.9% |  |  | 100.0% |  |  | 99.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 3 presents the simulation results for Test 2.

Table 3 Simulation results of Test 2

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.3% | -3.3% | -3.0% |  |  |  | -0.6% | -2.1% | -2.1% |
| Class B | -0.2% | -1.3% | -1.4% | -0.2% | -0.8% | -0.9% | 0.0% | -0.9% | -1.0% |
| Overall (Test vs Ref) | -0.5% | -1.9% | -1.9% | -0.2% | -0.8% | -0.9% | -0.2% | -1.2% | -1.3% |
| Overall (Test vs single layer) | 18.7% | 29.5% | 30.5% | 16.3% | 27.5% | 29.7% | 14.6% | 27.2% | 30.9% |
| EL only (Test vs Ref) | -1.1% | -2.6% | -2.5% | -0.5% | -1.2% | -1.3% | -0.5% | -1.7% | -1.8% |
| Enc Time[%] | 98.4% |  |  | 99.3% |  |  | 99.8% |  |  |
| Dec Time[%] | 99.2% |  |  | 100.6% |  |  | 99.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.1% | -2.2% | -2.0% |  |  |  | -0.9% | -1.6% | -1.6% |
| Class B | 0.1% | -0.7% | -0.7% | 0.1% | -0.4% | -0.5% | 0.0% | -0.4% | -0.5% |
| Overall (Test vs Ref) | -0.3% | -1.1% | -1.1% | 0.1% | -0.4% | -0.5% | -0.2% | -0.7% | -0.8% |
| Overall (Test vs single layer) | 25.7% | 34.9% | 36.7% | 22.6% | 32.6% | 35.3% | 22.0% | 33.4% | 37.9% |
| EL only (Test vs Ref) | -0.6% | -1.5% | -1.5% | 0.0% | -0.5% | -0.6% | -0.5% | -1.0% | -1.1% |
| Enc Time[%] | 100.7% |  |  | 101.3% |  |  | 101.3% |  |  |
| Dec Time[%] | 99.6% |  |  | 100.3% |  |  | 99.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | -2.4% | -2.2% |  |  |  | -0.9% | -1.6% | -1.6% |
| Class B | -0.1% | -1.0% | -1.0% | 0.0% | -0.5% | -0.6% | 0.1% | -0.6% | -0.5% |
| Overall (Test vs Ref) | -0.4% | -1.4% | -1.4% | 0.0% | -0.5% | -0.6% | -0.2% | -0.9% | -0.8% |
| Overall (Test vs single layer) | 27.6% | 35.4% | 36.7% | 24.6% | 32.2% | 34.3% | 23.5% | 32.2% | 35.9% |
| EL only (Test vs Ref) | -0.9% | -1.8% | -1.8% | -0.2% | -0.8% | -0.9% | -0.5% | -1.2% | -1.1% |
| Enc Time[%] | 99.9% |  |  | 100.3% |  |  | 100.6% |  |  |
| Dec Time[%] | 99.5% |  |  | 99.5% |  |  | 99.4% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 4 presents the simulation results for Test 3.

Table 4 Simulation results of Test 3

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | -2.0% | -1.9% |  |  |  | -0.5% | -1.8% | -1.9% |
| Class B | 0.1% | -0.9% | -1.0% | -0.1% | -0.7% | -0.8% | 0.0% | -0.8% | -0.8% |
| Overall (Test vs Ref) | -0.1% | -1.2% | -1.2% | -0.1% | -0.7% | -0.8% | -0.1% | -1.1% | -1.1% |
| Overall (Test vs single layer) | 19.1% | 30.4% | 31.4% | 16.4% | 27.6% | 29.9% | 14.7% | 27.4% | 31.1% |
| EL only (Test vs Ref) | -0.6% | -1.7% | -1.7% | -0.4% | -1.0% | -1.1% | -0.4% | -1.5% | -1.6% |
| Enc Time[%] | 97.9% |  |  | 98.6% |  |  | 99.2% |  |  |
| Dec Time[%] | 99.8% |  |  | 101.9% |  |  | 101.1% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.9% | -0.7% |  |  |  | -0.7% | -1.3% | -1.2% |
| Class B | 0.3% | -0.3% | -0.4% | 0.2% | -0.3% | -0.3% | 0.1% | -0.2% | -0.3% |
| Overall (Test vs Ref) | 0.1% | -0.5% | -0.5% | 0.2% | -0.3% | -0.3% | -0.1% | -0.6% | -0.6% |
| Overall (Test vs single layer) | 26.2% | 35.8% | 37.6% | 22.7% | 32.7% | 35.5% | 22.2% | 33.6% | 38.3% |
| EL only (Test vs Ref) | 0.0% | -0.6% | -0.6% | 0.1% | -0.4% | -0.4% | -0.4% | -0.8% | -0.8% |
| Enc Time[%] | 100.6% |  |  | 100.7% |  |  | 100.5% |  |  |
| Dec Time[%] | 100.8% |  |  | 101.3% |  |  | 101.1% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | -1.2% | -1.0% |  |  |  | -0.8% | -1.4% | -1.3% |
| Class B | 0.2% | -0.7% | -0.8% | 0.0% | -0.4% | -0.4% | 0.1% | -0.5% | -0.5% |
| Overall (Test vs Ref) | -0.1% | -0.8% | -0.8% | 0.0% | -0.4% | -0.4% | -0.2% | -0.8% | -0.8% |
| Overall (Test vs single layer) | 28.1% | 36.2% | 37.5% | 24.7% | 32.4% | 34.5% | 23.6% | 32.4% | 36.0% |
| EL only (Test vs Ref) | -0.3% | -1.1% | -1.1% | -0.1% | -0.6% | -0.6% | -0.4% | -1.0% | -1.0% |
| Enc Time[%] | 99.2% |  |  | 99.6% |  |  | 99.8% |  |  |
| Dec Time[%] | 100.5% |  |  | 100.3% |  |  | 100.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusions

The BD-rate results by the cross-checker perfectly match those provided by the proponents on RA and LD-P settings. The BD-rate results on LD-B setting have not been checked yet.

It is recommended that a temporal merging candidate shall not be derived when a co-located reference layer merging candidate is derived in enhancement layer.

# References

[1] “Description of Scalable Extensions Core Experiment 5 (SCE5): Inter-layer syntax prediction using HEVC base layer”,
V. Seregin, P. Onno, S. Liu, E. Alshina, C. Kim, H. Yang, JCTVC-L1105, Geneva, January 2013

[2] “SHVC Test Model 1 (SHM 1)”,
J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, JCTVC-L1007, Geneva, January 2013

[3] “Common SHM test conditions and software reference configurations”,
X. Li, J. Boyce, P. Onno, Y. Ye, JCTVC-L1009, Geneva, January 2013

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).