| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0129_r1 |
| --- | --- |

| Title: | AHG 9: Inter-layer Prediction Indication at Picture Level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon Do-Kyoung Kwon Madhukar Budagavi | Tel: Email: | hendry.hendry@lge.com bm.jeon@lge.com d-kwon@ti.com madhukar@ti.com |
| Source: | LG Electronics and Texas Instruments |  |  |

_____________________________

# Abstract

This contribution proposes two flags called ilp_flag and ilp_ref_flag and their use during decoding process. The proposed flags indicate whether or not the current picture is needed for inter-layer prediction and inter prediction, respectively. It is assessed that the two flags may helps to lighten decoding process by allowing decoder to recognize pictures that can be skip from decoding process.

In the first revision, description of why two flags instead of one flag to indicate inter-layer prediction at picture level is needed and also more details on proposed item related to reference picture list management based on the proposed two flags is described.

# Introduction

In the two previous JCT-VC meetings, two flags for inter-layer prediction indication at picture level have been proposed [1][2] and were suggested to be further studied, particularly in the respect of how these two flags affect decoding process. For readers’ convenience, the motivation and example use case of the two flags are copied from [2].

The proposed flags are:

- ilp_flag to indicate whether or not the current slice / picture is needed for inter-layer prediction.

- ilp_ref_flag to indicate whether or not the current slice / picture is needed for inter prediction by other pictures whose ilp_flag equals 1.

The motivation for having the two flags is as follow:

- As a way to trade off coding efficiency and memory bandwidth in multi-loop HEVC scalable extension, some of pictures are not used for inter-layer prediction. Thus, given a particular target layer for decoding and output, coded pictures that are not needed to decode pictures in the target output layer should be clearly indicated so that they can be easily discarded and not to be decoded which saves computing resource.

- Given that a picture may or may not be decoded depending on the given target layer, reference picture list construction may be more complicated since it is necessary to differentiate the reference picture list construction process for a picture / slice when it is in the target layer and when it is not in the target layer (i.e., its layer is less than the target layer). Using different RPS for target and non-target layer does not seem to be a desired solution as it may require more bit count for signalling. The two flags may be utilized so that the same RPS can be used for reference picture list construction pictures in both target and non-target layers.

Figure 1 illustrates an example of a bitstream with two spatial layers (i.e., BL and EL1). Both layers are coded following random access configuration in the common test condition (CTC) structure; however, for BL, only pictures marked as IL and ILR are allowed to be used for inter-layer prediction to reduce memory / DPB size requirement. With this configuration, the BL only requires storage of 3 decoded pictures in the DPB when it is not to be output / displayed.

Decoding process for the bitstream shown in the Figure 1 depends on whether or not pictures in BL are output / displayed. Pictures that are not needed for inter-layer prediction and not needed to be output can be omitted from decoding process.

Case 1: only the highest layer needs to be output

- TargetDecLayerIdList = {0, 1}

- TargetOutputLayerIdList = {1}

- Thus, pictures need to be decoded: (00 01) (80 81) (40 41) (21) (11) (31) (61) (51) (71) (160 161) (120 121) (102) (92) (112) (142) (131) (151)

Case 2: both layers need to be output

- TargetDecLayerIdList = {0, 1}

- TargetOutputLayerIdList = {0, 1}

- Thus, pictures need to be decoded: (00 01) (80 81) (40 41) (20 21) (10 11) (30 31) (60 61) (50 51) (70 71) (160 161) (120 121) (100 101) (90 91) (110 111) (140 141) (130 131) (150 151)

Figure 1 – Example of use of ilp_flag and ilp_ref_flag.

It may be argued that to indicate whether or not a reference picture from reference layer is needed for ILP, one flag (e.g., the above ilp_flag) is sufficient. However, there may be two cases related to reference picture and ILP:

- The reference picture is needed for ILP by current picture.

- The reference picture is not needed for ILP by current picture but needed by other picture in the same layer and that picture is needed for ILP.

By having these two cases indicated by different flags, it is assessed that decoder may be able to mark reference picture that is only needed for ILP by current picture but not needed for reference by other ILP reference picture as ‘unused for reference’ earlier as proposed in section 2.3.2

# Proposal

## Proposed signalling

The syntax and semantic of the proposed flags are as follow:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| ilp_flag | u(1) |
| ilp_ref_flag | u(1) |
| … |  |
| } |  |

ilp_flag equals 1 indicates that current slice may be used for inter-layer prediction. Otherwise, ilp_flag equals 0 indicates that current slice is not used for inter-layer prediction. It is requirement of the bitstream that the value of this flag shall be the same in all slice segment headers of a coded picture. When not present, the value of ilp_flag is inferred to be 1.

ilp_ref_flag equals 1 indicates that current slice may be referred / used for inter prediction by other picture(s) that are needed for inter-layer prediction. ilp_ref_flag equals 0 indicates that current slice is not  referred / used for inter prediction by other picture(s) that are needed for inter-layer prediction. It is requirement of the bitstream that the value of this flag shall be the same in all slice segment headers of a coded picture. When not present, the value of ilp_ref_flag is inferred to be 1.

## Proposed text for decoding process

The specifications of subclause 8.1 apply with the following modification.

The variable TargetOutputLayerIdList, which identifies the list of nuh_layer_id_values, in increasing order of nuh_layer_id values, of the NAL units to be outputted, is specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetOutputLayerIdList, TargetOutputLayerIdList is set by the external means.

–	Otherwise, TargetOutputLayerIdList includes nuh_layer_id j that its output_layer_flag[ active_layer_set ][j ] equals 1 for j starts from 0 to vps_max_layers_minus1, inclusively.

When the values of ilp_flag and ilp_ref_flag of the current picture are both equal 0 and num_layer_id of current NAL unit is not included in TargetOutputLayerIdList, the current picture may not be decoded / may be skipped from decoding process

After decoding all layer picture in the same access unit, all layer pictures whose ilp_ref_flag equals 0 and num_layer_id of NAL unit is not included in TargetOutputLayerIdLiet are removed from decoded picture buffer.

## Proposed modification to reference picture list construction

### Modification to marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0

For more efficient DPB space usage, we propose that after decoding of a coded picture with the highest nuh_layer_id, reference pictures from reference layer with the same POC that the picture whose ilp_flag equals 1 but ilp_ref_flag equals 0 shall be marked as “unused for reference”.

### Modification to reference picture list construction

When pictures that are not needed for inter-layer prediction are removed / not decoded, reference picture list for pictures in the same POC in the layer that needed for output and layer not needed for output will be different. Figure 2 illustrates the same example shown in Figure 1 in the decoder side, where the pictures that are not needed for inter-layer prediction are removed in BL. As illustrated, the temporal prediction for pictures marked as ILR are different from that of in EL1, which implies that we may need to have different RPSs for BL and EL1. However, by utilizing the proposed ilp_flag and ilp_ref_flag, a single RPS can be used for all layers.

Figure 2 – Same example with Figure 1, in which the pictures not needed for inter-layer prediction are removed in the decoder side.

We propose that reference picture lists of a slice are constructed as follow:

- If the ilp_flag of the slice equals 0, the reference picture lists for the slice are constructed based on RPS only. That is, no change to the current reference picture lists construction

- If the ilp_flag of the slice equals 1, the reference picture lists for the slice are constructed by including reference pictures that:

  - Have ilp_ref_flag equals 1, and

  - Are included in the RPS for that slice.

The required modification to the reference picture lists construction can be described as follow (removed text from current specification is indicated by striken red highlighted text and the newly added text are indicated by yellow highlighted):

- G.7.4.7.2  Reference picture list modification semantics

The specifications in subclause Error: Reference source not found apply with following modifications.

–	Equation (7-43) specifying the derivation of NumPocTotalCurr is replaced by:

NumPocTotalCurr = 0
for( i = 0; i < NumNegativePics[ CurrRpsIdx ]; i++)
	if(UsedByCurrPicS0[ CurrRpsIdx ][ i ])
		NumPocTotalCurr++
for( i = 0; i < NumPositivePics[ CurrRpsIdx ]; i++)
	if(UsedByCurrPicS1[ CurrRpsIdx ][ i ])
		NumPocTotalCurr++
for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ )
	if( UsedByCurrPicLt[ i ])
		NumPocTotalCurr++
NumPocTotalCurr += NumDirectRefLayers NumRefPicSetInterLayer [ LayerIdInVps[ nuh_layer_id ] ]

- G.8.1.1  The Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer reference pictures RefPicSetInterLayer and NumRefPicSetInterLayer[LayerIdInVps[ nuh_layer_id ] ].

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0, k = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) {

ilpFlag = ilp_flag of the picture with picture order count equal to PicOrderCnt and nuh_layer_id

equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ][ i ] ]

if (ilpFlag = = 1) {   
  RefPicSetInterLayer[ i k ] = the picture with picture order count equal to PicOrderCnt and ilpFlag equal to 1 and nuh_layer_id equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ][ i ] ]
  RefPicSetInterLayer[ i k] is marked as "used for long-term reference"

k++

}   
}

NumRefPicSetInterLayer[LayerIdInVps[ nuh_layer_id ] ] = k;

- G.8.3.4  Decoding process for reference picture lists construction

…

Let IlpRefList	denotes	list of ilp_ref_flag of pictures and IlpRefList [x] denotes ilp_ref_flag of picture with POC x. The reference picture lists construction is as the following:

At the beginning of the decoding process for each slice, the reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {

for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++ , i++ ) 	
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetStCurrBefore[ i ]]))
			RefPicListTemp0[ rIdx++ ] = RefPicSetStCurrBefore[ i ] 
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++ , i++ ) 	
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetStCurrAfter [ i ]]))
			RefPicListTemp0[ rIdx++ ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++ , i++ )
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetLtCurr [ i ]]))
			RefPicListTemp0[ rIdx++ ] = RefPicSetLtCurr[ i ]
}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetStCurrAfter [ i ]]))
			RefPicListTemp1[ rIdx++ ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetStCurrBefore [ i ]]))
			RefPicListTemp1[ rIdx++ ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetLtCurr [ i ]]))
			RefPicListTemp1[ rIdx++ ] = RefPicSetLtCurr[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

# References

[1] D.-K. Kwon, M. Budagavi and M. Zhou, “Hierarchical inter-layer prediction in multi-loop scalable extension of HEVC,” JCTVC-K0264, the 11th JCTVC meeting, Shanghai, CN, October 2012.

[2] Hendry, B. M. Jeon, D.-K. Kwon and M. Budagavi, “Inter layer prediction indication flags and pictures management,” JCTVC-L0258, the 12th JCTVC meeting, Geneva, CH, January 2013.

[3] “SHVC Draft Text 1,” JCTVC-L1008, the 12th JCTVC meeting, Geneva, CH, January 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).