| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0130_r1 |
| --- | --- |

| Title: | AHG 9: Signalling required DPB size for in layer set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon Do-Kyoung Kwon Madhukar Budagavi | Tel: Email: | hendry.hendry@lge.com bm.jeon@lge.com d-kwon@ti.com madhukar@ti.com |
| Source: | LG Electronics and Texas Instruments |  |  |

_____________________________

# Abstract

This contribution is the result of further study recommended for previous contribution JCTVC-L0262. This contribution proposes to signal the required DPB size for each layer in layer set. It is assessed that the information may help decoder to understand the memory requirement for selecting the layer set it will be operating on.

# Introduction

For multi-layers HEVC extension bitstreams (e.g., SHVC and MV-HEVC), signalling of Decoded Picture Buffer (DPB) size is important to inform decoder of the required number of memory units in term of decoded picture so that a decoder can handle a given bitstream at any operating point. By observing such information, decoder can, for example, determine whether it can decode the given input bitstream at a particular operating point / layer set.

Figure 1 illustrates situation where an SHVC bitstream has three layers with three layer sets that give decoders possibility to operate at different highest layers. For the given example, a decoder that operates at layer set 0 will be required to store up to 6 decoded pictures of BL; a decoder that operates at layer set 1 will be required to store up to 3 decoded pictures of BL and 6 decoded pictures of EL1; and finally a decoder that operates at layer set 2 will be required to store up to 3 decoded pictures of BL, 3 decoded pictures of EL1, and 6 decoded pictures of EL2. If the decoder decides that it does not have enough memory to decode the bitstream up to the EL2 layer, it may target for operating at layer set 1 or at layer set 0.

Figure 1 – Illustration of different DPB size requirement for layer sets.

# Proposal

To do the signalling as illustrated in section 1, we propose two options. The first option simply always signals the required DPB for each layer in layer set. The second option offers a control flag for not signalling the required DPB for each layer in layer set.

Option 1

The proposed syntax element(s) are highlighted with yellow in the following table.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_num_layer_sets_minus1 | ue(v) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) |  |
| for( j = 0; j <= vps_max_layer_id; j++ ) { |  |
| layer_id_included_flag[ i ][ j ] | u(1) |
| if( layer_id_included_flag[ i ][ j ] ) |  |
| layer_set_max_dec_pic_buffering_minus1[ i ][ j ] | ue(v) |
| } |  |
| … |  |
| } |  |

layer_set_max_dec_pic_buffering_minus1[ i ][ j ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers for decoding of pictures with layer id equals j and layer set i is being the target layer set for decoding.

Option 2

The proposed syntax element(s) are highlighted with yellow in the following table.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_num_layer_sets_minus1 | ue(v) |
| vps_layer_set_dec_pic_buffering_info_present_flag | u(1) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) |  |
| for( j = 0; j <= vps_max_layer_id; j++ ) { |  |
| layer_id_included_flag[ i ][ j ] | u(1) |
| if(vps_layer_set_dec_pic_buffering_info_present_flag && layer_id_included_flag[ i ][ j ]) |  |
| layer_set_max_dec_pic_buffering_minus1[ i ][ j ] | ue(v) |
| } |  |
| … |  |
| } |  |

vps_layer_set_dec_pic_buffering_info_present_flag equals 1 indicates that the syntax element layer_set_max_dec_pic_buffering[ i ][ j ] is present. Otherwise, vps_layer_set_dec_pic_buffering_info_present_flag equals 0 indicates that the syntax element layer_set_max_dec_pic_buffering[ i ][ j ] is not present.

layer_set_max_dec_pic_buffering_minus1[ i ][ j ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers for decoding of pictures with layer id equals j and layer set i is being the target layer set for decoding. When not present, due to vps_layer_set_dec_pic_buffering_info_present_flag equal to 0, the value of layer_set_max_dec_pic_buffering[i][j] is inferred to be equal to vps_max_dec_pic_buffering_minus1[vps_max_sub_layers_minus1].

# References

- Hendry, B. Jeon, D.-K. Kwon, M. Budagavi and M. Zhou, “AHG 9: Signalling of required DPB size in VPS,” Document JCTVC-L0262, January 2013.

- D.-K. Kwon, M. Budagavi and M. Zhou, “Hierarchical inter-layer prediction in multi-loop scalable extension of HEVC,” Document JCTVC-K00264, October 2012.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).