| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0131 |
| --- | --- |

| Title: | AHG 9: Sub-bitstream extraction for pictures not needed for inter-layer prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeongmoon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the 12th JCTVC meeting, ideas to signal indication of inter-layer prediction at both sequence and picture level have been proposed. Such indication will enable identification of pictures that can be removed from the input bitstream. This contribution proposes sub-bitstream extraction process that consider removal based on whether or not a picture in is needed by other picture for inter-layer prediction.

# Problem Description

In the 12th JCTVC meeting, ideas to signal indication of inter-layer prediction at both sequence and picture level have been proposed [1][2]. Such indication will enable identification of pictures that can be removed from the input bitstream. Figure 1 illustrates configuration scenario where the use of ILP is disabled for certain temporal sub-layer. Pictures represented with dashed vertical lines are pictures in temporal sub-layers that are not used for ILP by its collocated picture in the higher layer. If pictures in layer 0 are not meant to be outputted / displayed, pictures that are not needed for ILP can be removed from the bitstream through sub-bitstream extraction process.

For removing pictures that are not needed for ILP through sub-bitstream extraction process, there are four pieces of information needed, they are:

- The highest temporal sub-layer to be decoded.

- List of target layer for decoding.

- List of maximum sub-layer for ILP in each layer.

- List of target layer for output.

The highest temporal sub-layer and the list of target layer for to be decoded are already specified in HEVC specification. The list of maximum sub-layers for ILP in each layer is proposed in the next section and the list of target layer for output can simply be populated from the highest layer in the list of target layer for decoding. For the illustrated scenario, the values of required information are as follow:

- The highest temporal sub-layer to be decoded is 3.

- List of target layer for decoding is {0, 1}.

- List of maximum sub-layer for ILP in each layer {1, 3}.

- List of target layer for output {1}.

Figure 1 – Example of disabling ILP for temporal sub-layer 2 and 3 in layer 0.

Current SHVC specification does not specify sub-bitstream extraction process to remove pictures that are indicated not used for inter-layer prediction. In the following section, we propose modified text to support the extraction process.

# Proposal

Inputs to this process are a bitstream, a target highest TemporalId value tIdTarget, and a target layer identifier list layerIdListTarget, and a target layer identifier list layerIdListOutput.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream of the process specified in this subclause with tIdTarget equal to any value in the range of 0 to 6, inclusive, and layerIdListTarget equal to the layer identifier list associated with a layer set specified in the active video parameter set shall be a conforming bitstream.

NOTE 1 – A conforming bitstream contains one or more coded slice segment NAL units with nuh_layer_id equal to 0 and TemporalId equal to 0.

The output sub-bitstream is derived as follows:

–	When one or more of the following two conditions are true, remove all SEI NAL units that have nuh_layer_id equal to 0 and that contain a non-nested buffering period SEI message, a non-nested picture timing SEI message, or a non-nested decoding unit information SEI message:

–	layerIdListTarget does not include all the values of nuh_layer_id in all NAL units in the bitstream.

–	tIdTarget is less than the greatest TemporalId in all NAL units in the bitstream.

NOTE 2 – A "smart" bitstream extractor may include appropriate non-nested buffering picture SEI messages, non-nested picture timing SEI messages, and non-nested decoding unit information SEI messages in the extracted sub-bitstream, provided that the SEI messages applicable to the sub-bitstream were present as nested SEI messages in the original bitstream.

–	Remove all NAL units with TemporalId greater than tIdTarget or nuh_layer_id not among the values included in layerIdListTarget.

–	Remove all NAL units with nuh_layer_id among the values included in layerIdListTarget but not among the values included in layerIdListOutput; nal_unit_type is not one of intra random access point (IRAP) picture; and max_sublayer_for_ilp_plus1[nuh_layer_id] equals 0.

–	Remove all NAL units with nuh_layer_id among the values included in layerIdListTarget but not among the values included in layerIdListOutput; and TemporalId equals to or greater than max_sublayer_for_ilp_plus1[nuh_layer_id].

# References

[1] “Inter layer prediction indication flags and pictures management,” JCTVC-L0258, the 12th JCTVC meeting, Geneva, CH, January 2013.

[2] “Design Considered for Signalling Inter-layer Prediction Indication,” JCTVC-L0449, the 12th JCTVC meeting, Geneva, CH, January 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).