- H.7.3.8.5  Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( nuh_layer_id > 0 && InterLayerTextureRlEnableFlag) |  |
| texture_rl_flag[ x0 ][ y0 ] | ae(v) |
| if( ! texture_rl_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| if(( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) ¦¦ PredMode[ x0 ][ y0 ] = = MODE_TEXTURE_RL) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| if ( PredMode[ x0 ][ y0 ] != MODE_INTRA && PredMode[ x0 ][ y0 ] != MODE_INTRA_BL && PartMode = = PART_2Nx2N) |  |
| APC_enable_flag[ x0 ][ y0 ] | ae(v) |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

- H.7.4.9.5 Coding unit semantics

The specifications in subclause F.7.4.9.5 apply with following modifications and additions.

texture_rl_flag[ x0 ][ y0 ] specifies the prediction mode for the current coding unit. texture_rl_flag[ x0 ][ y0 ] equal to 1 specifies that the prediction samples are derived using reconstructed samples of the reference layer representation as specified in subclause Error: Reference source not found; texture_rl_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded in either intra prediction or inter prediction mode.

When texture_rl_flag is equal to 1, the variable PredMode[ x ][ y ] and PartMode[ x ][ y ] are derived as follows.

for x = x0..x0 + nCbS − 1 and y = y0..y0 + nCbS – 1,

PredMode[ x ][ y ] = MODE_TEXTURE_RL		(H-1)

PartMode [ x ][ y ] = PART_2Nx2N		(H-2)

When texture_rl_flag[ x0 ][ y0 ] not present, it is inferred to be equal to 0.

When texture_rl_flag[ x0 ][ y0 ] is equal to 1, the current coding unit is regarded as being coded with intra prediction mode in the in-loop filter process specified in subclause H.8.7.

APC_enable_flag[ x0 ][ y0 ] specifies whether the adaptive predictor compensatoin is enabled for the current coding unit. APC_enable_flag [ x0 ][ y0 ] equal to 1 specifies that the prediction samples are derived by averaging the temporal predicted samples and the reconstructed samples of the reference layer representation as specified in subclause Error: Reference source not found.5. When APC_enable_flag [ x0 ][ y0 ] not present, it is inferred to be equal to 0.

- H.8.8.5  Decoding process for adaptive predictor compensation

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	two variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

–	a (PicWRL)x(PicHRL) array rlPicSampleL,

–	a (PicWRL)x(PicHRL) array rlPicSampleL,

–	an (nPbW)x(nPbH) array interPredSamplesL of luma inter prediction samples, where the interPredSamplesL [ x ][ y ] with ( x = 0 .. nPbW – 1, y = 0 .. nPbH – 1 ) are derived by invoking the luma sample interpolation process specified in subclause 8.5.3.3.1 with predSamplesL and luma sample location ( x ,  y ) given as inputs and predSamplesL[ x ][ y ] as output.

–	the reference indices refIdxL0 and refIdxL1 of the current luma prediction block,

Output of this process is a (nPbW)x(nPbH) array predredSamplesL of luma prediction samples.

For each luma sample location ( xL = 0..nPbW − 1, yL = 0..nPbH − 1 ) inside the prediction luma sample array predSampleL, the corresponding prediction luma sample value predSampleL[ xL][ yL ] is derived as follows:

- If the nPbW is equal to 8, the nPbH is equal to 8, the refIdxL0 is equal to or larger than 0, and the refIdxL1 is equal to or larger than 0, the following applies:

–	predSampleL[ xL][ yL ] = (interPredSamplesL [ xL][ yL ] + rlPicSampleL [ xC + xL][ yC + yL ]) >> 1

- Otherwise, the following applies:

–	predSampleL[ xL][ yL ] = interPredSamplesL [ xL][ yL ]