| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0132 |
| --- | --- |

| Title: | Non-SCE3.1: Disabling adaptive predictor compensation for 8x8 bi-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang, Polin Lai, Shan Liu, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yuwen.huang, polin.lai, shan.liu, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In SCE3.1, the adaptive predictor compensation (APC) is proposed to use the reconstructed base layer (BL) samples to refine the enhancement layer (EL) sample predictors. However, the worst case bandwidth is increased by 33%, and the worst case computations of in terms of adders and multipliers are increased by 41% and 44%, respectively. To reduce the worst bandwidth and computations, in this contribution, the APC is disabled when the coding unit (CU) size is 8x8 and the CU uses bi-prediction. Simulation results reportedly show 0.3%, 0.4%, 0.6%, 1.4%, 1.4%, 2.4%, 0.5%, 0.3%, and 0.7% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, LDP-SNR, LDB-2x, LDB-1.5x, and LDB-SNR, respectively, compared with SHM-1.0 IntraBL mode anchors. The encoding time increase is 4%, and the decoding time increase is roughly zero. The average bandwidth increase is 3%. The average computations in terms of adders and multipliers are roughly the same as the anchor. The worst case bandwidth and the worst case computations are unchanged.

# Introduction

In scalable video coding, the current base layer (BL) reconstructed picture may predict the current enhancement layer (EL) picture better than the EL reference picture for regions that newly appear or have light changes. If the BL quantization distortion is not large, the EL original samples should be close to the BL reconstructed samples. Therefore, in the EL inter prediction, the BL reconstructed samples can be used to refine the EL motion compensated predictors. The proposed technique is termed adaptive predictor compensation (APC). The APC was first proposed in JCTVC-L0072 [1] and were further investigated in SCE 3.1 [2]. However, in SCE 3.1, the worst case bandwidth is increased by 33%, and the worst case computations of in terms of adders and multipliers are increased by 41% and 44% respectively. In this contribution, the worst case bandwidth and computations are reduced by disabling the APC when the coding unit (CU) size is 8x8 and the CU uses bi-prediction. The average and worst case bandwidths and computations are investigated in this contribution. The rest of this contribution is organized as follows. The APC will be described in Section 2, simulation results, bandwidth and computation analyses will be shown in Section 3, and finally conclusions will be given in Section 4.

# Proposed method

In SCE 3.1, APC was proposed to use the BL reconstructed samples to refine the EL predictors. The refined EL sample predictors are generated by averaging the EL original sample predictors and the BL reconstructed samples of the corresponding block, as shown by the following equation.
	PredEL’ = (PredEL + RecBL) >> 1								(1)
The proposed APC is only applied to the luma component of each inter-coded coding unit (CU) with a partition mode equal to 2Nx2N and at least one non-zero residue. In addition, a CU-level APC_enable_flag is signaled to indicate whether the proposed APC is applied.

After analyzing the bandwidths and computations, we found that the worst case bandwidths and computations of APC will happen when the CU size equal to and the CU uses bi-prediction. To improve the worst case, the APC is disabled in this condition.

# Simulation results

## BD-rates and run times

Experiments were conducted based on SHM-1.0 IntraBL mode. Simulation results of disabling the proposed APC for 8x8 bi-prediction are shown in Table 1 and reportedly show 0.3%, 0.4%, 0.6%, 1.4%, 1.4%, 2.4%, 0.5%, 0.3%, and 0.7% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, LDP-SNR, LDB-2x, LDB-1.5x, and LDB-SNR, respectively. Compared with the original APC in SCE3.1, the average coding efficiency loss is less than 0.1% in terms of BD-rate. Compared with the anchor, the encoding time increase is 4%, and the decoding time increase is roughly zero.

## Bandwidths and computations

The bandwidths and computations of disabling APC for 8x8 bi-prediction are also evaluated. Experiments were conducted according to the methodology defined in JCTVC-L0440 [3], and the AHG17 released software is used. The detailed results are provided in JCTVC-M0132_Bandwidths&Computations.xls. The average bandwidths and computations are shown in Table 2. The average bandwidth increase is 3%. The average computations in terms of adders and multipliers are roughly the same as the anchor. The worst case bandwidths, computations, and memory usages are summarized in Table 3. They are all the same as those of the SHM-1.0 anchor.

# Conclusions

To reduce the worst case bandwidth and computations of the adaptive predictor compensation (APC) in SCE3.1, the APC is disabled for 8x8 bi-prediction. Simulation results reportedly show 0.3%, 0.4%, 0.6%, 1.4%, 1.4%, 2.4%, 0.5%, 0.3%, and 0.7% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, LDP-SNR, LDB-2x, LDB-1.5x, and LDB-SNR, respectively. The encoding time is roughly increased by 4%, and the decoding time is roughly unchanged. The average bandwidth increase is 3%, and the average computations are roughly unchanged. The worst case bandwidth, the worst case computations, and the memory usages in terms of number of reference frames and table size are all the same as those of the SHM-1.0 anchor. Compared with SCE3.1, the average coding efficiency loss is less than 0.1% in terms of BD-rate.

# References

- T.-D. Chuang, Y.-W. Huang, P. Lai, S. Liu, S. Lei, “TE3-4.3.1: Adaptive predictor compensation by using reconstructed BL texture,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0072, January, 2013.

- T.-D. Chuang, Y.-W. Huang, P. Lai, S. Liu, S. Lei, “SCE3.1: Adaptive predictor compensation,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0119, April, 2013.

- E. François, A. Tabatabai, E. Alshina, “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0440, January, 2013.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  BD-rates and run times of disabling APC for 8x8 bi-prediction

Table 2.  Average bandwidths and computations of disabling APC for 8x8 bi-prediction

Table 3.  Worst case bandwidth and computations of disabling APC for 8x8 bi-prediction

|  | The worst case computations |  | The worst case bandwidth |  | Memory usage |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Multipliers | Adders | MemBand (2D:4x2) | MemBand (2D:8x2) | Number of Ref. Frames | Tables Size |
| Proposed | 100% | 100% | 100% | 100% | 100% | 100% |