| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0133 |
| --- | --- |

| Title: | Non-SCE5 : Simplified inter-layer MV scaling and sample position mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In SHM-1.1, a division for deriving the picture resolution ratio of the enhancement layer (EL) to the base layer (BL) has to be performed for every inter-layer motion vector (MV) scaling and inter-layer position mapping, although the picture resolution ratio is a fixed value after the BL and EL picture sizes are determined. To reduce the complexity, in this contribution, the scaling factors for inter-layer MV scaling and sample position mapping can be first derived in the beginning of the slice encoding or decoding. Then, for all inter-layer MV scaling derivation and sample position mapping derivation during the rest of the slice encoding or decoding, the scaling factors are reused, and no division is required. For the sake of unification, the proposed dynamic range of the inter-layer scaling factors is the same as that of the MV scaling factor in HEVC. Therefore, the HEVC MV scaling module can be reused for inter-layer MV scaling. Simulation results reportedly show no coding efficiency change in SHM-1.1 IntraBL mode and RefIdx mode. The encoding time is roughly unchanged, and the decoding time is reduced by 2% and 1% for IntraBL mode and RefIdx mode, respectively.

# Introduction

In scalable video coding, the scaled base layer (BL) motion vectors (MVs) can be used for enhancement layer (EL) coding. In SHM-1.1 IntraBL mode, the BL center MV is inserted as the 1st merging candidate. In SHM-1.1 RefIdx mode, the motion fields of the inter-layer reference picture are filled with the BL MVs. If the picture resolutions of the EL and BL are different, an inter-layer MV scaling process is required. In addition, to get the corresponding MV in BL, the corresponding sample position in BL has to be derived. The inter-layer sample position mapping is required.

In SHM-1.1, the BL MV is scaled based on the picture resolution ratio of the EL to the BL. The scaled MV is derived as follows.
  mvEL_X = (mvBL_X × picEL_W+ (picBL_W/2-1)×sign(mvBL_X )) / picBL_W  		(1)
  mvEL_Y = (mvBL_Y × picEL_H+ (picBL_H/2-1)×sign(mvBL_Y )) / picBL_H   		(2)
In the Equations (1) and (2), (mvEL_X, mvEL_Y) is the scaled MV at EL, (mvBL_X, mvBL_Y) is the center MV of the corresponding block at BL, picEL_W and picEL_H are the picture width and height of the EL picture, and picBL_W and picBL_H are the picture width and height of the BL picture.

In the inter-layer sample position mapping, for a sample at EL, the sample position of the corresponding sample in BL is derived as follows:
  xBL = ( xEL × picBL_W + picEL_W / 2) / picEL_W						(3)
  yBL = ( yEL × picBL_H + picEL_H / 2 ) / picEL_H	    					(4)
In Equations (3) and (4), the (xEL, yEL) is the sample position of the sample at EL, and the (xBL, yBL) is the sample position of the corresponding sample at BL.

From the above equations, a division has to be performed for every inter-layer MV scaling and inter-layer sample position mapping, although the picture resolution ratio of the EL to the BL is a fixed value after the BL and EL picture sizes are determined. To reduce the complexity, simplified inter-layer MV scaling and sample position mapping are proposed in this contribution. The rest of this contribution is organized as follows. The proposed algorithm will be described in Section 2, simulation results will be shown in Section 3, and finally conclusions will be given in Section 4.

# Proposed methods

In the beginning of the slice encoding or decoding, the scaling factors for inter-layer MV scaling and sample position mapping are first derived. The scaling factors are reused for all inter-layer MV scaling derivation and sample position mapping derivation during the rest of the slice encoding or decoding to significantly reduce division operations. The details are described as follows.

## Inter-layer MV scaling

For inter-layer MV scaling, we first derive two inter-layer scaling factors for inter-layer MV scaling. One scaling factor is for the horizontal direction MV (MV_x), and the other is for the vertical direction MV (MV_y). The inter-layer MV scaling factors are first derived as follows.
  ILScalingFactor_x = Clip (-4096, 4095, ((picEL_W <<8) + (picBL_W>>1)) / picBL_W)	(5)
  ILScalingFactor_y = Clip (-4096, 4095, ((picEL_H <<8) + (picBL_H>>1)) / picBL_H)		(6)
In Equations (5) and (6), picEL_W and picEL_H are the picture width and height of the EL picture, and picBL_W and picBL_H are the picture width and height of the BL picture.

Then, the scaled MV is derived as follows.
  	mvEL_X = Clip (-32768, 32767, sign (ILScalingFactor_x×mvBL_X) ×
   				((abs (ILScalingFactor_x ×mvBL_X) +127)) >> 8)             		(7)
  	mvEL_Y = Clip (-32768, 32767, sign (ILScalingFactor_y×mvBL_Y) ×
            	((abs (ILScalingFactor_y ×mvBL_Y) +127)) >> 8)             		(8)
In Equations (7) and (8), (mvEL_X, mvEL_Y) is the scaled MV at EL, and (mvBL_X, mvBL_Y) is the center MV of the corresponding block at BL.

Note that, the dynamic range of the inter-layer scaling factors (ILScalingFactor_x and ILScalingFactor_y) is the same as that of the MV scaling factors in HEVC. Equations (7) and (8) are the same as the equation (8-154) in HEVC FDIS text [1]. The HEVC MV scaling module can be reused for inter-layer MV scaling.

## Inter-layer sample position mapping

Similar to the inter-layer MV scaling, we first derive two inter-layer position mapping scaling factors. One scaling factor is for the horizontal direction position, and the other is for the vertical direction position. The inter-layer position mapping scaling factors are first derived as follows.
  ILPosScalingFactor_x = ((picBL_W <<14) + (picEL_W>>1)) / picEL_W			(9)
  ILPosScalingFactor_y = ((picBL_H <<14) + (picEL_H>>1)) / picEL_H			(10)
In Equations (9) and (10), picEL_W and picEL_H are the picture width and height of the EL picture, and picBL_W and picBL_H are the picture width and height of the BL picture.

Then, for a sample at EL, the sample position of the corresponding sample at BL is derived as follows.
  xBL = ( xEL ×ILPosScalingFactor_x + (1<<13)) >> 14                       		(11)
  yBL = ( yEL ×ILPosScalingFactor_y + (1<<13)) >> 14					(12)
In Equations (11) and (12), (mvEL_X, mvEL_Y) is the scaled MV at EL, and (mvBL_X, mvBL_Y) is the center MV of the corresponding block at BL.

# Simulation results

Experiments were conducted based on SHM-1.1 IntraBL mode and RefIdx mode. All the results were simulated according to the common test conditions defined in JCTVC-L1009 [2]. Simulation results of the proposed methods in IntraBL mode are shown in Table 1. Simulation results of the proposed methods in RefIdx mode are shown in Table 2. No coding efficiency loss is observed for both modes. The bitstreams are exactly the same as the anchor bitstreams. The encoding time is roughly unchanged, and the decoding time is reduced by 2% and 1% for IntraBL mode and RefIdx mode, respectively.

# Conclusions

In this contribution, simplified inter-layer MV scaling and sample position mapping are proposed. The scaling factors for inter-layer MV scaling and sample position mapping can be first derived in the beginning of the slice encoding or decoding. The scaling factors are then reused for all inter-layer MV scaling derivation and sample position mapping derivation during the rest of slice encoding or decoding to significantly reduce division operations. Moreover, the dynamic range of the proposed scaling is the same as that of HEVC for unification. Simulation results reportedly show no coding efficiency loss in both SHM-1.1 IntraBL mode and RefIdx mode. The encoding time is roughly unchanged, and the decoding time is reduced by 2% and 1% for IntraBL mode and RefIdx mode, respectively.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent),” Document of Joint Collaborative Team on Video Coding, JCTVC-L1003, January, 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1009, January, 2013.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the proposed methods in IntraBL mode

Table 2.  Results of the proposed methods in RefIdx mode