| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0134 |
| --- | --- |

| Title: | AHG9: Inter-layer SPS prediction for HEVC extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thomas Rusert Ericsson Research 164 80 Stockholm Sweden | Tel: Email: | +46 76 7602660 thomas.rusert@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The contribution is a follow-up from JCTVC-L0137 with minor modifications considering agreed SPS/VPS designs according to JCTVC-L1003_v34 and JCTVC-L1008.

The contribution addresses a redundancy problem that is asserted to be present when two or more different SPSs are present for different layers in multi-layer HEVC extensions. Since many SPS syntax elements may have identical values across SPSs, those values would be duplicated, thus causing bit overhead for parameter set transmission and storage.

First, criteria for the design of VPS and SPS for HEVC extensions are suggested. Specifically, it is suggested that (1) the VPS should (per its primary design purpose) include parameters that are useful for bitstream extraction and/or capability exchange, (2) parameters that are not required for bitstream extraction and/or capability exchange should not be present in the VPS, (3) sequence-level parameters that may change in some layers while not affecting the general layer structure should not be present in the VPS. For the design of the SPS with nuh_layer_id great than 0, it is suggested that (4) parameters present in the VPS should not be duplicated in the SPS (except for those parameters that are required for base layer decoding), and (5) sequence-level parameters that are not present in the VPS should be included in the SPS.

Then, following the design criteria, it is proposed for HEVC extensions to introduce inter-layer SPS prediction to reduce SPS-related redundancy. This implies extending the definition of SPS in the extension specifications. In the proposed SPS prediction scheme, SPS parameters are inherited at the time of SPS activation. The reference SPS for prediction is determined as the active SPS of the reference layer, which is signaled in the referencing SPS. The parameters to be inherited are indicated by flags.

# Introduction

The scope of this contribution is to reduce SPS related bit overhead in HEVC extensions. The proposed solution is applicable for both 3D video and scalability extensions of HEVC. For multiview texture and depth coding, SPS parameters are very similar across views. Likewise, for spatial scalability or SNR scalability or potentially other forms of scalability, when different layers are associated with dedicated SPSs, the SPSs typically have high similarity across layers.

One approach for reducing SPS overhead is to re-use SPSs across layers, i.e. to activate the same SPS for more than one layer. The effectiveness of that approach is limited since even though many SPS parameters typically have identical values across layers, some SPS parameters still have different values. The number of redundant bits is typically in the order of few hundred bits per SPS.

This contribution proposes prediction of SPS parameters across layers. The proposed prediction mechanism uses a reference mechanism that is performed at SPS activation time. The syntax elements for a current SPS associated with a current layer are conditionally inferred from the active SPS of a referenced layer below the current layer. The presence of such layer references in the SPS is indicated by flags that are present for layers above the base layer (nuh_layer_id greater than 0).

# Criteria for the design of VPS and SPS for HEVC extensions

## General design criteria

The purpose of the VPS is to serve as an interface for systems to access information about the coded video sequence that is relevant for sub-bitstream extraction or capability exchange. In the HEVC base specification design, the VPS is mandatory to be present in a coded video sequence but not required to be parsed by decoders as it does not contain any information relevant for the decoding process that is not contained in the SPS. It is however anticipated that VPS parsing will be required in the decoding process of multi-layer HEVC extensions.

The following general criteria to be considered for the VPS/SPS design for HEVC extensions are suggested:

- The contents of a VPS should change infrequently

A VPS may contain sequence-level parameters that are relevant for the decoding of one or multiple layers. Thus activation of a new VPS requires that the decoding process is initialized, i.e. an IDR picture is present in all layers. Activation of a new SPS for a specific layer requires that an IDR picture is present for that layer; however it may not require an IDR picture to be present for all layers. As of the coding efficiency loss associated with the use of IDR pictures, if a parameter that is associated with a specific layer may change while other layers are unaffected, that parameter should preferably be contained in the SPS rather than the VPS.

- The amount of data associated with higher layers in the VPS should be small

After sub-bitstream extraction, the VPS is included in all extracted sub-bitstreams (as long as the base layer is part of the sub-bitstream). Any VPS parameters associated with higher layers are useless for the decoding of extracted sub-bitstreams and will thus have a negative impact on the coding efficiency of the sub-bitstreams. This is particularly relevant for applications with frequent in-band transmission of parameter sets (broadcast/multicast).

Re-writing the VPS upon sub-bitstream extraction sounds like a solution to the problem, however changing the VPS may require presence of IDR pictures for all layers which is undesirable in terms of coding efficiency (see criteria 1 above). Furthermore, VPS rewriting is potentially complex and may not be applicable in some applications.

- The size of parameter sets in general, and the VPS in particular, should be small

When transmitted out-of band, the size of parameter sets should be small such as to avoid bit overhead and thus delay in e.g. session setup. When the VPS in particular is used for capability exchange, it should be small to avoid bit overhead and thus delay in the capability exchange. Thus it is desirable that the amount of VPS parameters that are not needed for the capability exchange is minimized.

## Specific design criteria

Based on the general design criteria above, the following specific VPS design criteria are derived:

- Parameters that are useful for bitstream extraction and/or capability exchange should be included in the VPS.

- Parameters that are not required for bitstream extraction and/or capability exchange should not be present in the VPS.

- Sequence-level parameters that may change in some layers while not affecting the general layer structure should not be present in the VPS. Such parameters can be present in the SPS instead.

Complementary, the following SPS design criteria are derived:

- Parameters present in the VPS should not be duplicated in the SPS (except for those parameters that are required for base layer decoding).

- Sequence-level parameters that are not present in the VPS should be included in the SPS. This includes specifically:

  - Parameters that are not required for bitstream extraction and/or capability exchange.

  - Parameters that may change in one or few layers while not affecting the general layer structure.

  - Coding tool related parameters.

# SPS design with inter-layer SPS prediction

The proposed SPS design follows the design criteria suggested in Section 2. According to those criteria, the following syntax elements and structures are not included in the proposed SPS design if nuh_layer_id is greater than 0, since they are signalled in the VPS or VPS extension.

- sps_max_sub_layers_minus1

- sps_temporal_id_nesting_flag

- profile_tier_level( )

- sps_sub_layer_ordering_info_present_flag

- sps_max_dec_pic_buffering[ i ]

- sps_max_num_reorder_pics[ i ]

- sps_max_latency_increase[ i ]

The remaining parameters in the SPS are considered not useful for bitstream extraction and/or capability exchange and/or anticipated to be sequence-level parameters that may change in some layers while not affecting the general layer structure. Thus according to the design criteria above it is proposed that they remain in the SPS.

The remaining SPS parameters (excluding the sps_id, vps_id, and sps_extension fields) can be grouped into the following sets. The number of bits used for these parameters under common conditions (random access configuration) is given in square brackets […].

- representation format related parameters (picture size, chroma format, bit depth) [52]

- reference picture sets (reference picture signaling for short-term and long-term prediction) [140]

- video usability information (VUI) [1]

- coding tool related parameters (all other parameters) [29]

Some of those parameters may be different across layers (e.g. representation format), which then requires a separate SPS RBSP to be present for each layer that has specific sequence parameter values. In the resulting set of SPS RBSPs, some parameters (e.g. reference picture sets) may be identical, thus redundancy potentially occurs. In order to reduce such redundancy, an inter-layer SPS prediction scheme is proposed as follows.

## Proposed scheme

Proposed syntax and semantics as well as modified text for PPS/SPS activation are provided further below. The definitions are based on JCTVC-L1003_v34 and JCTVC-L1008. All changes are highlighted in yellow.

The proposed layer referencing mechanism is conditionally present in SPSs with nuh_layer_id greater than 0. The presence of SPS prediction is indicated using ref_layer_present_flag. If present, a reference to another SPS is signaled by means of a reference to a reference layer, using delta_ref_layer_id_minus1. Furthermore, the SPS parameters are grouped into three groups, and use of SPS prediction is indicated by flags (rep_format_inheritance_flag, coding_param_inheritance_flag, ref_pic_sets_inheritance_flag). If one of those flags is equal to 1, the SPS parameters associated with the respective group are omitted in the SPS, and SPS prediction is used instead.

If a reference to a reference layer is present in a given SPS, then during the activation of the SPS for a given layer, values of syntax elements that are not present in the given SPS are inferred from the active SPS in the reference layer, and thus remain in effect in the given layer.

## Syntax

The following replaces the contents of subclause F.7.3.2.2.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id == 0 ) { |  |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| sps_seq_parameter_set_id | ue(v) |
| if( nuh_layer_id > 0 ) { |  |
| ref_layer_present_flag | u(1) |
| if( ref_layer_present_flag = = 1 ) { |  |
| delta_ref_layer_id_minus1 | ue(v) |
| rep_format_inheritance_flag | u(1) |
| coding_param_inheritance_flag | u(1) |
| ref_pic_sets_inheritance_flag | u(1) |
| } |  |
| } |  |
| if( rep_format_inheritance_flag = = 0 ) { |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| } |  |
| if( coding_param_inheritance_flag = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| if( nuh_layer_id == 0 ) { |  |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |
| if( coding_param_inheritance_flag = = 0 ) { |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| } |  |
| if( ref_pic_sets_inheritance_flag = = 0 ) { |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( coding_param_inheritance_flag = = 0 ) { |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Semantics

The specifications in subclause F.7.4.3.2 apply with the following additions.

ref_layer_present_flag equal to 1 indicates that sequence parameter set prediction is used. If ref_layer_present_flag is not present, it is inferred to be equal to 0.

delta_ref_layer_id_minus1 + 1 specifies the difference of the value of nuh_layer_id and the value of nuh_layer_id for the reference layer used for sequence parameter set prediction. The value of delta_view_ref_layer_id_minus1 shall be in the range of 0 to nuh_layer_id - 1, inclusive.

rep_format_inheritance_flag equal to 1 indicates that representation format related parameters are inherited from the sequence parameter set active for the layer that has nuh_layer_id equal to nuh_layer_id - delta_ref_layer_id_minus1 - 1. If rep_format_inheritance_flag is not present, it is inferred to be equal to 0.

coding_param_inheritance_flag equal to 1 indicates that coding tool related parameters are inherited from the sequence parameter set active for the layer that has nuh_layer_id equal to nuh_layer_id - delta_ref_layer_id_minus1 - 1. If coding_param_inheritance_flag is not present, it is inferred to be equal to 0.

ref_pic_sets_inheritance_flag equal to 1 indicates that reference parameter sets are inherited from the sequence parameter set active for the layer identified by nuh_layer_id equal to nuh_layer_id - delta_ref_layer_id_minus1 - 1. If ref_pic_sets_inheritance_flag is not present, it is inferred to be equal to 0.

## PPS/SPS activation

The specifications in subclause F.7.4.2.4.2 apply with the following additions.

- When a SPS RBSP having nuh_layer_id equal to nuhLayerIdA is activated for a layer having nuh_layer_id equal to nuhLayerIdB and delta_ref_layer_id_minus1 is present in the SPS RBSP, the values of all SPS parameters for the layer having nuh_layer_id equal to nuhLayerIdA - delta_view_ref_layer_id_minus1 - 1 not present in the SPS RBSP that is activated for the layer having nuh_layer_id equal to nuhLayerIdB shall be considered in effect for the layer having nuh_layer_id equal to nuhLayerIdB.

- When a SPS RBSP having nuh_layer_id equal to nuhLayerIdA is active for the layer having nuh_layer_id equal to nuhLayerIdB and delta_ref_layer_id_minus1 is present in the SPS RBSP and the SPS RBSP active for the layer with nuh_layer_id equal to nuhLayerIdA - delta_view_ref_layer_id_minus1 - 1 is deactivated, then the SPS RBSP active for the layer having nuh_layer_id equal to nuhLayerIdB shall be deactivated by the activation of another SPS RBSP for that layer.

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).