| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0135 |
| --- | --- |

| Title: | On motion field compression in RefIdx mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Shan Liu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, shan.liu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In SHM-1.0 RefIdx mode, the decoded base layer (BL) picture is upsampled and put into the reference pictures list of the enhancement layer (EL) as the inter-layer reference picture (ILRP). The motion field of the ILRP is filled with the upsampled motion field from BL. To reduce the buffer size, the motion field of the ILRP is compressed with the unit size of 16x16 samples after the motion mapping. The center MV of each 16x16 block is used to represent the MV of the 16x16 block after compression. However, in HEVC motion field compression, the above-left MV of the 16x16 block is used to represent the MV of the 16x16 block. For unification, it is proposed to use the above-left MV of the 16x16 block to represent the MV of the 16x16 block for ILRP motion field compression. Simulation results reportedly show no coding efficiency loss or run time change caused by the proposed unification.

# Proposed method

In HEVC, the motion fields of the reference pictures are used for temporal collocated motion vector (MV) derivation. To reduce the buffer size, the motion fields are compressed with the unit size of 16x16 samples. For each 16x16 block, the above-left MV of the 16x16 block is used to represent the MV of the 16x16 block after motion field compression. In SHM-1.0 RefIdx mode, the motion field compression is also applied to the inter-layer reference picture (ILRP) [1]. However, the position of the representative MV is different. In this contribution, it is proposed to unify the position of the representative MV in motion field compression in HEVC and ILRP. The above-left MV of the 16x16 block is used to represent the MV of the 16x16 block for ILRP motion field compression.

# Simulation results

Experiments were conducted based on SHM-1.0 RefIdx mode. All the results were simulated according to the common test conditions defined in JCTVC-L1009 [2]. As shown in Table 1, the proposed motion field compression unification causes no coding efficiency loss or run time difference.

Table 1.  Results of the proposed motion field compression unification

# Conclusions

We propose to use the above-left MV of the 16x16 block to represent the MV of the 16x16 block for the inter-layer reference picture motion field compression, which can be regarded as unification with the temporal motion storage reduction in HEVC. No impact on coding efficiency or run time is observed.

# References

- J. Chen, V. Seregin, L. Guo, M. Karczewicz, “Non-TE5: on motion mapping in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0336, January, 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1009, January, 2013.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).