| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0136 |
| --- | --- |

| Title: | AHG5: 32x32 transform in chroma for 4:2:2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | V. Kolesnikov, C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 vladimir.kolesnikov@cisra.canon.com.au chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents an approach to the residual quad-tree for the 4:2:2 chroma format that allows a 32x32 transform to be used into the chroma channel.  At present, the 32x32 transform is used in chroma channels for the 4:4:4 chroma format.  It is asserted that reusing the transform logic required for the 4:4:4 chroma format in the 4:2:2 chroma format is beneficial.  Simulation results show a gains of 0.0%, -0.3% and 0.3% in luma, chroma for AI Main tier in 4:2:2, 0.0%, -0.4%, -0.4% in luma and chroma for RA Main and 0.0%, -0.7%, -0.9% in luma and chroma for LD Main.

# Problem statement

At the 12th JCT-VC meeting in Geneva, a proposal to use square transforms for the 4:2:2 chroma format was adopted [1].  According to the proposal in cases were where the chroma block had a height of 32, the chroma block was coded using multiple 16x16 transforms, thus reusing existing 16x16 transform logic already present for chroma processing.

We assert that hardware implementations frequently decouple the processing of the luma and the chroma channels into a ‘luma processing path’ and a ‘chroma processing path’. Thus, the implied hardware burden for implementers is separated between these two paths.

The required transforms for HM-10.0-RExt2.0 are listed in Table 1.

Table 1. Transform sizes for HM-10.0-RExt2.0  implementation.

| Luma | Chroma |  |  |
| --- | --- | --- | --- |
|  | 4:2:0 | 4:2:2 | 4:4:4 |
| 4x4 | 4x4 | 4x4 | 4x4 |
| 8x8 | 4x4 | 4x4 | 8x8 |
| 16x16 | 8x8 | 8x8 | 16x16 |
| 32x32 | 16x16 | 16x16 | 32x32 |

Table 1 indicates that a 32x32 transform is introduced into the chroma processing path for the 4:4:4 chroma format but is not used for other chroma formats.

Figure 1 shows a residual quad-tree with one split of a 64x64 coding unit into four 32x32 transform units, for the 4:2:2 chroma format. As can be seen, up to eight 16x16 transforms are used in each chroma channel to cover the 64x64 coding unit.

Figure 1. Residual quad-tree of 64x64 coding unit with one split.

We assert that this is undesirable, because the 32x32 transform is available for chroma due to the 4:4:4 chroma format but is not being used in the 4:2:2 chroma format.

# Description of the proposed change

For the 4:2:2 chroma format, the residual quad-tree (RQT) hierarchy is modified to include support using  32x32 transforms in chroma as shown in Figure 2.

Figure 2. Residual quad-tree of a 64x64 coding unit with one luma split and no chroma split.

When the RQT is present in a 64x64 CU and the partition mode is 2Nx2N, 32x32 transforms are used in chroma, as shown in Figure 2.  In other cases, an explicit ‘split_transform_flag’ is signalled at the 64x64 level of the RQT to specify if chroma should be split (as per Figure 1) or not split (as per Figure 2).

# Experiment Results

Results are presented in Table 2 below.  Simulations were performed under the common conditions [3]. Refer to the accompanying spreadsheet for results with greater precision.  Execution took place on a Linux cluster using GCC 3.4.6.

Context initialization values were not updated in this simulation.

Table 2. Results for 4:2:2 chroma format in HM-10.0-RExt2.0.

# Working Draft changes

Working draft changes are applied against L1005-v3 [3].  See file included in this archive.

# Conclusion

This contribution presents an approach to the residual quad-tree that enables 32x32 transforms, present in the design for the 4:4:4 chroma format, to also be used for the 4:2:2 chroma format.  Simulation results show a gains of 0.0%, -0.3% and 0.3% in luma, chroma for AI Main tier in 4:2:2, 0.0%, -0.4%, -0.4% in luma and chroma for RA Main and 0.0%, -0.7%, -0.9% in luma and chroma for LD Main.

# References

- J. Sole, R. Joshi, M. Karczewicz (Qualcomm), A. Gabriellini, M. Mrak (BBC), “Non-CE1: Square transform blocks for 4:2:2”, Joint Collaborative Team on Video Coding, JCTVC-L0351, Geneva, January 2013.

- D. Flynn, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions” Joint Collaborative Team on Video Coding, JCTVC-L1006, Geneva, January 2013.

- D. Flynn, J. Sole, T. Suzuki, et al, “High Efficiency Video Coding (HEVC) Range Extensions text specification:Draft 2 (for PDAM)”, Joint Collaborative Team on Video Coding, JCTVC-L1005, January 2013.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).