| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0137 |
| --- | --- |

| Title: | AHG5: Square transform deblocking for 4:2:2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, V. Kolesnikov 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au vladimir.kolesnikov@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

At the 12th JCT-VC meeting, square transforms were adopted for the rectangular blocks present in the chroma channels when the 4:2:2 chroma format is used.  This adoption results in boundaries between the pairs of square transforms that are not deblocked.  It is asserted that having transform boundaries in the design that are not deblocked is inconsistent with the 4:2:0 design and could lead to deblocking artefacts.  This contribution introduces deblocking to the boundaries between the square transforms in chroma when the 4:2:2 chroma format is in use.  The simulation results show 0.0%, -0.2% and -0.2% gains in luma and chroma for AI Main tier, 0.0%, -0.1% and -0.2% gains in luma and chroma for RA Main tier and 0.0%, -0.1% and -0.1% gains in luma and chroma for LB Main tier.

# Problem Statement

At the 12th JCT-VC meeting in Geneva, a proposal to use square transforms for the chroma channels for the 4:2:2 chroma format was adopted [1].

When square transforms are used, additional transform boundaries are created, between the square transforms, which are not deblocked in the present design.  Transform boundaries without deblocking could result in reduced subjective performance. Transform boundaries without deblocking is also inconsistent with the design of 4:2:0, where deblocking is always applied on transform boundaries.

# Description of the proposed change

For the 4:2:2 chroma format, deblocking is introduced for the boundaries between the square transforms used in chroma (see Figure 1). The light blue lines are already deblocked and the dark blue (dashed) lines are the additional chroma boundaries that are deblocked as a result of this proposal.

The conditions under which chroma deblocking occur are not altered with this proposal (i.e. intra only boundaries).

This method was presented at the 12th JCT-VC meeting [4].

Figure 1. Deblocking of square transforms in HM-10.0-RExt2

# Experiment Results

Results are presented in Table 1 below. Simulations were performed under the Range extensions common conditions [2]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Table 1. Simulation result of square transform chroma deblocking for 4:2:2.

# Working Draft changes

Working draft changes are applied against L1005-v3 [3].  See file included in archive.

# Conclusion

This contribution presents an approach for deblocking of the boundaries between square transforms resulting from splitting the rectangular chroma regions in the 4:2:2 chroma format into square regions and using a square transform for each region.  The simulation results show 0.0%, -0.2% and -0.2% gains in luma and chroma for AI Main tier, 0.0%, -0.1% and -0.2% gains in luma and chroma for RA Main tier and 0.0%, -0.1% and -0.1% gains in luma and chroma for LB Main tier

# References

- J. Sole, R. Joshi, M. Karczewicz (Qualcomm), A. Gabriellini, M. Mrak (BBC), “Non-CE1: Square transform blocks for 4:2:2”, Joint Collaborative Team on Video Coding, JCTVC-L0351, Geneva, January, 2013.

- D. Flynn, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions” Joint Collaborative Team on Video Coding, JCTVC-L1006, Geneva, January, 2012.

- D. Flynn, J. Sole, T. Suzuki, et al, “High Efficiency Video Coding (HEVC) Range Extensions text specification:Draft 2 (for PDAM)”, Joint Collaborative Team on Video Coding, JCTVC-L1005, January 2013.

- C. Rosewarne, V. Kolesnikov, M. Maeda, “Non-CE1: Test 3 - Square transform deblocking for 4:2:2”, Joint Collaborative Team on Video Coding, JCTVC-L150, January 2013.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).