| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0138 |
| --- | --- |

| Title: | AHG5: On chroma intra prediction for 4:2:2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

Chroma intra-prediction in the 4:2:2 chroma format is performed on rectangular blocks and thus an adjustment to the intra-prediction angle is used so that the resulting angle corresponds to the angle used for luma blocks.  The adjustment results in new intra-prediction angles being used (effectively new modes), that are not used in luma or in non-4:2:2 chroma formats.  This contribution proposes to alter the mapping between intra-prediction modes and intra-prediction angles so that no new modes are introduced.

# Problem statement

An adjustment is made to the ‘invAngle’ and ‘intraPredAngle’ parameters for chroma intra-prediction for the 4:2:2 chroma format.  There are two aspects to this adjustment.  Firstly, the parameters are adjusted to compensate for the rectangular block shape.  Secondly, for particular intra-prediction modes the parameters are restricted to prevent access to reference samples outside the defined region.  The mapping table between the intra-prediction mode and the parameters is shown in Table 1.

Table 1. Intra-prediction parameters mapping in HM-10.0-RExt2.0.

|  | predModeIntra | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| !4:2:2 ¦¦ luma | intraPredAngle | . | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | -2 | -5 | -9 | -13 | -17 | -21 | -26 |
|  | invAngle | . | . | . | . | . | . | . | . | . | . | -4096 | -1638 | -910 | -630 | -482 | -390 | -315 |
| 4:2:2 && chroma | intraPredAngle | . | 32 | 32 | 32 | 32 | 26 | 18 | 10 | 4 | 0 | -4 | -10 | -18 | -26 | -32 | -32 | -32 |
|  | invAngle | . | . | . | . | . | . | . | . | . | . | -2048 | -819 | -455 | -315 | -256 | -256 | -256 |
|  | predModeIntra | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| !4:2:2 ¦¦ luma | intraPredAngle | -32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |
|  | invAngle | -256 | -315 | -390 | -482 | -630 | -910 | -1638 | -4096 | . | . | . | . | . | . | . | . | . |
| 4:2:2 && chroma | intraPredAngle | -16 | -13 | -11 | -9 | -7 | -5 | -3 | -1 | 0 | 1 | 2 | 4 | 6 | 8 | 10 | 13 | 16 |
|  | invAngle | -512 | -630 | -780 | -964 | -1260 | -1820 | -3276 | -8192 | . | . | . | . | . | . | . | . | . |

As can be seen in Table 1, the parameter adjustment results in effectively new intra-prediction modes that do not exist in the 4:2:0 design.

We assert that these new intra-prediction modes increase implementation complexity and result in a design that further departs from the 4:2:0 design.

# Description of the proposed change

Method 1.

To remove the new intra-prediction modes, this contribution proposes to ‘quantise’ the resulting intra-prediction parameters for the new modes to the closest pre-existing intra-prediction parameters.  The proposed mapping table is shown in Table 2  (changes with respect to Table 1 are highlighted).

Table 2. Method 1 – proposed intra-prediction parameters mapping.

|  | predModeIntra | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| !4:2:2 ¦¦ luma | intraPredAngle | . | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | -2 | -5 | -9 | -13 | -17 | -21 | -26 |
|  | invAngle | . | . | . | . | . | . | . | . | . | . | -4096 | -1638 | -910 | -630 | -482 | -390 | -315 |
| 4:2:2 && chroma | intraPredAngle | . | 32 | 32 | 32 | 32 | 26 | 17 | 9 | 5 | 0 | -5 | -9 | -17 | -26 | -32 | -32 | -32 |
|  | invAngle | . | . | . | . | . | . | . | . | . | . | -1638 | -910 | -482 | -315 | -256 | -256 | -256 |
|  | predModeIntra | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| !4:2:2 ¦¦ luma | intraPredAngle | -32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |
|  | invAngle | -256 | -315 | -390 | -482 | -630 | -910 | -1638 | -4096 | . | . | . | . | . | . | . | . | . |
| 4:2:2 && chroma | intraPredAngle | -17 | -13 | -9 | -9 | -5 | -5 | -2 | -2 | 0 | 2 | 2 | 5 | 5 | 9 | 9 | 13 | 17 |
|  | invAngle | -482 | -630 | -910 | -910 | -1638 | -1638 | -4096 | -4096 | . | . | . | . | . | . | . | . | . |

Method 2.

In method 1, the adjustment performed for intra-prediction modes 15-17 results in parameters corresponding to intra-prediction mode 18.  Using the mapping table approach, it is possible to remap these parameters to parameters corresponding to intra-prediction modes greater than 18.  In this approach, a threshold between the predominantly vertical modes and predominantly horizontal modes will be 15 (instead of 18).

The resulting mapping table is shown in Table 5 (changes with respect to Table 2 are highlighted).

Table 3. Method 2 – proposed intra-prediction parameters mapping.

|  | predModeIntra | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| !4:2:2 ¦¦ luma | intraPredAngle | . | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | -2 | -5 | -9 | -13 | -17 | -21 | -26 |
|  | invAngle | . | . | . | . | . | . | . | . | . | . | -4096 | -1638 | -910 | -630 | -482 | -390 | -315 |
| 4:2:2 && chroma | intraPredAngle | . | 32 | 32 | 32 | 32 | 26 | 17 | 9 | 5 | 0 | -5 | -9 | -17 | -26 | -32 | -26 | -21 |
|  | invAngle | . | . | . | . | . | . | . | . | . | . | -1638 | -910 | -482 | -315 | -256 | -315 | -390 |
|  | predModeIntra | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| !4:2:2 ¦¦ luma | intraPredAngle | -32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |
|  | invAngle | -256 | -315 | -390 | -482 | -630 | -910 | -1638 | -4096 | . | . | . | . | . | . | . | . | . |
| 4:2:2 && chroma | intraPredAngle | -17 | -13 | -9 | -9 | -5 | -5 | -2 | -2 | 0 | 2 | 2 | 5 | 5 | 9 | 9 | 13 | 17 |
|  | invAngle | -482 | -630 | -910 | -910 | -1638 | -1638 | -4096 | -4096 | . | . | . | . | . | . | . | . | . |

# Experiment Results

Results are presented in Table 4 and Table 5 below. Simulations were performed under the common conditions [3]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Context initialization values were not updated in this simulation.

Table 4. Method 1 results for the 4:2:2 chroma format.

Table 5. Method 2 results for the 4:2:2 chroma format.

# Working Draft changes

See file included in this archive.

# Conclusion

This contribution presented a method for setting the intra-prediction parameters (intraPredangle and invAngle) for chroma when the 4:2:2 chroma format is in use.  This method sets the intra-prediction parameters such that no new intra-prediction parameters are required to be supported by the reference sample generation and intra-prediction processes.  The proponents assert that this results in a complexity reduction.

# References

- J. Sole, R. Joshi, M. Karczewicz, A. Gabriellini, M. Mrak, “Non-CE1: Square transform blocks for 4:2:2”, Joint Collaborative Team on Video Coding, JCTVC-L0351, January 2013.

- D. Flynn, J. Sole, T. Suzuki, et al, “High Efficiency Video Coding (HEVC) Range Extensions text specification:Draft 2 (for PDAM)”, Joint Collaborative Team on Video Coding, JCTVC-L1005, January 2013.

- D. Flynn, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions”, Joint Collaborative Team on Video Coding, JCTVC-L1006, January 2013.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).