| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0139 |
| --- | --- |

| Title: | Non SCE1 : Inter-layer Intra Mode Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuguang Zuo, Lu Yu Room 401, Xindian Building，No.38, Zheda Road, Hangzhou Zhejiang 310027 | Email: | baishanfuxin@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

The contribution presents a method to exploit the correlation of intra modes from different layers to improve the coding efficiency of enhancement layer (EL) in SHVC. It uses both the intra modes of EL neighbor s and the co-located base layer (BL) block to predict that of EL block by adding them into the EL MPM list (denoted as MPM[x] with x=0, 1, 2). Two methods of how to rank these intra modes in the MPM list is introduced in this contribution. It is reported that 0.5% and 0.1% gain can be achieved for enhancement layer stream in AI 2x and AI1.5x cases respectively with both methods.

# Introduction

In SHVC, there is a large correlation between intra modes of EL block and EL neighbors and BL co-located block . So it will be more beneficial to add the intra modes of EL neighbors and BL co-located block to the MPM list. Several proposals of using such intra prediction mode correlation are studied in SCE1 Section 4.3.[1].

Also in SHVC, a new IntraBL mode is introduced to EL block, and the new IntraBL mode is treated as non-intra coded in SHM 1, when either left or above EL neighbour block is IntraBL mode, it is tagged as “unavailable” and treated as DC mode in MPM derivation. The operation may decrease the efficiency of the use of EL neighbors if they would be intra mode without IntraBL mode introduced.

Accordingly, this contribution uses the method of changing the intra mode of left or above neighbor according to BL blocks when they are IntraBL mode. Then it proposes to use intra mode from co-located BL block for MPM derivation.

# Algorithm Description

As shown in FIG.1, the modes of the left neighbor, above neighbor and BL co-located block of EL block are tagged as ModeLeft, ModeTop and ModeBL. The proposed method introduces how to derive the MPM list of EL block with ModeLeft, ModeTop and ModeBL.

FIG. 1

Method1:

Step1: When left or above neighbour is IntraBL mode. if the co-located BL CU of left or above neighbour is intra coded, the intra mode of the co-located BL block of left or above neighbour will be assigned to ModeLeft or ModeTop[2].

Step2: Derive the MPM list of EL block with ModeLeft, ModeTop and ModeBL:

- When ModeBL is intra and is not planar/dc, it is used to derive the MPM list of the enhancement layer block as follows:

- If all of ModeLeft, ModeTop and ModeBL are the same, signed as MODE3:

MPM[0] = MODE3,

MPM[1] = MODE3 + 1,

MPM[2] = MODE3 - 1.

- If two of ModeLeft, ModeTop and ModeBL are the same, signed as MODE2, the rest different one is signed as MODE1:

  - If MODE2 > 1, then MPM[0] = MODE2,

MPM[1] = MODE2 + 1

MPM[2] = MODE2 - 1.

  - Otherwise, MPM[0] = MODE2,

MPM[1] = MODE1

MPM[2] = ( MODE1 > 1) ? !MODE2 : vertical

- If ModeLeft, ModeTop and ModeBL are different from each other,

MPM[0] = ModeBL,

MPM[1] = ModeLeft

MPM[2] = ModeTop.

- Otherwise, derive the MPM list the same way as the base layer.

Method 2:

Step1: When left or above neighbour is IntraBL mode. if the co-located BL CU of EL block is coded with an angular intra mode, the intra mode of the co-located BL block of EL block will be assigned to ModeLeft or ModeTop[2].

Step2: Derive the MPM list of EL block with ModeLeft, ModeTop and ModeBL:

- When ModeBL is intra and is not planar/dc, it is used to derive the MPM list of the enhancement layer block as follows:

- If all of ModeLeft, ModeTop and ModeBL are the same, signed as MODE3:

MPM[0] = MODE3,

MPM[1] = MODE3 + 1,

MPM[2] = MODE3 - 1.

- If two of ModeLeft, ModeTop and ModeBL are the same, signed as MODE2, the rest different one is signed as MODE1:

  - If MODE2 > 1, then MPM[0] = MODE2,

MPM[1] = MODE2 + 1

MPM[2] = MODE2 - 1.

  - Otherwise, MPM[0] = MODE2,

MPM[1] = MODE1

MPM[2] = ( MODE1 > 1) ? !MODE2 : vertical

- If ModeLeft, ModeTop and ModeBL are different from each other,

MPM[0] = ModeBL,

MPM[1] = ModeLeft

MPM[2] = ModeTop.

- Otherwise, derive the MPM list the same way as the base layer.

# Simulation results

The proposed solution is implemented in SHM1 and tested using All Intra configurations suggested in SCE1 [1]. The simulation results are summarized in , 2, 3, 4. Detailed results are available in accompanying excel files. The encoding time is not accurate because of the different output config. The decoding time is accurate.

Table 1 Summary of simulation results of Method 1

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.1% | -0.1% |  |  |  |
| Class B | -0.4% | -0.2% | -0.3% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.332% | -0.205% | -0.210% | -0.105% | 0.034% | 0.002% |
| Overall (Test vs single layer) | 12.044% | 13.411% | 13.134% | 10.188% | 10.325% | 9.708% |
| EL only (Test vs Ref) | -0.480% | -0.348% | -0.357% | -0.080% | 0.078% | 0.038% |
| Enc Time[%] | 107.4% |  |  | 82.3% |  |  |
| Dec Time[%] | 102.6% |  |  | 100.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2 Summary of simulation results of Method 1 with MDCS OFF

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.198% | -0.021% | -0.002% |  |  |  |
| Class B | -0.222% | -0.072% | -0.035% | -0.104% | 0.008% | 0.014% |
| Overall (Test vs Ref) | -0.215% | -0.057% | -0.025% | -0.104% | 0.008% | 0.014% |
| Overall (Test vs single layer) | 12.178% | 13.587% | 13.356% | 10.190% | 10.295% | 9.721% |
| EL only (Test vs Ref) | -0.338% | -0.165% | -0.136% | -0.100% | 0.027% | 0.033% |
| Enc Time[%] | 131.1% |  |  | 71.4% |  |  |
| Dec Time[%] | 107.2% |  |  | 100.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 3 Summary of simulation results of Method 2

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.291% | -0.145% | -0.082% |  |  |  |
| Class B | -0.393% | -0.266% | -0.295% | -0.107% | 0.031% | 0.006% |
| Overall (Test vs Ref) | -0.363% | -0.231% | -0.234% | -0.107% | 0.031% | 0.006% |
| Overall (Test vs single layer) | 12.009% | 13.380% | 13.104% | 10.187% | 10.320% | 9.712% |
| EL only (Test vs Ref) | -0.519% | -0.383% | -0.390% | -0.083% | 0.074% | 0.043% |
| Enc Time[%] | 87.7% |  |  | 63.6% |  |  |
| Dec Time[%] | 104.7% |  |  | 99.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 4 Summary of simulation results of Method 2 with MDCS OFF

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.229% | -0.050% | -0.015% |  |  |  |
| Class B | -0.249% | -0.097% | -0.065% | -0.106% | 0.009% | 0.017% |
| Overall (Test vs Ref) | -0.243% | -0.084% | -0.051% | -0.106% | 0.009% | 0.017% |
| Overall (Test vs single layer) | 12.147% | 13.556% | 13.326% | 10.188% | 10.296% | 9.723% |
| EL only (Test vs Ref) | -0.371% | -0.199% | -0.168% | -0.103% | 0.027% | 0.035% |
| Enc Time[%] | 89.1% |  |  | 57.4% |  |  |
| Dec Time[%] | 103.7% |  |  | 99.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

In this contribution, MPM derivation using BL intra prediction mode is proposed and BD-rate saving is reported by simulation. It is recommended to further investigate the proposed solution in CE or AHG.

# Reference

- Ali Tabatabai, Krishna Rapaka, Ankur Saxena and Shan Liu, “Description of Core Experiment SCE 1: Intra Prediction Improvements in SHVC”, JCTVC-L1101, Geneva, January 2013.

- Jun Xu, Ali Tabatabai, “Non-TE5: Inter-layer Intra MPM derivation for EL in SHVC”, JCTVC-L0220, Geneva, January 2013.

# Working draft changes

## G8.4.x Derivation process for luma intra prediction mode

Input to this process is a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

In this process, the luma intra prediction mode IntraPredModeY[ xPb ][ yPb ] is derived.

Table 8-3 specifies the value for the intra prediction mode and the associated names.

Table 8-3 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..34 | INTRA_ANGULAR2..INTRA_ANGULAR34 |

IntraPredModeY[ xPb ][ yPb ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1.

Figure 8-1 – Intra prediction mode directions (informative)

IntraPredModeY[ xPb ][ yPb ] is derived by the following ordered steps:

- candIntraPredModeBL is derived as follows:

- The specification in clause Gxx applies to derive the luma position of the reference layer picture (xPbRef, yPbRef) with (xPb, yPb).

- If CuPredMode[xPbRef][yPbRef] of reference layer picture is Mode_Intra, candIntraPredModeBL is set equal to IntraPredModeY[xPbRef][yPbRef] of reference layer picture

- Otherwise,

candIntraPredModeBL is set to -1.

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xPb − 1, yPb ) and ( xPb, yPb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the location ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If availableX is equal to FALSE, candIntraPredModeX is set equal to INTRA_DC.

- Otherwise, if CuPredMode[ xNbX ][ yNbX ] is equal to MODE_TEXTURE_RL and candIntraPredModeBL >1 , candIntraPredModeBL is assigned to candIntraPredModeX,

- Otherwise, if CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC,

- Otherwise, if X is equal to B and yPb − 1 is less than ( ( yPb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), candIntraPredModeB is set equal to INTRA_DC.

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..2 is derived as follows:

- If candIntraPredModeBL > 1 and nuh_layer_id greater than 0, the candModeList[ x ] with x = 0..2 is derived as follows:

- If candIntraPredModeA =  candIntraPredModeB = candIntraPredModeBL

candModeList[ 0 ] = candIntraPredModeA	(8-15)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(8-16)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(8-17)

- If two of candIntraPredModeA, candIntraPredModeB, candIntraPredModeBL are the same,

  - If candIntraPredModeA = candIntraPredModeBL

candModeList[ 0 ] = candIntraPredModeA	(8-1)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(8-2)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(8-3)

  - If candIntraPredModeB = candIntraPredModeBL

candModeList[ 0 ] = candIntraPredModeB	(8-21)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeB + 29 ) % 32 )	(8-22)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeB − 2 + 1 ) % 32 )	(8-23)

  - If candIntraPredModeA = candIntraPredModeB

--If candIntraPredModeA > 1

candModeList[ 0 ] = candIntraPredModeA	(8-24)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(8-25)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(8-26)

--Otherwise

candModeList[ 0 ] = candIntraPredModeA	(8-27)

candModeList[ 1 ] = candIntraPredModeBL	(8-28)

candModeList [2] = (candModeList[ 1 ] > 1) ? ! candModeList[ 0 ]: vertical	(8-29)

- If candIntraPredModeA, candIntraPredModeB , candIntraPredModeBL are different from each other.

candModeList[ 0 ] = candIntraPredModeBL	(8-30)

candModeList[ 1 ] = candIntraPredModeA	(8-31)

candModeList[ 2 ] = candIntraPredModeB	(8-32)

- Otherwise, the candModeList[ x ] with x = 0..2 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

  - If candIntraPredModeA is less than 2 (i.e. equal to INTRA_PLANAR or INTRA_DC), candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = INTRA_PLANAR	(8-33)

candModeList[ 1 ] = INTRA_DC		(8-34)

candModeList[ 2 ] = INTRA_ANGULAR26	(8-35)

  - Otherwise, candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-36)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(8-37)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(8-38)

- Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

  - candModeList[ 0 ] and candModeList[ 1 ] are derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-39)

candModeList[ 1 ] = candIntraPredModeB	(8-40)

  - If neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_PLANAR, candModeList[ 2 ] is set equal to INTRA_PLANAR,

  - Otherwise, if neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_DC, candModeList[ 2 ] is set equal to INTRA_DC,

  - Otherwise, candModeList[ 2 ] is set equal to INTRA_ANGULAR26.

- IntraPredModeY[ xPb ][ yPb ] is derived by applying the following procedure:

- If prev_intra_luma_pred_flag[ xPb ][ yPb ] is equal to 1, the IntraPredModeY[ xPb ][ yPb ] is set equal to  candModeList[ mpm_idx ].

- Otherwise, IntraPredModeY[ xPb ][ yPb ] is derived by applying the following ordered steps:

- The array candModeList[ x ], x = 0..2 is modified as the following ordered steps:

  - When candModeList[ 0 ] is greater than candModeList[ 1 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 1 ] ) = Swap( candModeList[ 0 ], candModeList[ 1 ] )	(8-41)

  - When candModeList[ 0 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 2 ] ) = Swap( candModeList[ 0 ], candModeList[ 2 ] )	(8-42)

  - When candModeList[ 1 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 1 ], candModeList[ 2 ] ) = Swap( candModeList[ 1 ], candModeList[ 2 ] )	(8-43)

- IntraPredModeY[ xPb ][ yPb ] is derived by the following ordered steps:

  - IntraPredModeY[ xPb ][ yPb ] is set equal to rem_intra_luma_pred_mode[ xPb ][ yPb ].

  - For i equal to 0 to 2, inclusive, when IntraPredModeY[ xPb ][ yPb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xPb ][ yPb ] is incremented by one.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).