| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0140r1 |
| --- | --- |

| Title: | AHG9: Inter-layer RPS Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuo Lu, Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Shuo.Lu@jp.sony.com Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

HEVC employs RPS (Reference Picture Set) to manage DPB (decoded picture buffer).

In many cases, scalable layers are coded using the same GOP structure, and inter-layer RPS prediction is proposed by JCTVC-L0231.

We support the basic idea of the proposal, but it contains the following problems:

- Only in slice header –In Version 1 RPS is transmitted both in SPS & slice header

- One flag for both Short-Term and Long-Term Ref Frames –In Ref_idx Framework base layer images are marked as long term ref which means base layer and enhancement layer cannot have same long term ref set.

- used_by_curr_pic_flag should not be predicted –Enhancement layer may have less numbers of DPBs although GOP structure is same as Base layer

- In AVC-baselayer case prediction is not possible but not considered

This document proposes syntax to solve the problems stated above.

# Introduction / Problem Statement / …

Inter-layer RPS prediction has been proposed by JCTVC-L0231.However it contains the following problems:

- Only in slice header –In Version 1 RPS is transmitted both in SPS & slice header

- One flag for both Short-Term and Long-Term Ref Frames – In Ref_idx Framework base layer images are marked as long term ref.

- used_by_curr_pic_flag should not be predicted –Enhancement layer may have less numbers of DPBs although GOP structure is same as Base layer, since resolution of enhancement layer pictures may be 2 times or 1.5 times bigger than base layer. Accordingly the necessary memory size for DPB is increased. Even under same GOP structure, different choices of ref pictures should be supported. The different layers can reduce or increase ref pictures respectively.

- In AVC-baselayer case prediction is not possible but not considered

# Proposal

Variations of proposed syntax are shown from 2.1 to 2.5.

## Proposal 1

The following changes in syntax are proposed to support inter layer RPS prediction in both SPS and slice header.

Table 2.1.1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| inter_layer_copy_flag | u(1) |
| if(!inter_layer_copy_flag){ |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ){ |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |

Table 2.1.2

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| pic_order_cnt_lsb | u(v) |
| inter_layer_copy_flag | u(1) |
| if (! inter_layer_copy_flag){ |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| ...... |  |
| } |  |
| } |  |

inter_layer_copy_flag equals to 1 indicates that in current layer both short term and long term rps information are inherited directly from its referred layer. 0 indicates that both short term and long term rps information are transmitted exclusively for current layer. When not specified or the reference layer is the base layer and avc_base_layer_flag in vps_extension() is 1, its value shall be 0.

## Proposal 2

The following syntax is proposed to support more flexible short-term RPS prediction between layers. Inside RPS, the used_by_curr_pic_flag will be transmitted exclusively to allow enhancement layer and referenced base layer to have same GOP structure while different DPB size.

Table 2.2.1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| inter_layer_prediction_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i, inter_layer_prediction_flag ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ){ |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

Table 2.2.2

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| pic_order_cnt_lsb | u(v) |
| inter_layer_prediction_flag | u(1) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets, inter_layer_prediction_flag ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| ...... |  |

Table 2.2.3

| short_term_ref_pic_set( stRpsIdx, inter_layer_prediction ) { | Descriptor |
| --- | --- |
| if( stRpsIdx != 0 ) |  |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| if(!inter_layer_rps_predition){ |  |
| if( stRpsIdx = = num_short_term_ref_pic_sets ) |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| } |  |
| for( j = 0; j <= NumDcs[ RefRpsIdx ]; j++ ) { |  |
| used_by_curr_pic_flag[ j ] | u(1) |
| if( !used_by_curr_pic_flag[ j ] ) |  |
| use_delta_flag[ j ] | u(1) |
| } else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

inter_layer_prediction_flag equals to 1 indicates that when short term rps prediction is enabled in current layer , RPS parameters delta_rps_sign, delta_rps_sign and abs_delta_rps_minus1 are inherited directly from the reference layer. When not specified or the reference layer is the base layer and avc_base_layer_flag in vps_extension() is 1, its value shall be 0.

## Proposal 3

The following syntax is proposed to enable same functionality as 2.2 but allow changing the flag by each of the reference picture.

Table 2.3.1

| short_term_ref_pic_set( stRpsIdx ) { | Descriptor |
| --- | --- |
| if( stRpsIdx != 0 ) |  |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| inter_layer_rps_predition_flag | u(1) |
| if(!inter_layer_rps_predition_flag){ |  |
| if( stRpsIdx = = num_short_term_ref_pic_sets ) |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| } |  |
| for( j = 0; j <= NumDcs[ RefRpsIdx ]; j++ ) { |  |
| used_by_curr_pic_flag[ j ] | u(1) |
| if( !used_by_curr_pic_flag[ j ] ) |  |
| use_delta_flag[ j ] | u(1) |
| } else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

inter_layer_prediction_flag equals to 1 indicates that when short term rps prediction is enabled in current layer, parameters delta_rps_sign, delta_rps_sign and abs_delta_rps_minus1 inherited directly from its referred layer. When not specified or the reference layer is the base layer and avc_base_layer_flag in vps_extension() is 1, its value shall be 0.

## Proposal 4.1

The following syntax are proposed to enable inter layer prediction for short-term and long-term RPS separately both in SPS and slice header.

Table 2.4.1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| inter_layer_short_copy_flag | u(1) |
| if(!inter_layer_short_copy_flag){ |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| } |  |
| long_term_ref_pics_present_flag | u(1) |
| inter_layer_long_copy_flag | u(1) |
| if(!inter_layer_long_copy_flag){ |  |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ){ |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |

Table 2.4.2

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| pic_order_cnt_lsb | u(v) |
| inter_layer_short_copy_flag | u(1) |
| if(!inter_layer_short_copy_flag){ |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| } |  |
| if( long_term_ref_pics_present_flag ) { |  |
| inter_layer_long_copy_flag |  |
| if(!inter_layer_long_copy_flag){ |  |
| ...... |  |
| } |  |
| } |  |

inter_layer_short_copy_flag equals to 1 indicates that in current layer short term RPS information are inherited directly from the reference layer. 0 indicates that in current layer short term RPS information are transmitted. When not specified or the reference layer is the base layer and avc_base_layer_flag in vps_extension() is 1, its value shall be 0.

inter_layer_long_copy_flag equals to 1 indicates that in current layer long term RPS information are inherited directly from the reference layer. 0 indicates that in current layer long term RPS information are transmitted. When not specified or the reference layer is the base layer and avc_base_layer_flag in vps_extension() is 1, its value should be 0. When current scalable framework is refIdex-based, the value shall be 0.

## Proposal 4.2

The long term RPS prediction should be avoided when refIdx framework is applied. “rfpidx_scalaility” represents syntax element [to be defined somewhere like vps_extension] that indicates current framework is refidx-based.

Table 2.5.1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| inter_layer_copy_flag | u(1) |
| if(!inter_layer_copy_flag){ |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( I = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| } |  |
| if(!inter_layer_copy_flag ¦¦ rpfidx_scalability){ |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ){ |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |

Table 2.5.2

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| pic_order_cnt_lsb | u(v) |
| inter_layer_copy_flag | u(1) |
| if (! inter_layer_copy_flag){ |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| } |  |
| if(! inter_layer_copy_flag ¦¦ rpfidx_scalability){ |  |
| if( long_term_ref_pics_present_flag ) { |  |
| ...... |  |
| } |  |
| } |  |

inter_layer_copy_flag equals to 1 indicates that in current layer both short term and long term RPS information are inherited directly from the reference layer. 0 indicates that both short term and long term RPS information are transmitted explicitly for the current layer. When not specified or the reference layer is the base layer and avc_base_layer_flag in vps_extension() is 1, its value shall be 0.

- Conclusion

Inter-layer RPS prediction is proposed by JCTVC-L0231. We support the basic idea of the proposal, but c the following problems are claimed:

-	Only in slice header –In Version 1 RPS is transmitted both in SPS & slice header

-	One flag for both Short-Term and Long-Term Ref Frames –In Ref_idx Framework Long-Term Ref Frame is used for Base layer Images

-	Used_by_curr_pic_flag should not be predicted –Enhancement layer may have less numbers of DPBs although GOP structure is same as Base layer

-	In AVC-baselayer case prediction is not possible but not considered

In this document variations of the proposed syntax to solve the problems stated above is shown. It is recommended that the proposed modifications be adopted into SHVC draft.

# References

[1] “Inter-layer slice header syntax element prediction in SHVC”, JCTVC-L0231, January 2013

[2] “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent)”, JCTVC-L1003, January 2013

[3] “SHVC Test Model 1 (SHM 1)”, JCTVC-L1007, January 2013

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).