| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0141 |
| --- | --- |

| Title: | SCE5: Result of SCE5.2.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan Jun Xu 1730 N. First Street San Jose, CA 95112, USA Joonyoung Park, Byeongmoon Jeon 19, Yangjae-daero 11gil, Seocho-gu, Seoul 137-130, Korea | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com +1-408-352-4086 jun.xu@am.sony.com +82-10-6773-9179 {jy.park,bm.jeon}@lge.com |
| Source: | Sony Corp. / Sony Electronics, LG |  |  |

_____________________________

# Abstract

Minimum inter-PU size is 4x8 or 8x4 in HEVC version 1, but for temporal motion vector prediction motion information is stored by 16x16-unit to reduce buffer size. In current SHM same decimation ratio is applied for co-located BL motion information. However, especially with 2x spatial scalability this decimation ratio is too coarse and will cause loss in coding efficiency.

There are 2 proposals on this issue, one method as proposed by JCTVC-L0070 is to postpone motion data compression after encoding/decoding of the enhancement layer so that uncompressed motion data of the base layer is used for enhancement layer.

The other method proposed by JCTVC-L0279 is first to compress by 2:1 after encoding/decoding of base layer and then by 2:1 after encoding/decoding of the enhancement layer.

Under the tests of SCE5.2.2, impact on coding efficiency and complexity with these proposals related to motion data compression has been studied.

# Proposed Algorithm

As shown in Fig 1.1(a), motion data compression by 4:1 is executed after encoding/decoding of the base layer in current SHM. Especially in the case of 2x spatial scalability 4:1 compression at the base layer means that 8:1 compression at the enhancement layer, and causes loss in coding efficiency.

There have been 2 proposals to solve this problem: one method as proposed by JCTVC-L0070 is to postpone motion data compression after encoding/decoding of the enhancement layer [Fig 1.1(b)]. With this method uncompressed motion data of the base layer is used for enhancement layer.

The other method proposed by JCTVC-L0279 is first to compress by 2:1 after encoding/decoding of base layer and then by 2:1 after encoding/decoding of the enhancement layer [Fig 1.1(c)].

Fig 1.1

# Simulation

## Simulation Condition

SHM-1.0 is used as the anchor. On top of this anchor software the proposed methods have been implemented by the author.

The followings are macro settings in “Typedef.h”:

1:1 Comp@EL:

#define SCE522_MOTION_COMP		1

#define HALF_COMP			0

2:1 Comp@EL

#define SCE522_MOTION_COMP		1

#define HALF_COMP			1

Class A and B sequences have been tested with {RA, LP}_{2x,1.5x,SNR} conditions.

## Simulation Result

Summary of the results are shown in Table 2.2.1 (1:1 Comp@EL) and Table 2.2.2 (2:1 Comp@EL) respectively. Complete results are shown in JCTVC-M0141_1_1.xls and JCTVC-M0141_1_2.xls, respectively.

As can be seen gain by -1.1%, -0.5%, -0.5%, -0.9%, -0.4% and -0.6% is obtained with 1:1 compression at the enhancement layer with RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively. Gain by -0.8%, -0.4%, -0.4%, -0.6%, -0.3% and -0.5% is obtained with 2:1 compression at the enhancement layer with RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively.

Discussions on buffer size requirement of each proposal are contained in JCTVC-Mxxxd [4].

The author would like to thank MediaTek for crosschecking [3].

Table 2.2.1 (1:1 Comp@EL)

Table 2.2.2 (2:1 Comp@EL)

# Conclusion

This document reports experimental result of SCE5.2.2 on motion data compression. Simulation results show that with 1:1 compression at the enhancement layer, gain by -1.1%, -0.5%, -0.5%, -0.9%, -0.4% and -0.6% is obtained with 1:1 compression at the enhancement layer with RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively, and gain by -0.8%, -0.4%, -0.4%, -0.6%, -0.3% and -0.5% is obtained with 2:1 compression at the enhancement layer with RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively.

Discussions on buffer size requirement of each proposal are contained in JCTVC-Mxxxd.

It is recommended to consider for adoption of the proposing method with the discussion of required buffer size for each of the proposals.

# References

[1] “Description of Core Experiment SCE5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-L1105, January 2013

[2] “Common SHM test conditions and software reference configurations”, JCTVC-L1009, January 2013

[3] (Crosscheck report by MediaTek)

[4] “SCE5: Buffer Size Estimation of SCE5.2.x”, JCTVC-Mxxxd, April 2013

[5] “TE5-5.2.1: Inter-layer motion vector prediction”, JCTVC-L0070, January 2013

[6] “SHVC: On Motion Data Compression”, JCTVC-L0279, January 2013

# Patent rights declaration(s)

Sony Corp. / Sony Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).